THE REGIONAL DIRECTOR

KENYA RURAL ROADS AUTHORITY KEIYO MARAKWET REGION

P. O. BOX 213-30700 ITEN

KENYA RURAL ROADS AUTHORITY

TENDER FOR SPOT IMPROVEMENT OF

UFANISI-ENDO KABOR  ROAD

KEIYO SOUTH CONSTITUENCY, GOK FUNDING FY 2022/2023

INVITATION TO TENDER (ITT) No.:

KeRRA/008/KEMA/KS/039/GOK-2022/2023-106

(WOMEN)

AUGUST , 2023

THE DIRECTOR GENERAL

KENYA RURAL ROADS AUTHORITY P. O. BOX 48151-00100

NAIROBI

CONTENTS

INVITATION TO TENDER (ITT) No.:	1

KeRRA/008/KEMA/KS/039/GOK-2022/2023-106	1

AUGUST , 2023	1

THE DIRECTOR GENERAL	1

PART 1	TENDERING PROCEDURES	12

SECTION II - INSTRUCTIONS TO TENDERERS	13

A. GENERAL PROVISIONS	13

1. Scope of Tender	13

2. Fraud and Corruption	13

3. Eligible Tenderers	13

4. Eligible Goods, Equipment, and Services	15

5. Tenderer's Responsibilities	16

PART3: Conditions of Contract and Contract Forms	16

7. Clariﬁcation of Tender Document, Site Visit, Pre-Tender Meeting	16

8. Amendment of Tender Documents	17

10. Language of Tender	17

11. Documents Comprising the Tender	18

12. Form of Tender and Schedules	18

13. Alternative Tenders	18

14. Tender Prices and Discounts	19

15. Currencies of Tender and Payment	19

16. Documents Comprising the Technical Proposal	20

17. Documents Establishing the Eligibility and Qualiﬁcations of the Tenderer	20

18. Period of Validity of Tenders	21

19. Tender Security	21

20. Format and Signing of Tender	23

D. SUBMISSION AND OPENING OF TENDERS	23

22. Deadline for Submission of Tenders	24

23. Late Tenders	24

24. Withdrawal, Substitution, and Modiﬁcation of Tenders	24

25. Tender Opening	25

E. Evaluation and Comparison of Tenders	25

27. Clariﬁcation of Tenders	26

28. Deviations, Reservations, and Omissions	26

29. Determination of Responsiveness	26

30. Non-material Non-conformities	26

31. Arithmetical Errors	27

32. Conversion to Single Currency	27

33. Margin of Preference and Reservations	27

34. Nominated Subcontractors	27

35. Evaluation of Tenders	28

36. Comparison of Tenders	28

37. Abnormally Low Tenders and Abnormally High Tenders Abnormally Low Tenders	28

Abnormally High Tenders	29

38. Unbalanced and/or Front-Loaded Tenders	29

39. Qualiﬁcations of the Tenderer	29

40. Lowest Evaluated Tender	30

41. Procuring Entity's Right to Accept Any Tender, and to Reject Any or All Tenders.	30

43. Notice of Intention to enter into a Contract	30

44. Stand still Period	30

45. Debrieﬁng by the Procuring Entity	31

46. Letter of Award	31

47. Signing of Contract	31

48. Performance Security	31

49. Publication of Procurement Contract	31

50. Procurement Related Complaint	32

SECTION III - TENDER DATA SHEET (TDS)	33

SECTION IV- EVALUATION AND QUALIFICATION CRITERIA	37

1 General Provisions	37

1. Preliminary examination for Determination of Responsiveness	37

2. Tender Evaluation (ITT 35)	37

3. Multiple Contracts	37

4. Alternative Tenders (ITT 13.1)	38

5. Margin of Preference	38

6. Post qualiﬁcation and Contract award (ITT 39), more speciﬁcally,	39

c) History of non-performing contracts:	40

d) Pending Litigation	41

e) Litigation History	41

QUALIFICATION FORM	43

APPENDIX TO THE QUALIFICATION CRITERIA	53

SECTION V - TENDERING FORMS	57

QUALIFICATION FORMS	57

1. FORM EQU: EQUIPMENT	57

2. FORM PER -1	58

Contractor's Representative and Key Personnel Schedule	58

Contractor' Representative and Key Personnel	58

3. FORM PER-2:	59

Declaration	60

4. TENDERERS QUALIFICATION WITHOUT PRE-QUALIFICATION	61

FORM ELI -1.1 Tenderer Information Form	61

FORM ELI -1.2	62

FORM CON – 2	63

5. 58: FORM FIN – 3.1	64

Financial Situation and Performance	64

5.4.1. Financial Data	64

Sources of Finance	64

Financial documents	64

5.5 FORM FIN – 3.2	65

Average Annual Construction Turnover	65

5. 60 FORM FIN – 3.3:	66

Financial Resources	66

5.7 FORM FIN – 3.4:	66

Current Contract Commitments / Works in Progress	66

5.8 FORM EXP - 4.1	67

General Construction Experience	67

5.9 FORM EXP - 4.2(a)	68

Speciﬁc Construction and Contract Management Experience	68

10.1 FORM EXP- 4.2(b)	69

Construction Experience in Key Activities	69

OTHER FORMS	70

6. FORM OFTENDER	70

To:	The Regional Director,	70

Box 213-30070 ITEN	70

Date signed	day of ,	72

A. TENDERER’S ELIGIBILITY- CONFIDENTIALBUSINESS QUESTIONNAIRE	73

Instruction to Tenderer	73

(a)	Tenderer's details	73

(e) DISCLOSURE OF INTEREST- Interest of the Firm in the Procuring Entity.	74

ii) Conﬂict of interest disclosure	75

B. CERTIFICATE OF INDEPENDENT TENDER DETERMINATION	76

C. SELF - DECLARATION FORMS FORM SD1	77

SELF DECLARATION THAT THE PERSON/TENDERER WILL NOT ENGAGE IN ANY CORRUPT OR FRAUDULENT PRACTICE	78

Witness	79

D. APPENDIX 1- FRAUD AND CORRUPTION	80

1. Purpose	80

2. Requirements	80

Schedule of Currency requirements	84

PART II - WORK REQUIREMENTS	85

SECTION V – DRAWINGS	85

List of Drawings	85

i. Figure C.1-Cross Section A (Minor Standard Cross-Section)	85

ii. Figure C2- Cross Section B (Reduced Cross-Section)	85

vi. Figure C6 -Mitre Drains	85

vii. Figure C7 -Scour Checks	85

viii. Figure C8-Dimensions Of Scour Checks For Standard Drain	85

ix. Figure C9a-Masonry Scour Checks	85

x. Figure C9b- Concrete Scour Checks	85

xi. Figure C10- Culvert Entry/Exit Structure Types	85

FIGURE C2- CROSS SECTION B (REDUCED CROSS-SECTION)	88

FIGURE C6 -MITRE DRAINS	92

FIGURE C7 -SCOUR CHECKS	93

FIGURE C8-DIMENSIONS OF SCOUR CHECKS FOR STANDARD DRAIN	94

FIGURE C9a-MASONRY SCOUR CHECKS	95

I.Dimensio»s in mm	95

PLAN OF DRAIN WITH SCOUR CHECK	95

FIGURE C9b- CONCRETE SCOUR CHECKS	96

FIGURE C10- CULVERT ENTRY/EXIT STRUCTURE TYPES	97

PROFILE I                                                      PROFILE II	103

(CONCRETE)	(CONCRETE)	103

PROFILE III	PROFILE IV	104

SECTION VI – SPECIFICATIONS FOR ROUTINE MAINTENANCE /SPOT IMPROVEMENT WORKS	111

Standard Specification	111

SECTION 01	: PRELIMINARY AND GENERAL ITEMS	111

01-40- 001: Mobilization and Establishment of Site	111

01-40-002: Clearance on Completion	111

01-40-006: Traffic Control	111

01–40–007: Drinking Water	111

A Lump Sum shall be paid on a Monthly basis upon the approval of the Engineer that adequate supplies have been provided	112

Scope	112

04–50–002: Grass Cutting (Manual)	112

EA Burning of the grass shall not be allowed and care shall be taken not to damage roadsidefixtures such as signs and marker posts.	112

04-50-003	Bush Clearing - heavy04-50-	112

04-50-005: Pruning Tree Branches	112

04–50–008: Clearing of Obstructions	113

SECTION 05:	EARTHWORKS	113

05-70- 001: Grassing	113

SECTION 08:	CULVERT AND DRAINAGE WORKS	114

08-50-02 Ditch Cleaning (Manual) - Partially Silted08-50- 003	 - Fully Silted	114

i. Partially silted	114

08- 60 - 001/2/3/4/5: Culverts Cleaning (Partially blocked)	115

08- 60 - 006/7/8/9/10 : Culvert Cleaning (Fully blocked):	115

Supply and Installation of Concrete Pipe Culverts08-60-031	116

08 – 60-33 `	900 mm surrounds	116

Payment	117

08- 70- 001: Head Wall Repair - Masonry	117

08-70-002: Headwall Repair - Concrete	118

08 - 70 - 008:	Scour Check Repair - masonry08 - 70 -	118

010:	Scour Check Repair - concrete	118

EB	118

08- 70- 011: Stone Pitching Repair	119

08- 80- 004	Drift Maintenance – desilting	119

08-80-005	Drift Repairs – Concrete	119

08-80-003: Gabions	120

Payment	120

08-80-003 : Rockfill to Gabions	120

Payment	121

SECTION 10:	121

BILL 10: GRADING AND GRAVELLING	121

Scope:	121

10-60-001:	Provide Gravel Wearing Course (Excavation, Free haul, spreading and Compaction ofGravel)	122

Free haul, spreading and Compaction of Gravel	122

10 CARRIAGEWAY REPAIR WORKS (GRAVEL)	123

10- 50- 006	Light Manual Reshaping (Potholes, Ruts and Gullies)	123

10- 50- 009:	Light Manual Reshaping (Grub edge and Reshape Carriageway)	124

SECTION 11:	SHOULDER MAINTENANCE AND REPAIR	124

11-50-001: Shoulder Grading	124

SECTION 15:	PAVED CARRIAGEWAY MAINTENANCE	125

15 – 50 – 001:	Pothole Patching - hot mix 15 – 50 –	125

002:	Pothole Patching - cold mix	125

15 – 50 – 004:	Road Edge Repairs	126

The measurement shall be the area of the repair including the bituminous edge and shoulder	126

15 – 60 – 001: Spot Sealing - (Fine cracks)	126

15-60-002	Crack Sealing	127

15 – 80 – 001 to 005	Base repairs	127

16-80-02 :	Dense Bitumen Macadam	128

15-80-005: Neat Gravel	128

SECTION 17: BRIDGE MAINTENANCE	129

17 - 50 - 001:	Cleaning and Clearing - Deck	129

17 - 50 - 002:	Cleaning and Clearing - Riverbed	129

Payment shall be made for the labour and material instructed by the Engineer to be used on the work	129

SECTION 20: ROAD FURNITURE REPAIR AND MAINTENANCE	130

20-50- 001: Traffic Sign Maintenance	130

20-50- 003:	Guardrail Repair	130

20- 50- 004: Marker Posts Replacement	130

20-50-006: Sign cleaning and repainting	131

SECTION 25:	HIV/AIDS AWARENESS AND PREVENTION CAMPAIGN	131

Scope:	131

25-50-01 HIV / AIDS Awareness Campaign	131

The measurement shall be the calendar month or part thereof, measured over the duration of thecampaign	131

25-50-02 AIDS Prevention Campaign	131

1. Objectives	131

2. Day work Schedule	132

3. Provisional Sums	132

4. The Bills of Quantities	132

SECTION VII - BILLS OF QUANTITIES	133

PART III - CONDITIONS OF CONTRACT AND CONTRACT FORMS	138

SECTION VIII - GENERAL CONDITIONS OF CONTRACT	138

General Conditions of Contract	138

A. General	138

1. Deﬁnitions	138

2. Interpretation	140

3. Language andLaw	141

4. Project Manager's Decisions	141

5. Delegation	141

6. Communications	141

7. Subcontracting	141

8. Other Contractors	141

9. Personnel and Equipment	141

10. Procuring Entity's and Contractor's Risks	141

11. Procuring Entity's Risks	142

12. Contractor's Risks	142

13. Insurance	142

14. Site Data	142

15. Contractor to Construct the Works	142

16. The Works to Be Completed by the Intended Completion Date	143

17. Approval by the Project Manager	143

18. Safety	143

19. Discoveries	143

20. Possession of the Site	143

21. Access to the Site	143

22. Instructions, Inspections and Audits	143

23. Appointment of the Adjudicator	144

24. Settlement of Claims and Disputes	144

Arbitration	145

Arbitration with National Contractors	146

Alternative Arbitration Proceedings	146

Failure to Comply with Arbitrator's Decision	146

Contract operations to continue	146

25. Fraud and Corruption	146

B. Time Control	147

1. Program	147

2. Extension of the Intended Completion Date	147

3. Acceleration	147

4. Delays Ordered by the Project Manager	147

5. Management Meetings	147

6. Early Warning	148

C. Quality Control	148

1. Identifying Defects	148

2. Tests	148

3. Correction of Defects	148

4. Uncorrected Defects	148

D. Cost Control	148

1. Contract Price7	148

2. Changes in the Contract Price8	148

3. Variations	149

4. Cash Flow Forecasts	149

5. Payment Certiﬁcates	149

6. Payments	150

7. Compensation Events	150

8. Tax	151

9. Currency y of Payment	151

10. Price Adjustment	151

P = A + B Im/Io	151

11. Retention	151

12. Liquidated Damages	151

13. Bonus	152

14. Advance Payment	152

15. Securities	152

16. Dayworks	152

17. Cost of Repairs	152

18. Completion	153

19. Taking Over	153

20. Final Account	153

21. Operating and Maintenance Manuals	153

22. Termination	153

23. Payment upon Termination	154

24. Property	154

25. Release from Performance	154

FORM No 1: NOTIFICATION OF INTENTION TO AWARD	155

FORMAT	155

a) The successful tenderer	155

b) Other Tenderers	155

Signature:	Name:	156

[Option 1 - Unconditional Demand Bank Guarantee]	164

[Option 2– Performance Bond]	165

PERFORMANCE BOND No.:	165

Section IX - Special Conditions of Contract	168

(Amended and issued pursuant to PPRA CIRCULAR No. 02/2022)	170

SECTION I:	INVITATION TO TENDER

Keiyo Marakwet Region

Kenya Rural Roads Authority,

P.O. Box 213-30700

ITEN

Regional Email address: keiyomarakwet@kerra.go.ke

TENDER No. KeRRA/008/KEMA/KS/039/GOK-22/23-106–FOR SPOT IMPROVEMENT OF UFANISI-ENDO KABOR ROAD

The Authority invites bids from eligible bidders for the spot improvement of UFANISI-ENDO KABOR  ROAD to be funded by the GOK in the financial year 2022/2023 (KEIYO SOUTH CONSTITUENCY)

  - The tender is reserved-WOMEN the bidder must have been Registered by National Treasury, under WOMEN category .

  - Bidders can participate in several tenders, however, a bidder shall not participate in the same tender individually and also as a Joint Venture.

  - Qualiﬁed and interested tenderers may obtain further information and inspect the Tender Documents at KeRRA Regional Offices during ofﬁce hours [0900to 1500 hours] at the address given below.

  - A complete set of tender documents may be purchased or obtained by interested tenders upon payment of a non- refundable fees of KShs. 1,000/= Banker's Cheque and payable to the address given below. Tender documents may be obtained electronically from www.kerra.go.ke or www.tenders.go.ke. Tender documents obtained electronically will be free of charge.

  - Tender documents may be viewed and downloaded for free from the website www.kerra.go.ke or www.tenders.go.ke. Tenderers who download the tender document must forward their particulars immediately to facilitate any further clariﬁcation or addendum.

  - Tenders shall be quoted be in Kenya Shillings and shall include all taxes. Tenders shall remain valid for 175 days from the date of opening of tenders.

  - All Tenders must be accompanied by a “Tender-Securing Declaration,”

  - The Tenderer MUST chronologically serialize ALL pages of the tender documents submitted in the order of 1,2,3,4,……

  - ( Where Applicable) A bid bond of 2% of the Budget to be provided in the form of:-

- a bank guarantee;

- a guarantee by an insurance company registered and licensed by the Insurance Regulatory Authority listed by the Authority;

- or a guarantee issued by a ﬁnancial institution approved and licensed by the Central Bank of  Kenya, from a reputable source, and an eligible country.

  - Completed tenders will be available from 28th August, 2023 and must be delivered to the address below on or before 15th September, 2023 at 10:00am.Tenders will be opened immediately thereafter at the Keiyo Marakwet Regional office. Tenders will be publicly opened in the presence of the Tenderers' designated representatives who choose to attend.

The address referred to above are:

Further information and tender documents can be obtained from

The Deputy Director’s

Kenya Rural Roads Authority,

Keiyo-Marakwet Region

P.O. Box 213–30700, ITEN.

Email: keiyomarakwet@kerra.go.ke

Submission address

The Deputy Director’s

Kenya Rural Roads Authority,

Keiyo-Marakwet Region

P.O. Box 213–30700, ITEN.

Tender opening address

The Deputy Director’s Office

Kenya Rural Roads Authority,

Keiyo-Marakwet Region, ITEN

DEPUTY DIRECTOR

KeRRA KEIYO-MARAKWET REGION

PART 1	TENDERING PROCEDURES

SECTION II - INSTRUCTIONS TO TENDERERS

- GENERAL PROVISIONS

  - Scope of Tender

The Procuring Entity as deﬁned in the Appendix to Conditions of Contract invites tenders for Works Contract as described in the tender documents. The name, identiﬁcation, and number of lots (contracts)of this Tender Document are speciﬁed in the TDS.

  - Fraud and Corruption

    - The Procuring Entity requires compliance with the provisions of the Public Procurement and Asset Disposal Act, 2015, Section 62 “Declaration not to engage in corruption”. The tender submitted by a person shall include a declaration that the person shall not engage in any corrupt or fraudulent practice and a declaration that the person or his or her sub-contractors are not debarred from participating in public procurement proceedings.

    - The Procuring Entity requires compliance with the provisions of the Competition Act 2010, regarding collusive practices in contracting. Any tenderer found to have engaged in collusive conduct shall be disqualiﬁed and criminal and/or civil sanctions may be imposed. To this effect, Tenders shall be required to complete and sign the “Certiﬁcate of Independent Tender Determination” annexed to the Form of Tender.

    - Tenderers shall permit and shall cause their agents (where declared or not), subcontractors, sub- consultants, service providers, suppliers, and their personnel, to permit the Procuring Entity to inspect all accounts, records and other documents relating to any initial selection process, pre-qualiﬁcation process, tender submission, proposal submission, and contract performance (in the case of award), and to have them audited by auditors appointed by the Procuring Entity.

    - Unfair Competitive Advantage -Fairness and transparency in the tender process require that the ﬁrms or their Afﬁliates competing for a speciﬁc assignment do not derive a competitive advantage from having provided consulting services related to this tender. To that end, the Procuring Entity shall indicate in the Data Sheet and make available to all the ﬁrms together with this tender document all information that would in that respect give such ﬁrm any unfair competitive advantage over competing ﬁrms.

  - Eligible Tenderers

    - A Tenderer may be a ﬁrm that is a private entity, a state-owned enterprise or institution subject to ITT 3.8, or an individual or any combination of such entities in the form of a joint venture (JV) under an existing agreement or with the intent to enter into such an agreement supported by a letter of intent. In the case of a joint venture, all members shall be jointly and severally liable for the execution of the entire Contract in accordance with the Contract terms. The JV shall nominate a Representative who shall have the authority to conduct all business for and on behalf of any and all the members of the JV during the tendering process and, in the event the JV is awarded the Contract, during contract execution. Members of a joint venture may not also make an individual tender, be a subcontractor in a separate tender or be part of another joint venture for the purposes of the same Tender. The maximum number of JV members shall be speciﬁed in the TDS.

    - Public Ofﬁcers of the Procuring Entity, their Spouses, Child, Parent, Brothers or Sister. Child, Parent, Brother or Sister of a Spouse, their business associates or agents and ﬁrms/organizations in which they have a substantial or controlling interest shall not be eligible to tender or be awarded a contract. Public Ofﬁcers are also not allowed to participate in any procurement proceedings.

    - A Tenderer shall not have a conﬂict of interest. Any tenderer found to have a conﬂict of interest shall

be disqualiﬁed. A tenderer may be considered to have a conﬂict of interest for the purpose of this tendering process, if the tenderer:

      - Directly or indirectly controls, is controlled by or is under common control with another tenderer; or

      - Receives or has received any direct or indirect subsidy from another tenderer; or

      - Has the same legal representative as another tenderer; or

      - Has a relationship with another tenderer, directly or through common third parties, that puts it in a position to inﬂuence the tender of another tenderer, or inﬂuence the decisions of the Procuring Entity regarding this tendering process; or

      - Any of its afﬁliates participated as a consultant in the preparation of the design or technical speciﬁcations of the goods or works that are the subject of the tender; or

      - any of its afﬁliates has been hired (or is proposed to be hired) by the Procuring Entity as a consultant for Contract implementation; or

      - Would be providing goods, works, or non-consulting services resulting from or directly related to consulting services for the preparation or implementation of the contract speciﬁed in this Tender Document; or

      - Has a close business or personal relationship with senior management or professional staff of the Procuring Entity who has the ability to inﬂuence the bidding process and:

      - are directly or indirectly involved in the preparation of the Tender document or speciﬁcations of the Contract, and/or the Tender evaluation process of such contract; or

ii) May be involved in the implementation or supervision of such Contract unless the conﬂict stemming from such relationship has been resolved in a manner acceptable to the Procuring Entity throughout the tendering process and execution of the Contract.

    - A tenderer shall not be involved incorrupt, coercive, obstructive or fraudulent practice. A tenderer that is proven to have been involved in any of these practices shall be automatically disqualiﬁed.

    - A Tenderer (either individually or as a JV member) shall not participate in more than one Tender, except for permitted alternative tenders. This includes participation as a subcontractor in other Tenders. Such participation shall result in the disqualiﬁcation of all Tenders in which the ﬁrm is involved. Members of a joint venture may not also make an individual tender, be a subcontractor in a separate tender or be part of another joint venture for the purposes of the same Tender.

    - A Tenderer may have the nationality of any country, subject to the restrictions pursuant to ITT3.9. A Tenderer shall be deemed to have the nationality of a country if the Tenderer is constituted, incorporated or registered in and operates inconformity with the provisions of the laws of that country, as evidenced by its articles of incorporation (or equivalent documents of constitution or association) and its registration documents, as the case may be. This criterion also shall apply to the determination of the nationality of proposed subcontractors or sub-consultants for any part of the Contract including related Services.

    - A Tenderer that has been debarred from participating in public procurement shall be ineligible to tender or be awarded a contract. The list of debarred ﬁrms and individuals is available from the website of PPRA www.ppra.go.ke.

    - A Tenderer that is a state-owned enterprise or a public institution in Kenya may be eligible to tender and be awarded a Contract(s) only if it is determined by the Procuring Entity to meet the following conditions, i.e. if it is:

- A legal public entity of Government and/or public administration,

- ﬁnancially autonomous and not receiving any signiﬁcant subsidies or budget support from any public entity or Government, and

- Operating under commercial law and vested with legal rights and liabilities similar to any commercial enterprise to enable it compete with ﬁrms in the private sector on an equal basis.

    - Firms and individuals shall be ineligible if their countries of origin are:

      - as a matter of law or ofﬁcial regulations, Kenya prohibits commercial relations with that country, or

      - by an act of compliance with a decision of the United Nations Security Council taken under Chapter VII of the Charter of the United Nations, Kenya prohibits any import of goods or contracting of works or services from that country, or any payments to any country, person, or entity in that country.

A tenderer shall provide such documentary evidence of eligibility satisfactory to the Procuring Entity, as the Procuring Entity shall reasonably request.

    - Foreign tenderers are required to source at least forty (40%) percent of their contract inputs (in supplies, local subcontracts and labor) from citizen suppliers and contractors. To this end, a foreign tenderer shall provide in its tender documentary evidence that this requirement is met. Foreign tenderers not meeting this criterion will be automatically disqualiﬁed. Information required to enable the Procuring Entity determine if this condition is met shall be provided for this purpose in “SECTION III- EVALUATION AND QUALIFICATION CRITERIA, Item 9”.

    - Pursuant to the eligibility requirements of ITT4.10, a tender is considered a foreign tenderer, if the tenderer is not registered in Kenya or if the tenderer is registered in Kenya and has less than 51 percent ownership by Kenyan citizens. JVs are considered as foreign tenderers if the individual member ﬁrms are not registered in Kenya or if are registered in Kenya and have less than 51 percent ownership by Kenyan citizens. The JV shall not subcontract to foreign ﬁrms more than 10 percent of the contract price, excluding provisional sums.

    - The National Construction Authority Act of Kenya requires that all local and foreign contractors be registered with the National Construction Authority and be issued with a Registration Certiﬁcate before they can undertake any construction works in Kenya. Registration shall not be a condition for tender, but it shall be a condition of contract award and signature. A selected tenderer shall be given opportunity to register before such award and signature of contract. Application for registration with National Construction Authority may be accessed from the website www.nca.go.ke.

    - The Competition Act of Kenya requires that ﬁrms wishing to tender as Joint Venture undertakings which may prevent, distort or lessen competition in provision of services are prohibited unless they are exempt in accordance with the provisions of Section 25 of the Competition Act, 2010. JVs will be required to seek for exemption from the Competition Authority. Exemption shall not be a condition for tender, but it shall be a condition of contract award and signature. A JV tenderer shall be given opportunity to seek such exemption as a condition of award and signature of contract. Application for exemption from the Competition Authority of Kenya may be accessed from the website www.cak.go.ke.

4.14 A Kenyan tenderer shall be eligible to tender if it provides evidence of having fulﬁlled his/her tax obligations by producing a valid tax compliance or valid tax certiﬁcate issued by the Kenya Revenue Authority.

  - Eligible Goods, Equipment, and Services

    - Goods, equipment and services to be supplied under the Contract may have their origin in any country that is not ineligible under ITT3.9. At the Procuring Entity's request, Tenderers may be required to provide evidence of the origin of Goods, equipment and services.

    - Any goods, works and production processes with characteristics that have been declared by the relevant national environmental protection agency or by other competent authority as harmful to human beings and to the environment shall not be eligible for procurement.

  - Tenderer's Responsibilities

    - The tenderer shall bear all costs associated with the preparation and submission of his/her tender, and the Procuring Entity will in no case be responsible or liable for those costs.

    - The tenderer, at the tenderer's own responsibility and risk, is encouraged to visit and examine and inspect the Site of the Works and its surroundings and obtain all information that may be necessary for preparing the tender and entering into a contract for construction of the Works. The costs of visiting the Site shall be at the tenderer's own expense.

    - The Tenderer and any of its personnel or agents will be granted permission by the Procuring Entity to enter up on its premises and lands for the purpose of such visit. The Tenderer shall indemnify the Procuring Entity against all liability arising from death or personal injury, loss of or damage to property, and any other losses and expenses incurred as a result of the examination and inspection.

    - The tenderer shall provide in the Form of Tender and Qualiﬁcation Information, a preliminary description of the proposed work method and schedule, including charts, as necessary or required.

- CONTENTS OF TENDER DOCUMENTS

  - Sections of Tender Document

    - The tender document consists of Parts 1, 2, and 3, which includes all the sections speciﬁed below, and which should be read in conjunction with any Addenda issued in accordance with ITT10.

PART 1: Tendering Procedures Section I: Instructions to Tenderers Section II: Tender Data Sheet (TDS)

Section III: Evaluation and Qualiﬁcation Criteria Section Section IV: Tendering Forms

PART 2: Works' Requirements Section V: Bills of Quantities Section VI: Speciﬁcations Section VII: Drawings

PART3: Conditions of Contract and Contract Forms

Section VIII: General Conditions (GCC) Section IX: Particular Conditions of Contract Section X: Contract Forms

    - The Invitation to Tender Notice issued by the Procuring Entity is not part of the Contract documents.

    - Unless obtained directly from the Procuring Entity, the Procuring Entity is not responsible for the completeness of the Tender document, responses to requests for clariﬁcation, the minutes of a pre- arranged site visit and those of the pre-Tender meeting (if any), or Addenda to the Tender document in accordance with ITT 10. In case of any contradiction, documents obtained directly from the Procuring Entity shall prevail.

    - The Tenderer is expected to examine all instructions, forms, terms, and speciﬁcations in the Tender Document and to furnish with its Tender all information and documentation as is required by the Tender document.

  - Clariﬁcation of Tender Document, Site Visit, Pre-Tender Meeting

    - Tenderer requiring any clariﬁcation of the Tender Document shall contact the Procuring Entity in writing at the Procuring Entity's address speciﬁed in the TDS or raise its enquiries during the pre- Tender meeting if provided for in accordance with ITT 7.2. The Procuring Entity will respond in writing to any request for clariﬁcation, provided that such request is received no later than the period

speciﬁed in the TDS prior to the deadline for submission of tenders. The Procuring Entity shall forwardcopies of its response to all tenderers who have acquired the Tender D documents in accordance with ITT 7.4, including a description of the inquiry but without identifying its source. If so speciﬁed in the TDS, the Procuring Entity shall also promptly publish its response at the web page identiﬁed in the TDS. Should the clariﬁcation result in changes to the essential elements of the Tender Documents, theProcuring Entity shall amend the Tender Documents following the procedure under ITT 8 and ITT 22.2.

    - The Tenderer, at the Tenderer's own responsibility and risk, is encouraged to visit and examine and inspect the site(s) of the required contracts and obtain all information that may be necessary for preparing a tender. The costs of visiting the Site shall be at the Tenderer’s own expense. The Procuring Entity shall specify in the TDS if a pre- arranged Site visit and or a pre-tender meeting will be held, when and where. The Tenderer's designated representative is invited to attend a pre-arranged site visit and a pre-tender meeting, as the case may be. The purpose of the site visit and the pre-tender meeting will be to clarify issues and to answer questions on any matter that may be raised at that stage.

    - The Tenderer is requested to submit any questions in writing, to reach the Procuring Entity not later than the period speciﬁed in the TDS before the meeting.

    - Minutes of a pre-arranged site visit and those of the pre-tender meeting, if applicable, including the text of the questions asked by Tenderers and the responses given, together with any responses prepared after the meeting, will be transmitted promptly to all Tenderers who have acquired the Tender Documents. Minutes shall not identify the source of the questions asked.

    - The Procuring Entity shall also promptly publish anonymized (no names) Minutes of the pre-arranged site visit and those of the pre-tender meeting at the web page identiﬁed in the TDS. Any modiﬁcation to the Tender Documents that may become necessary as a result of the pre-arranged site visit and those of the pre-tender meeting shall be made by the Procuring Entity exclusively through the issue of an Addendum pursuant to ITT 8 and not through the minutes of the pre-Tender meeting. Non-attendance at the pre-arranged site visit and the pre-tender meeting will not be a cause for disqualiﬁcation of a Tenderer.

  - Amendment of Tender Documents

    - At any time prior to the deadline for submission of Tenders, the Procuring Entity may amend the Tender Documents by issuing addenda.

    - Any addendum issued shall be part of the Tender Documents and shall be communicated in writing to all who have obtained the Tender Documents from the Procuring Entity. The Procuring Entity shall also promptly publish the addendum on the Procuring Entity's website in accordance with ITT 7.5.

    - To give Tenderers reasonable time in which to take an addendum into account in preparing their Tenders, the Procuring Entity should extend the deadline for the submission of Tenders, pursuant to ITT 22.2.

- PREPARATION OF TENDERS

  - Cost of Tendering

The Tenderer shall meet all costs associated with the preparation and submission of its Tender, and the Procuring Entity shall not be responsible or liable for those costs, regardless of the conduct or outcome of the tendering process.

  - Language of Tender

The Tender, as well as all correspondence and documents relating to the tender exchanged by the tenderer and the Procuring Entity, shall be written in the English Language. Supporting documents and

printed literature that are part of the Tender may be in another language provided they are accompanied by an accurate and notarized translation of the relevant passages into the English Language, in which case, for purposes of interpretation of the Tender, such translation shall govern.

  - Documents Comprising the Tender

    - The Tender shall comprise the following:

      - Form of Tender prepared in accordance with ITT 12;

      - Schedules including priced Bill of Quantities, completed in accordance with ITT 12 and ITT 14;

      - Tender Security or Tender-Securing Declaration, in accordance with ITT 19.1;

      - Alternative Tender, if permissible, in accordance with ITT 13;

      - Authorization: written conﬁrmation authorizing the signatory of the Tender to commit the Tenderer, in accordance with ITT 20.3;

      - Qualiﬁcations: documentary evidence in accordance with ITT 17 establishing the Tenderer's qualiﬁcations to perform the Contract if its Tender is accepted;

      - Conformity: a technical proposal in accordance with ITT 16;

      - Any other document required in the TDS.

    - In addition to the requirements under ITT 11.1, Tenders submitted by a JV shall include a copy of the Joint Venture Agreement entered into by all members. Alternatively, a letter of intent to execute a Joint Venture Agreement in the event of a successful Tender shall be signed by all members and submitted with the Tender, together with a copy of the proposed JV Agreement. Change of membership and conditions of the JV prior to contract signature will render the tender liable for disqualiﬁcation.

  - Form of Tender and Schedules

    - The Form of Tender and Schedules, including the Bill of Quantities, shall be prepared using the relevant forms furnished in Section IV, Tendering Forms. The forms must be completed without any alterations to the text, and no substitutes shall be accepted except as provided under ITT 20.3. All blank spaces shall be ﬁlled in with the information requested. The Tenderer shall chronologically serialize all pages of the tender documents submitted.

    - The Tenderer shall furnish in the Form of Tender information on commissions and gratuities, if any, paid or to be paid to agents or any other party relating to this Tender.

  - Alternative Tenders

    - Unless otherwise speciﬁed in the TDS, alternative Tenders shall not be considered.

    - When alternative times for completion are explicitly invited, a statement to that effect will be included in the TDS, and the method of evaluating different alternative times for completion will be described in Section III, Evaluation and Qualiﬁcation Criteria.

    - Except as provided under ITT13.4 below, Tenderers wishing to offer technical alternatives to the requirements of the Tender Documents must ﬁrst price the Procuring Entity's design as described in the Tender Documents and shall further provide all information necessary for a complete evaluation of the alternative by the Procuring Entity, including drawings, design calculations, technical speciﬁcations, breakdown of prices, and proposed construction methodology and other relevant details. Only the technical alternatives, if any, of the Tenderer with the Winning Tender conforming to the basic technical requirements shall be considered by the Procuring Entity.

    - When speciﬁed in the TDS, Tenderers are permitted to submit alternative technical solutions for speciﬁed parts of the Works, and such parts will be identiﬁed in the TDS, as will the method for their evaluating, and described in Section VII, Works' Requirements.

  - Tender Prices and Discounts

    - The prices and discounts (including any price reduction) quoted by the Tenderer in the Form of Tender and in the Bill of Quantities shall conform to the requirements speciﬁed below.

    - The Tenderer shall ﬁll in rates and prices for all items of the Works described in the Bill of Quantities. Items against which no rate or price is entered by the Tenderer shall be deemed covered by the rates for other items in the Bill of Quantities and will not be paid for separately by the Procuring Entity. An item not listed in the priced Bill of Quantities shall be assumed to be not included in the Tender, and provided that the Tender is determined substantially responsive notwithstanding this omission, the average price of the item quoted by substantially responsive Tenderers will be added to the Tender price and the equivalent total cost of the Tender so determined will be used for price comparison.

    - The price to be quoted in the Form of Tender, in accordance with ITT 12, shall be the total price of the Tender, including any discounts offered.

    - The Tenderer shall quote any discounts and the methodology for their application in the Form of Tender, in accordance with ITT 12

    - It will be speciﬁed in the TDS if the rates and prices quoted by the Tenderer are or are not subject to adjustment during the performance of the Contract in accordance with the provisions of the Conditions of Contract, excepting cases where the contract is subject to ﬂuctuations and adjustments, not ﬁxed price. In such a case, the Tenderer shall furnish the indices and weightings for the price adjustment formulae in the Schedule of Adjustment Data and the Procuring Entity may require the Tenderer to justify its proposed indices and weightings.

    - Where tenders are being invited for individual lots (contracts) or for any combination of lots (packages), tenderers wishing to offer discounts for the award of more than one Contract shall specify in their Tender the price reductions applicable to each package, or alternatively, to individual Contracts within the package. Discounts shall be submitted in accordance with ITT 14.4, provided the Tenders for all lots (contracts) are opened at the same time.

    - All duties, taxes, and other levies payable by the Contractor under the Contract, or for any other cause, as of the date 30 days prior to the deadline for submission of Tenders, shall be included in the rates and prices and the total Tender Price submitted by the Tenderer.

  - Currencies of Tender and Payment

    - The currency (ies) of the Tender and the currency (ies) of payments shall be the same.

    - Tenderers shall quote entirely in Kenya Shillings. The unit rates and the prices shall be quoted by the Tenderer in the Bill of Quantities, entirely in Kenya shillings

      - A Tenderer expecting to incur expenditures in other currencies for inputs to the Works supplied from outside Kenya (referred to as “the foreign currency requirements”) shall (if so allowed in the TDS) indicate in the Appendix to Tender the percentage(s) of the Tender Price (excluding Provisional Sums), needed by the Tenderer for the payment of such foreign currency requirements, limited to no more than two foreign currencies.

      - The rates of exchange to be used by the Tenderer in arriving at the local currency equivalent and the percentage(s) mentioned in (a) above shall be speciﬁed by the Tenderer in the Appendix to Tender and shall be based on the exchange rate provided by the Central Bank of Kenya on the date 30 days prior to the actual date of tender opening. Such exchange rate shall apply for all foreign payments under the Contract.

    - Tenderers may be required by the Procuring Entity to justify, to the Procuring Entity's satisfaction, their local and foreign currency requirements, and to substantiate that the amounts included in the unit rates and prices and shown in the Schedule of Adjustment Data in the Appendix to Tender are reasonable, in which case a detailed breakdown of the foreign currency requirements shall be provided by

Tenderers.

  - Documents Comprising the Technical Proposal

The Tenderer shall furnish a technical proposal including a statement of work methods, equipment, personnel, schedule and any other information as stipulated in Section IV, Tender Forms, and insufﬁcient detail to demonstrate the adequacy of the Tenderer's proposal to meet the work's requirements and thecompletion time.

  - Documents Establishing the Eligibility and Qualiﬁcations of the Tenderer

    - Tenderers shall complete the Form of Tender, included in Section IV, Tender Forms, to establish Tenderer's eligibility in accordance with ITT 4.

    - In accordance with Section IV, Evaluation and Qualiﬁcation Criteria, to establish its qualiﬁcations to perform the Contract the Tenderer shall provide the information requested in the corresponding information sheets included in Section V, Tender Forms.

    - If a margin of preference applies as speciﬁed in accordance with ITT33. 1, national tenderers, individually or in joint ventures, applying for eligibility for national preference shall supply all information required to satisfy the criteria for eligibility speciﬁed in accordance with ITT 33.1.

    - Tenderers shall be asked to provide, as part of the data for qualiﬁcation, such information, including details of ownership, as shall be required to determine whether, according to the classiﬁcation established by the Procuring Entity, a particular contract or group of contractor’s qualiﬁes for a margin of preference. Further the information will enable the Procuring Entity identify any actual or potential conﬂict of interest in relation to the procurement and/or contract management processes, or possibility of collusion between tenderers, and thereby help to prevent any corrupt inﬂuence in relation to the procurement process or contract management.

    - The purpose of the information described in ITT 17.2 above overrides any claims to conﬁdentiality which a tenderer may have. There can be no circumstances in which it would be justiﬁed for a tenderer to keep information relating to its ownership and control conﬁdential where it is tendering to undertake public sector work and receive public sector funds. Thus, conﬁdentiality will not be accepted by the Procuring Entity as a justiﬁcation for a Tenderer's failure to disclose, or failure to provide required information on its ownership and control.

    - The Tenderer shall provide further documentary proof, information or authorizations that the Procuring Entity may request in relation to ownership and control which information on any changes to the information which was provided by the tenderer under ITT 6.4. The obligations to require this information shall continue for the duration of the procurement process and contract performance and after completion of the contract, if any change to the information previously provided may reveal a conﬂict of interest in relation to the award or management of the contract.

    - All information provided by the tenderer pursuant to these requirements must be complete, current and accurate as at the date of provision to the Procuring Entity. In submitting the information required pursuant to these requirements, the Tenderer shall warrant that the information submitted is complete, current and accurate as at the date of submission to the Procuring Entity.

    - If a tenderer fails to submit the information required by these requirements, its tender will be rejected. Similarly, if the Procuring Entity is unable, after taking reasonable steps, to verify to a reasonable degree the information submitted by a tenderer pursuant to these requirements, then the tender will be rejected.

    - If information submitted by a tenderer pursuant to these requirements, or obtained by the Procuring Entity (whether through its own enquiries, through notiﬁcation by the public or otherwise), shows any conﬂict of interest which could materially and improperly beneﬁt the tenderer in relation to the procurement or contract management process, then:

- If the procurement process is still on going, the tenderer will be disqualiﬁed from the procurement process,

- If the contract has been awarded to that tenderer, the contract award will be set aside,

- the tenderer will be referred to the relevant law enforcement authorities for investigation of whether the tenderer or any other persons have committed any criminal offence.

    - If a tenderer submits information pursuant to these requirements that is incomplete, inaccurate or out- of-date, or attempts to obstruct the veriﬁcation process, then the consequences ITT 17.8 will ensue unless the tenderer can show to the reasonable satisfaction of the Procuring Entity that any such act was not material, or was due to genuine error which was not attributable to the intentional act, negligence or recklessness of the tender.

  - Period of Validity of Tenders

18.1. Tenders shall remain valid for the Tender Validity period speciﬁed in the TDS. The Tender Validity period starts from the date ﬁxed for the Tender submission deadline (as prescribed by the Procuring Entity in accordance with ITT 22). A Tender valid for a shorter period shall be rejected by the Procuring Entity as non-responsive.

18.2 In exceptional circumstances, prior to the expiration of the Tender validity period, the Procuring Entity may request Tenderers to extend the period of validity of their Tenders. The request and the responses shall be made in writing. If a Tender Security is requested in accordance with ITT 19, it shall also be extended for thirty (30) days beyond the deadline of the extended validity period. A Tenderer may refuse the request without forfeiting its Tender security. A Tenderer granting their quest shall not be required or permitted to modify its Tender.

  - Tender Security

    - The Tenderer shall furnish as part of its Tender, either a Tender-Securing Declaration or a Tender Security as speciﬁed in the TDS, in original form and, in the case of a Tender Security, in the amount and currency speciﬁed in the TDS. A Tender-Securing Declaration shall use the form included in Section IV, Tender Forms.

    - If a Tender Security is speciﬁed pursuant to ITT19.1, the Tender Security shall be a demand guarantee in any of the following forms at the Tenderer's option:

- cash;

- a bank guarantee;

- a guarantee by an insurance company registered and licensed by the Insurance Regulatory Authority listed by the Authority; or

- a guarantee issued by a ﬁnancial institution approved and licensed by the Central Bank of Kenya, from a reputable source, and an eligible country.

    - If an unconditional bank guarantee is issued by a bank located outside Kenya, the issuing bank shall have a correspondent bank located in Kenya to make it enforceable. The Tender Security shall be valid for thirty (30) days beyond the original validity period of the Tender, or beyond any period of extension if requested under ITT 18.2.

    - If a Tender Security or Tender-Securing Declaration is speciﬁed pursuant to ITT 19.1, any Tender not accompanied by a substantially responsive Tender Security or Tender-Securing Declaration shall be rejected by the Procuring Entity as non-responsive.

    - If a Tender Security is speciﬁed pursuant to ITT 19.1, the Tender Security of unsuccessful Tenderers shall be returned as promptly as possible upon the successful Tenderer's signing the Contract and furnishing the Performance Security and any other documents required in the TDS. The Procuring Entity shall also promptly return the tender security to the tenderers where the procurement proceedings are terminated, all tenders were determined non-responsive or a bidder declines to extend tender validity period.

    - The Tender Security of the successful Tenderer shall be returned as promptly as possible once the

successful Tenderer has signed the Contract and furnished the required Performance Security, and

any other documents required in the TDS.

    - The Tender Security may be forfeited, or the Tender-Securing Declaration executed:

      - if a Tenderer withdraws its Tender during the period of Tender validity speciﬁed by the Tenderer on the Form of Tender, or any extension there to be provided by the Tenderer; or

      - if the successful Tenderer fails to:

- sign the Contract in accordance with ITT 47; or

- furnish a Performance Security and if required in the TDS, and any other documents required in the TDS.

    - Where tender securing declaration is executed, the Procuring Entity shall recommend to the PPRA that PPRA debars the Tenderer from participating in public procurement as provided in the law.

    - The Tender Security or the Tender-Securing Declaration of a JV shall be in the name of the JV that submits the Tender. If the JV has not been legally constituted into a legally enforceable JV at the time of tendering, the Tender Security or the Tender-Securing Declaration shall be in the names of all future members as named in the letter of intent referred to in ITT 4.1 and ITT 11.2.

    - A tenderer shall not issue a tender security to guarantee itself.

  - Format and Signing of Tender

    - The Tenderer shall prepare one original of the documents comprising the Tender as described in ITT 11 and clearly mark it “ORIGINAL.” Alternative Tenders, if permitted in accordance with ITT 13, shall be clearly marked “ALTERNATIVE.” In addition, the Tenderer shall submit copies of the Tender, in the number speciﬁed in the TDS and clearly mark them “COPY.” In the event of any discrepancy between the original and the copies, the original shall prevail.

    - Tenderers shall mark as “CONFIDENTIAL” all information in their Tenders which is conﬁdential to their business. This may include proprietary information, trade secrets, or commercial or ﬁnancially sensitive information.

    - The original and all copies of the Tender shall be typed or written in indelible ink and shall be signed by a person duly authorized to sign on behalf of the Tenderer. This authorization shall consist of a written conﬁrmation as speciﬁed in the TDS and shall be attached to the Tender. The name and position held by each person signing the authorization must be typed or printed below the signature. All pages of the Tender where entries or amendments have been made shall be signed or initialed by the person signing the Tender.

    - In case the Tenderer is a JV, the Tender shall be signed by an authorized representative of the JV on behalf of the JV, and so as to be legally binding on all the members as evidenced by a power of attorney signed by their legally authorized representatives.

    - Any inter-lineation, erasures, or overwriting shall be valid only if they are signed or initialed by the person signing the Tender.

- SUBMISSION AND OPENING OF TENDERS

  - Sealing and Marking of Tenders

    - The Tenderer shall deliver the Tender in a single sealed envelope, or in a single sealed package, or in a single sealed container bearing the name and Reference number of the Tender, addressed to the Procuring Entity and a warning not to open before the time and date for Tender opening date. Within

the single envelope, package or container, the Tenderer shall place the following separate, sealed envelopes:

      - in an envelope or package or container marked “ORIGINAL”, all documents comprising the Tender, as describedinITT11; and

      - in an envelope or package or container marked “COPIES” all required copies of the Tender; and

      - if alternative Tenders are permitted in accordance with ITT 13, and if relevant:

        - in an envelope or package or container marked “ORIGINAL - ALTERNATIVE TENDER”, the alternative Tender; and

        - in the envelope or package or container marked “COPIES-ALTERNATIVE TENDER”, all required copies of the alternative Tender.

The inner envelopes or packages or containers shall:

- bear the name and address of the Procuring Entity.

- Bear the name and address of the Tenderer; and

- Bear the name and Reference number of the Tender.

    - If an envelope or package or container is not sealed and marked as required, the Procuring Entity will assume no responsibility for the misplacement or premature opening of the Tender. Tenders that were misplaced or opened prematurely will not be accepted.

  - Deadline for Submission of Tenders

    - Tenders must be received by the Procuring Entity at the address speciﬁed in the TDS and no later than the date and time also speciﬁed in the TDS. When so speciﬁed in the TDS, Tenderers shall have the option of submitting their Tenders electronically. Tenderers submitting Tenders electronically shall follow the electronic Tender submission procedures speciﬁed in the TDS.

    - The Procuring Entity may, at its discretion, extend the deadline for the submission of Tenders by amending the Tender Documents in accordance with ITT 8, in which case all rights and obligations of the Procuring Entity and Tenderers previously subject to the deadline shall thereafter be subject to the deadline as extended.

  - Late Tenders

The Procuring Entity shall not consider any Tender that arrives after the deadline for submission of tenders, in accordance with ITT 22. Any Tender received by the Procuring Entity after the deadline for submission of Tenders shall be declared late, rejected, and returned unopened to the Tenderer.

  - Withdrawal, Substitution, and Modiﬁcation of Tenders

    - A Tenderer may withdraw, substitute, or modify its Tender after it has been submitted by sending a written notice, duly signed by an authorized representative, and shall include a copy of the authorization in accordance with ITT 20.3, (except that withdrawal notices do not require copies). The corresponding substitution or modiﬁcation of the Tender must accompany the respective written notice. All notices must be:

      - prepared and submitted in accordance with ITT 20 and ITT 21 (except that withdrawals notices do not require copies), and in addition, the respective envelopes shall be clearly marked “WITHDRAWAL,” “SUBSTITUTION,” “MODIFICATION;” and

      - received by the Procuring Entity prior to the deadline prescribed for submission of Tenders, in accordance with ITT 22.

    - Tenders requested to be withdrawn in accordance with ITT 24.1 shall be returned unopened to the Tenderers.

    - No Tender may be withdrawn, substituted, or modiﬁed in the interval between the deadline for submission of Tenders and the expiration of the period of Tender validity speciﬁed by the Tenderer on the Form of Tender or any extension thereof.

  - Tender Opening

    - Except in the cases speciﬁed in ITT 23 and ITT 24.2, the Procuring Entity shall publicly open and read out all Tenders received by the deadline, at the date, time and place speciﬁed in the TDS, in the presence of Tenderers' designated representatives and anyone who chooses to attend. Any speciﬁc electronic Tender opening procedures required if electronic Tendering is permitted in accordance with ITT 22.1, shall be as speciﬁed in the TDS.

    - First, envelopes marked “WITHDRAWAL” shall be opened and read out and the envelopes with the corresponding Tender shall not be opened but returned to the Tenderer. No Tender withdrawal shall be permitted unless the corresponding withdrawal notice contains a valid authorization to request the withdrawal and is read out attender opening.

    - Next, envelopes marked “SUBSTITUTION” shall be opened and read out and exchanged with the corresponding Tender being substituted, and the substituted Tender shall not be opened, but returned to the Tenderer. No Tender substitution shall be permitted unless the corresponding substitution notice contains a valid authorization to request the substitution and is read out at Tender opening. Next, envelopes marked “MODIFICATION” shall be opened and read out with the corresponding Tender No. Tender modiﬁcation shall be permitted unless the corresponding modiﬁcation notice contains a valid authorization to request the modiﬁcation and is read out at Tender opening.

    - Next, all remaining envelopes shall be opened one at a time, reading out: the name of the Tenderer and whether there is a modiﬁcation; the total Tender Price, per lot (contract) if applicable, including any discounts and alternative Tenders; the presence or absence of a Tender Security or Tender- Securing Declaration, if required; and any other details as the Procuring Entity may consider appropriate.

    - Only Tenders, alternative Tenders and discounts that are opened and read out at Tender opening shall be considered further for evaluation. The Form of Tender and pages of the Bill of Quantities (to be decided on by the tender opening committee) are to be initialed by the members of the tender opening committee attending the opening.

    - At the Tender Opening, the Procuring Entity shall neither discuss the merits of any Tender nor reject any Tender (except for late Tenders, in accordance with ITT 23.1).

    - The Procuring Entity shall prepare minutes of the Tender Opening that shall include, as a minimum:

      - The name of the Tenderer and whether there is a withdrawal, substitution, or modiﬁcation;

      - The Tender Price, per lot (contract) if applicable, including any discounts;

      - any alternative Tenders;

      - the presence or absence of a Tender Security, if one was required.

      - number of pages of each tender document submitted.

    - The Tenderers' representatives who are present shall be requested to sign the minutes. The omission of a Tenderer's signature on the minutes shall not invalidate the contents and effect of the minutes. A copy of tender opening register shall be issued to a tenderer upon request.

- Evaluation and Comparison of Tenders

  - Conﬁdentiality

    - Information relating to the evaluation of Tenders and recommendation of contract award shall not bedisclosed to Tenderers or any other persons not ofﬁcially concerned with the Tender process until information on Intention to Award the Contract is transmitted to all Tenderers in accordance with ITT43.

    - Any effort by a Tenderer to inﬂuence the Procuring Entity in the evaluation of the Tenders or Contract

award decisions may result in the rejection of its tender.

    - Notwithstanding ITT 26.2, from the time of tender opening to the time of contract award, if a tenderer wishes to contact the Procuring Entity on any matter related to the tendering process, it shall do so in writing.

  - Clariﬁcation of Tenders

    - To assist in the examination, evaluation, and comparison of the tenders, and qualiﬁcation of the tenderers, the Procuring Entity may, at its discretion, ask any tenderer for a clariﬁcation of its tender, given a reasonable time for a response. Any clariﬁcation submitted by a tenderer that is not in response to a request by the Procuring Entity shall not be considered. The Procuring Entity's request for clariﬁcation and the response shall be in writing. No change, including any voluntary increase or decrease, in the prices or substance of the tender shall be sought, offered, or permitted, except to conﬁrm the correction of arithmetic errors discovered by the Procuring Entity in the evaluation of the tenders, in accordance with ITT 31.

    - If a tenderer does not provide clariﬁcations of its tender by the date and time set in the Procuring

Entity's request for clariﬁcation, its Tender may be rejected.

  - Deviations, Reservations, and Omissions

    - During the evaluation of tenders, the following deﬁnitions apply:

      - “Deviation” is a departure from the requirements speciﬁed in the tender document;

      - “Reservation” is the setting of limiting conditions or withholding from complete acceptance of the requirements speciﬁed in the tender document; and

      - “Omission” is the failure to submit part or all of the information or documentation required in the Tender document.

  - Determination of Responsiveness

    - The Procuring Entity's determination of a Tender's responsiveness is to be based on the contents of the tender itself, as deﬁned in ITT 11.

    - A substantially responsive Tender is one that meets the requirements of the Tender document without material deviation, reservation, or omission. A material deviation, reservation, or omission is one that, if accepted, would:

      - Affect in any substantial way the scope, quality, or performance of the Works speciﬁed in the Contract; or

      - limit in any substantial way, inconsistent with the tender document, the Procuring Entity's rights or the tenderer's obligations under the proposed contract; or

      - if rectiﬁed, would unfairly affect the competitive position of other tenderers presenting substantially responsive tenders.

    - The Procuring Entity shall examine the technical aspects of the tender submitted in accordance with ITT 16, to conﬁrm that all requirements of Section VII, Works' Requirements have been met without any material deviation, reservation or omission.

    - If a tender is not substantially responsive to the requirements of the tender document, it shall be rejected by the Procuring Entity and may not subsequently be made responsive by correction of the material deviation, reservation, or omission.

  - Non-material Non-conformities

    - Provided that a tender is substantially responsive, the Procuring Entity may waive any non-

conformities in the tender.

    - Provided that a Tender is substantially responsive, the Procuring Entity may request that the tenderer submit the necessary information or documentation, within a reasonable period of time, to rectify nonmaterial non- conformities in the tender related to documentation requirements. Requesting information or documentation on such non-conformities shall not be related to any aspect of the price of the tender. Failure of the tenderer to comply with the request may result in the rejection of its tender.

    - Provided that a tender is substantially responsive, the Procuring Entity shall rectify quantiﬁable nonmaterial non-conformities related to the Tender Price. To this effect, the Tender Price shall be adjusted, for comparison purposes only, to reﬂect the price of a missing or non-conforming item or component in the manner speciﬁed in the TDS.

  - Arithmetical Errors

    - The tender sum as submitted and read out during the tender opening shall be absolute and ﬁnal and shall not be the subject of correction, adjustment or amendment in anyway by any person or entity.

    - Provided that the Tender is substantially responsive, the Procuring Entity shall handle errors on the

following basis:

      - Any error detected if considered a major deviation that affects the substance of the tender, shall lead to disqualiﬁcation of the tender as non-responsive.

      - Any errors in the submitted tender arising from a miscalculation of unit price, quantity, sub-total and total bid price shall be considered as a major deviation that affects the substance of the tender and shall lead to disqualiﬁcation of the tender as non-responsive. and

      - If there is a discrepancy between words and ﬁgures, the amount in words shall prevail

    - Tenderers shall be notiﬁed of any error detected in their bid during the notiﬁcation of award.

  - Conversion to Single Currency

For evaluation and comparison purposes, the currency (ies) of the Tender shall be converted into a single currency as speciﬁed in the TDS.

  - Margin of Preference and Reservations

    - A margin of preference may be allowed only when the contract is open to international competitive tendering where foreign contractors are expected to participate in the tendering process and where the contract exceeds the value/threshold speciﬁed in the Regulations.

    - A margin of preference shall not be allowed unless it is speciﬁed so in the TDS.

    - Contracts procured on basis of international competitive tendering shall not be subject to reservations exclusive to speciﬁc groups as provided in ITT 33.4.

    - Where it is intended to reserve a contract to a speciﬁc group of businesses (these groups are Small and Medium Enterprises, Women Enterprises, Youth Enterprises and Enterprises of persons living with disability, as the case may be), and who are appropriately registered as such by the authority to be speciﬁed in the TDS, a procuring entity shall ensure that the invitation to tender speciﬁcally indicates that only businesses or ﬁrms belonging to the speciﬁed group are eligible to tender. No tender shall be reserved to more than one group. If not so stated in the Invitation to Tender and in the Tender documents, the invitation to tender will be open to all interested tenderers.

  - Nominated Subcontractors

    - Unless otherwise stated in the TDS, the Procuring Entity does not intend to execute any speciﬁc

elements of the Works by subcontractors selected/nominated by the Procuring Entity. In case the Procuring Entity nominates a subcontractor, the subcontract agreement shall be signed by the Subcontractor and the Procuring Entity. The main contract shall specify the working arrangements between the main contractor and the nominated subcontractor.

    - Tenderers may propose subcontracting up to the percentage of total value of contracts or the volume of works as speciﬁed in the TDS. Subcontractors proposed by the Tenderer shall be fully qualiﬁed for their parts of the Works.

    - Domestic subcontractor's qualiﬁcations shall not be used by the Tenderer to qualify for the Works unless their specialized parts of the Works were previously designated so by the Procuring Entity in the TDS as can be met by subcontractors referred to hereafter as 'Specialized Subcontractors', in which case, the qualiﬁcations of the Specialized Subcontractors proposed by the Tenderer may be added to the qualiﬁcations of the Tenderer.

  - Evaluation of Tenders

    - The Procuring Entity shall use the criteria and methodologies listed in this ITT and Section III, Evaluation and Qualiﬁcation Criteria. No other evaluation criteria or methodologies shall be permitted. By applying the criteria and methodologies the Procuring Entity shall determine the Lowest Evaluated Tender in accordance with ITT 40.

    - To evaluate a Tender, the Procuring Entity shall consider the following:

      - Price adjustment in accordance with ITT 31.1(iii); excluding provisional sums and contingencies, if any, but including Day work items, where priced competitively;

      - Price adjustment due to discounts offered in accordance with ITT 14.4;

      - converting the amount resulting from applying (a) and (b) above, if relevant, to a single currency in accordance with ITT 32;

      - price adjustment due to quantiﬁable non-material non-conformities in accordance with ITT 30.3; and

      - any additional evaluation factors speciﬁed in the TDS and Section III, Evaluation and Qualiﬁcation Criteria.

    - The estimated effect of the price adjustment provisions of the Conditions of Contract, applied over the period of execution of the Contract, shall not be considered intender evaluation.

    - Where the tender involves multiple lots or contracts, the tenderer will be allowed to tender for one or more lots (contracts). Each lot or contract will be evaluated in accordance with ITT 35.2. The methodology to determine the lowest evaluated tenderer or tenderers based one lot (contract) or based on a combination of lots (contracts), will be speciﬁed in Section III, Evaluation and Qualiﬁcation Criteria. In the case of multiple lots or contracts, tenderer will be will be required to prepare the Eligibility and Qualiﬁcation Criteria Form for each Lot.

  - Comparison of Tenders

The Procuring Entity shall compare the evaluated costs of all substantially responsive Tenders established in accordance with ITT 35.2 to determine the Tender that has the lowest evaluated cost.

  - Abnormally Low Tenders and Abnormally High Tenders Abnormally Low Tenders

    - An Abnormally Low Tender is one where the Tender price, in combination with other elements of the Tender, appears so low that it raises material concerns as to the capability of the Tenderer in regards to the Tenderer's ability to perform the Contract for the offered Tender Price or that genuine

competition between Tenderers is compromised.

    - In the event of identiﬁcation of a potentially Abnormally Low Tender, the Procuring Entity shall seek written clariﬁcations from the Tenderer, including detailed price analyses of its Tender price in relation to the subject matter of the contract, scope, proposed methodology, schedule, allocation of risks and responsibilities and any other requirements of the Tender document.

    - After evaluation of the price analyses, in the event that the Procuring Entity determines that the Tenderer has failed to demonstrate its capability to perform the Contract for the offered Tender Price, the Procuring Entity shall reject the Tender.

Abnormally High Tenders

    - An abnormally high tender price is one where the tender price, in combination with other constituent elements of the Tender, appears unreasonably too high to the extent that the Procuring Entity is concerned that it (the Procuring Entity) may not be getting value for money or it may be paying too high a price for the contract compared with market prices or that genuine competition between Tenderers is compromised.

    - In case of an abnormally high price, the Procuring Entity shall make a survey of the market prices, check if the estimated cost of the contract is correct and review the Tender Documents to check if the speciﬁcations, scope of work and conditions of contract are contributory to the abnormally high tenders. The Procuring Entity may also seek written clariﬁcation from the tenderer on the reason for the high tender price. The Procuring Entity shall proceed as follows:

- If the tender price is abnormally high based on wrong estimated cost of the contract, the Procuring Entity may accept or not accept the tender depending on the Procuring Entity's budget considerations.

- If speciﬁcations, scope of work and/or conditions of contract are contributory to the abnormally high tender prices, the Procuring Entity shall reject all tenders and may retender for the contract based on revised estimates, speciﬁcations, scope of work and conditions of contract, as the case may be.

    - If the Procuring Entity determines that the Tender Price is abnormally too high because genuine competition between tenderers is compromised (often due to collusion, corruption or other manipulations), the Procuring Entity shall reject all Tenders and shall institute or cause competent Government Agencies to institute an investigation on the cause of the compromise, before retendering.

  - Unbalanced and/or Front-Loaded Tenders

    - If in the Procuring Entity's opinion, the Tender that is evaluated as the lowest evaluated price is seriously unbalanced and/or front loaded, the Procuring Entity may require the Tenderer to provide written clariﬁcations. Clariﬁcations may include detailed price analyses to demonstrate the consistency of the tender prices with the scope of works, proposed methodology, schedule and any other requirements of the Tender document.

    - After the evaluation of the information and detailed price analyses presented by the Tenderer, the Procuring Entity may as appropriate:

      - accept the Tender; or

      - require that the total amount of the Performance Security be increased at the expense of the Tenderer to a level not exceeding a 10% of the Contract Price; or

      - agree on a payment mode that eliminates the inherent risk of the Procuring Entity paying too much for undelivered works; or

      - reject the Tender,

  - Qualiﬁcations of the Tenderer

    - The Procuring Entity shall determine to its satisfaction whether the eligible Tenderer that is selected as having submitted the lowest evaluated cost and substantially responsive Tender, meets the qualifying criteria speciﬁed in Section III, Evaluation and Qualiﬁcation Criteria.

    - The determination shall be based upon an examination of the documentary evidence of the Tenderer's qualiﬁcations submitted by the Tenderer, pursuant to ITT 17. The determination shall not take into consideration the qualiﬁcations of other ﬁrms such as the Tenderer's subsidiaries, parent entities, afﬁliates, subcontractors (other than Specialized Subcontractors if permitted in the Tender document), or any other ﬁrm(s) different from the Tenderer.

    - An afﬁrmative determination shall be a prerequisite for award of the Contract to the Tenderer. A negative determination shall result in disqualiﬁcation of the Tender, in which event the Procuring Entity shall proceed to the Tenderer who offers a substantially responsive Tender with the next lowest evaluated price to make a similar determination of that Tenderer's qualiﬁcations to perform satisfactorily.

  - Lowest Evaluated Tender

Having compared the evaluated prices of Tenders, the Procuring Entity shall determine the Lowest Evaluated Tender. The Lowest Evaluated Tender is the Tender of the Tenderer that meets the Qualiﬁcation Criteria and whose Tender has been determined to be:

- Most responsive to the Tender document; and

- The lowest evaluated price.

  - Procuring Entity's Right to Accept Any Tender, and to Reject Any or All Tenders.

The Procuring Entity reserves the right to accept or reject any Tender and to annul the Tender process and reject all Tenders at any time prior to Contract Award, without there by incurring any liability to Tenderers. In case of annulment, all Tenders submitted and speciﬁcally, Tender securities, shall be promptly returned to the Tenderers.

- AWARD OF CONTRACT

  - Award Criteria

The Procuring Entity shall award the Contract to the successful tenderer whose tender has been determined to be the Lowest Evaluated Tender.

  - Notice of Intention to enter into a Contract

Upon award of the contract and Prior to the expiry of the Tender Validity Period the Procuring Entity shall issue a Notiﬁcation of Intention to Enter into a Contract/Notiﬁcation of award to all tenderers which shall contain, at a minimum, the following information:

- the name and address of the Tenderer submitting the successful tender;

- the Contract price of the successful tender;

- a statement of the reason(s) the tender of the unsuccessful tenderer to whom the letter is addressed was unsuccessful, unless the price information in (c) above already reveals the reason;

- the expiry date of the Standstill Period; and

- instructions on how to request a debrieﬁng and/or submit a complaint during the stand still period;

  - Stand still Period

  - The Contract shall not be signed earlier than the expiry of a Standstill Period of 14 days to allow

any dissatisﬁed tender to launch a complaint. Where only one Tender is submitted, the Standstill Period shall not apply.

  - Where a Standstill Period applies, it shall commence when the Procuring Entity has transmitted to each Tenderer the Notiﬁcation of Intention to Enter into a Contract with the successful Tenderer.

  - Debrieﬁng by the Procuring Entity

    - On receipt of the Procuring Entity's Notiﬁcation of Intention to Enter into a Contract referred to in ITT 43, an unsuccessful tenderer may make a concern(s) regarding their tender. The Procuring Entity shall provide the debrieﬁng within ﬁve days of receipt of the request.

    - Debrieﬁngs of unsuccessful Tenderers may be done in writing or verbally. The Tenderer shall bear its own costs of attending such a debrieﬁng meeting.

  - Letter of Award

Prior to the expiry of the Tender Validity Period and upon expiry of the Standstill Period speciﬁed in ITT 42.1, upon addressing a complaint that has been ﬁled within the Standstill Period, the Procuring Entity shall transmit the Letter of Award to the successful Tenderer. The letter of award shall request the successful tenderer to furnish the Performance Security within 21 days of the date of the letter.

  - Signing of Contract

    - Upon the expiry of the fourteen days of the Notiﬁcation of Intention to enter into contract and upon the parties meeting their respective statutory requirements, the Procuring Entity shall send the successful Tenderer the Contract Agreement.

    - Within fourteen (14) days of receipt of the Contract Agreement, the successful Tenderer shall sign, date, and return it to the Procuring Entity.

    - The written contract shall be entered into within the period speciﬁed in the notiﬁcation of award and before expiry of the tender validity period.

  - Performance Security

    - Within twenty-one (21) days of the receipt of the Letter of Award from the Procuring Entity, the successful Tenderer shall furnish the Performance Security and, any other documents required in the TDS, in accordance with the General Conditions of Contract, subject to ITT 38.2 (b), using the Performance Security and other Forms included in Section X, Contract Forms, or another form acceptable to the Procuring Entity. A foreign institution providing a bank guarantee shall have a correspondent ﬁnancial institution located in Kenya, unless the Procuring Entity has agreed in writing that a correspondent bank is not required.

    - Failure of the successful Tenderer to submit the above-mentioned Performance Security and other documents required in the TDS or sign the Contract shall constitute sufﬁcient grounds for the annulment of the award and forfeiture of the Tender Security. In that event the Procuring Entity may award the Contract to the Tenderer offering the next Best Evaluated Tender.

    - Performance security shall not be required for contract estimated to cost less than the amount speciﬁed in the Regulations.

  - Publication of Procurement Contract

Within fourteen days after signing the contract, the Procuring Entity shall publish the awarded contract at its notice boards and websites; and on the Website of the Authority. At the minimum, the notice shall contain the following information:

- name and address of the Procuring Entity;

- name and reference number of the contract being awarded, a summary of its scope and the selection method used;

- the name of the successful Tenderer, the ﬁnal total contract price, the contract duration.

- dates of signature, commencement and completion of contract;

- names of all Tenderers that submitted Tenders, and their Tender prices as read out at Tender opening.

  - Procurement Related Complaint

The procedures for making Procurement-related Complaints shall be speciﬁed in the TDS.

SECTION III - TENDER DATA SHEET (TDS)

The following speciﬁc data for the Works and Services to be procured shall complement, supplement, or amend the provisions in the Instructions to Tenderers (ITT). Whenever there is a conﬂict, the provisions herein shall prevail over those in ITT.

| ITT REF. | ITT DESCRIPTION |
| --- | --- |
| A. General |  |
| ITT 1 | The name of the contract is: FOR SPOT IMPROVEMENT OF UFANISI-ENDO KABOR ROAD The Reference number of the Contract is: Tender No: KeRRA/008/KEMA/KS/039/GOK/22/23-106 |
| ITT 2.3 | The information made available to competing firms is as follows: NONE |
| ITT 2.4 | The firms that provided consultancy services for the contract being tendered for are: NONE |
| ITT 3.1 | Maximum number of Joint Venture JV shall be Two (2). |
| ITT 3.12 | Valid NCA Registration Certificate and Practicing License in the required category; shall be provided as detailed in the Evaluation/Qualification Criteria SECTION IV |
| B. |  |
| ITT 7.1 | - The Tenderer will submit any request for clarification in writing at the Address: Provided in the detailed Tender Notice To reach the Procuring Entity not later than seven (7) days prior to the deadline of bid submission. - The Procuring Entity will publish the response at the Website www.kerra.go.ke or Regional Notice Board |
| ITT 7.2 | A) A Pre-arranged pretender site visit shall take place as per the tender notice |
| ITT 7.3 | The Tenderer will submit any questions in writing, to reach the Procuring Entity not later than the date specified in TDS- ITT 7.1 |
| ITT 7.4 | Not applicable |
| ITT 7.5 | The Procuring Entity's website where Minutes of the pre-Tender meeting and the pre- arranged pretender will be published is www.kerra.go.ke if applicable |
| C. Preparation of Tenders |  |
| ITT 11.1 (h) | The Tenderer shall submit the following additional documents in its Tender: As indicated in the Qualification Form/Criteria |
| ITT 13.1 | Alternative Tenders shall NOT be considered. |
| ITT 13.2 | Alternative times for completion shall NOT be permitted. |
| ITT 13.4 | Alternative technical solutions shall NOT be permitted for any parts of the Works. |
| ITT 14.5 | The prices quoted by the Tenderer shall be: Fixed and not subject to adjustments |
| ITT 15.2 | Foreign currency requirements not allowed. |
| ITT 18.1 | The Tender validity period shall be 175 days from the specified date of opening as indicated in the invitation to Tender |

| ITT REF. | ITT DESCRIPTION |
| --- | --- |
| ITT 18.2 | a) The Number of days beyond the expiry of the initial tender validity period will be30 days. The Tender price shall be adjusted by the following percentages of the tender price: - By 0 % of the local currency portion of the Contract price adjusted to reflect local inflation during the period of extension, and - By 0 % the foreign currency portion of the Contract price adjusted to reflect the international inflation during the period of extension. |
| ITT 19.1 | Where applicable a Tender Security of the amount APPLICABLE Shall be required. Where applicable A Tender-Securing Declaration Form Shall be required. |
| ITT 19.2 | The other security is Applicable should be in the form of: - a bank guarantee or - a guarantee by an insurance company registered and licensed by the Insurance Regulatory Authority listed by the Authority or - a guarantee issued by a ﬁnancial institution approved and licensed by the Central Bank of Kenya, from a reputable source |
| ITT 19.3 | Delete “If an unconditional bank guarantee is issued by a bank located outside Kenya, the issuing bank shallhave a correspondent bank located in Kenya to make it enforceable” Where applicable “The Tender Security shall be valid for thirty (30) days beyond the original validity period of the Tender, or beyond any period of extension if requested under ITT 18.2” |
| ITT 19.5 | Other documents required are as specified in Form No. 3; Contract Agreement |
| ITT 20.1 | In addition to the original of the Tender, the number of copies is: NONE |
| ITT 20.3 | The written confirmation of authorization to sign on behalf of the Tenderer shall consist of: Certificate of Independent Tender Determination Part B of Confidential Business Questionnaire and a power of attorney executed/attested by Commissioner of Oaths |
| D. Submission and Opening of Tenders |  |
| ITT 21.2 | A tender package or container that cannot fit in the tender box shall be received as follows: shall be received at the Supply Chain Management Offices of the location specified in the tender notice. |
| ITT 22.1 | (A) For Tender submission purposes only, the Procuring Entity's address is: As indicated in the Invitation to Tender Tenders shall not be submitted electronically. |
| ITT 25.1 | If Tenderers are allowed to submit Tenders electronically, they shall follow the electronic tender submission procedures specified below Not Applicable Tender opening dates and time shall be as indicated in the invitation to tender |
| ITT 25.6 | The number of representatives appointed to the Tender Opening Members of the Procuring Entity to sign is at least three. The Committee shall initial - cover page, - form of tender and - Bills of Quantities - the last page of the submitted bid and - where applicable Tender Securities and/or Tender-Securing Declaration Forms |
| E. Evaluation, and Comparison of Tenders |  |
| ITT 30.3 | For Comparison purposes only; The adjustment shall be based on the average price of the item or component as quoted on other substantially responsive Tenders. If the price of the item or component cannot be derived from the price of other substantially responsive Tenders, the Procuring Entity shall use its Lowest estimate. The Bidders Quoted Bid sum shall not change. |

| ITT REF. | ITT DESCRIPTION |
| --- | --- |
| ITT 31.2 | The error shall be considered a major deviation that leads to disqualification |
| ITT 32 | The currency that shall be used for Tender evaluation and comparison purposes to convert at the selling exchange rate all Tender prices expressed in various currencies into a single currency is-: Kenya Shillings |
| ITT 33.2 | A margin of preference shall NOT apply. |
| ITT 33.4 | As indicated by invitation to tender |
| ITT 34.1 | At this time, the Procuring Entity does not intend to execute certain specific parts of the Works by subcontractors selected in advance. |
| ITT 34.2 | Contractor's may propose subcontracting: Maximum percentage of subcontracting permitted is: 40% of the total contract amount. Tenderers planning to subcontract more than 10% of total volume of work shall specify, in the Form of Tender, the activity (ies) or parts of the Works to be subcontracted along with complete details of the subcontractors and their qualification and experience. |
| ITT 34.3 | The parts of the Works for which the Procuring Entity permits Tenderers to propose Specialized Subcontractors are designated as follows: Any Part of the works For the above-designated parts of the Works that may require Specialized Subcontractors, the relevant qualifications of the proposed Specialized Subcontractors will be added to the qualifications of the Tenderer for the purpose of evaluation. Applicable |
| ITT 35.2 (e) | Additional requirements apply. These are detailed in the evaluation criteria in Section IV, Evaluation and Qualification Criteria. |
| ITT 39 | Additional requirements are: As detailed in the Qualification Criteria/Form |
| ITT 48.1 | Performance security shall be in the form of unconditional Bank guarantee of 5% of the tender sum Performance security (where applicable) shall be in the form of unconditional Bank guarantee of 1% of the tender sum for reserved tenders (contracts reserved for special groups) Delete “A foreign institution providing a bank guarantee shall have a correspondent ﬁnancial institution located in Kenya, unless the Procuring Entity has agreed in writing that a correspondent bank is not required” |
| ITT 48.3 | The Performance Security shall be for contracts above Kenya Shillings Six Million |
| ITT 49 | The procedures for making a Procurement-related Complaint are available from the PPRA website info@ppra.go.ke or complaints @ppra.go.ke. If a Tenderer wishes to make a Procurement-related Complaint, the Tenderer should submit its complaint following these procedures, in writing (by the quickest means available, that is either by hand delivery or email to: For the attention: The Deputy Director Procuring Entity: Kenya Rural Roads Authority (KeRRA) Email address: keiyomarakwet@kerra.go.ke In summary, a Procurement-related Complaint may challenge any of the following: - The terms of the Tender Documents; and - The Procuring Entity's decision to award the contract. |
| ITT 50 | The procedures for making Procurement-related Complaints shall be speciﬁed in Special Conditions of Contracts |

SECTION IV- EVALUATION AND QUALIFICATION CRITERIA

General Provisions

- General Provisions

  - This section contains the criteria that the Employer shall use to evaluate tender and qualify tenderers. No other factors, methods or criteria shall be used other than speciﬁed in this tender document. The Tenderer shall provide all the information requested in the forms included in Section IV, Tendering Forms. The Procuring Entity shall use the Standard Tender Evaluation Document for Goods and Works for evaluating Tenders.

  - Wherever a Tenderer is required to state a monetary amount, Tenderers should indicate the Kenya Shilling equivalent using the rate of exchange determined as follows:

    - For construction turnover or ﬁnancial data required for each year - Exchange rate prevailing on the last day of the respective calendar year (in which the amounts for that year is to be converted) was originally established.

    - Value of single contract - Exchange rate prevailing on the date of the contract signature.

    - Exchange rates shall be taken from the publicly available source identiﬁed in the ITT 14.3. Any error in determining the exchange rates in the Tender may be corrected by the Procuring Entity.

  - Evaluation and contract award Criteria

The Procuring Entity shall use the criteria and methodologies listed in this Section to evaluate tenders and arrive at the Lowest Evaluated Tender. The tender that (i) meets the qualiﬁcation criteria,

(ii) has been determined to be substantially responsive to the Tender Documents, and (iii) is determined to have the Lowest Evaluated Tender price shall be selected for award of contract.

- Preliminary examination for Determination of Responsiveness

The Procuring Entity will start by examining all tenders to ensure they meet in all respects the eligibility criteria and other requirements in the ITT, and that the tender is complete in all aspects in meeting the requirements of “Part 2 – Procuring Entity's Works Requirements”, including checking for tenders with unacceptable errors, abnormally low tenders, abnormally high tenders and tenders that are front loaded. The Standard Tender Evaluation Report for Goods and Works for evaluating Tenders provides clear guidelines on how to deal with review of these requirements. Tenders that do not pass the Preliminary Examination will be considered irresponsive and will not be considered further.

[The Procuring Entity will provide the preliminary evaluation criteria. To facilitate, a template may be attached or clearly described all information and list of documentation to be submitted by Tenderers to enable preliminary evaluation of the Tender]

- Tender Evaluation (ITT 35)

Price evaluation: In addition to the criteria listed in ITT 35.2 (a) – (d) the following criteria shall apply:

  - Alternative Completion Times, if permitted under ITT 13.2, will be evaluated as follows:

N/A

  - Alternative Technical Solutions for speciﬁed parts of the Works, if permitted under ITT 13.4, will be evaluated as follows: N/A

  - Other Criteria; if permitted under ITT 35.2(d): N/A

- Multiple Contracts

  - Multiple contracts will be permitted in accordance with ITT 35.4. Tenderers are evaluated on basis of

Lots and the lowest evaluated tenderer identiﬁed for each Lot. The Procuring Entity will select one Option of the two Options listed below for award of Contracts.

OPTION 1

    - If a tenderer wins only one Lot, the tenderer will be awarded a contract for that Lot, provided the tenderer meets the Eligibility and Qualiﬁcation Criteria for that Lot.

    - If a tenderer wins more than one Lot, the tender will be awarded contracts for all won Lots, provided the tenderer meets the aggregate Eligibility and Qualiﬁcation Criteria for all the Lots.The tenderer will be awarded the combination of Lots for which the tenderer qualiﬁes and the others will be considered for award to second lowest the tenderers.

OPTION 2

The Procuring Entity will consider all possible combinations of won Lots [contract(s)] and determine the combinations with the lowest evaluated price. Tenders will then be awarded to the Tenderer or Tenderers in the combinations provided the tenderer meets the aggregate Eligibility and Qualiﬁcation Criteria for all the won Lots.

- Alternative Tenders (ITT 13.1)

An alternative if permitted under ITT 13.1, will be evaluated as follows:

The Procuring Entity shall consider Tenders offered for alternatives as speciﬁed in Part2-Works Requirements. Only the technical alternatives, if any, of the Tenderer with the Best Evaluated Tender conforming to the basic technical requirements shall be considered by the Procuring

- Margin of Preference

  - If the TDS so speciﬁes, the Procuring Entity will grant a margin of preference of ﬁfteen percent (15%)to be loaded one valuated price of the foreign tenderers, where the percentage of shareholding of Kenyan citizens is less than ﬁfty-one percent (51%).

  - Contractors applying for such preference shall be asked to provide, as part of the data for qualiﬁcation,such information, including details of ownership, as shall be required to determine whether, accordingto the classiﬁcation established by the Procuring Entity, a particular contract or or group of contractorsqualiﬁes for a margin of preference.

  - After Tenders have been received and reviewed by the Procuring Entity, responsive Tenders shall be assessed to ascertain their percentage of shareholding of Kenyan citizens. Responsive tenders to shallbe classiﬁed into the following groups:

    - Group A: tenders offered by Kenyan Contractors and other Tenderers where Kenyan citizens hold shares of over ﬁfty one percent (51%).

    - Group B: tenders offered by foreign Contractors and other Tenderers where Kenyan citizens hold shares of less than ﬁfty one percent (51%).

  - All evaluated tenders in each group shall, as a ﬁrst evaluation step, be compared to determine the lowest tender, and the lowest evaluated tender in each group shall be further compared with each other. If, as a result of this comparison, a tender from Group A is the lowest, it shall be selected for the award. If a tender from Group B is the lowest, an amount equal to the percentage indicated in Item 3.1 of the respective tender price, including unconditional discounts and excluding provisional sums and the cost of day works, if any, shall be added to the evaluated price offered in each tender from Group B. All tenders shall then be compared using new prices with added prices to Group Bandthe lowest evaluated tender from Group A. If the tender from Group A is still the lowest tender, it shall be selected for award. If not, the lowest evaluated tender from Group B based on the ﬁrst evaluation price shall be selected.

- Post qualiﬁcation and Contract award (ITT 39), more speciﬁcally,

- In case the tender was subject to post-qualiﬁcation, the contract shall be awarded to the lowest evaluated tenderer, subject to conﬁrmation of pre-qualiﬁcation data, if so required.

- In case the tender was not subject to post-qualiﬁcation, the tender that has been determined to be the lowest evaluated tenderer shall be considered for contract award, subject to meeting eachof the following conditions.

    - The Tenderer shall demonstrate that it has access to, or has available, liquid assets, unencumbered real assets, lines of credit, and other ﬁnancial means (independent of any contractual advance payment) sufﬁcient to meet the construction cash ﬂow estimated at a minimum of 35% of Engineer’s Estimate

    - Minimum average annual construction turnover of (Three times of the engineers estimates) Kenya Shillings 30 Million (equivalents calculated as total certiﬁed payments received for contracts in progress and/or completed within the last three (3) years 2020,2021, 2022.

    - For past experience:-

- Contracts of engineers estimates that are beyond 2.5 million, should provide at least two (2) contract(s) of a similar nature executed within Kenya, or the East African Community or abroad, that have been satisfactorily and substantially completed as a prime contractor, or joint venture member or sub-contractor each of minimum value Kenya shillings 10 M

- Contracts of engineers estimates that are between 1 million 2.5 million, should provide at least one (1) contract of a similar nature executed within Kenya, or the East African Community or abroad, that have been satisfactorily and substantially completed as a prime contractor, or joint venture member or sub-contractor each of minimum value Kenya shillings N/A

- Contracts of engineers estimates that are below 1 million, is not a must to provide contract of a similar nature executed within Kenya, or the East African Community or abroad, that have been satisfactorily and substantially completed as a prime contractor, or joint venture member or sub-contractor N/A

    - Contractor's Representative and Key Personnel, which are speciﬁed as a site agent with a minimum Qualification of Diploma in Civil Engineering and at least 5 years’ experience as a Site Agent.

Contractor’s key equipment listed on the table “Contractor's Equipment” below:

Contractor’s Equipment

| Item No. | Equipment Details | Minimum Number Required |
| --- | --- | --- |
| 1 | Pedestrian Roller – Man walk behind | 1 |
| 2 | Self-propelled single drum vibrating (10Tons) | 1 |
| 3 | Mobile concrete mixers | 1 |
| 4 | Excavator/loader | 1 |
| 5 | Concrete vibrators | 1 |
| 6 | Tippers payload 7 – 10 tonnes | 2 |
| 7 | Water tankers (10,000 litres capacity) | 1 |
| 8 | Motor Grader | 1 |
| 9 | Crawler Tractor | 1 |

  - Other conditions depending on their seriousness.

- History of non-performing contracts:

Tenderer and each member of JV incase the Tenderer is a JV, shall demonstrate that Non- performance of a contract did not occur because of the default of the Tenderer, or the member of

a JV in the last 3 years. The required information shall be furnished in the appropriate form.

- Pending Litigation

Financial position and prospective long-term proﬁtability of the Single Tenderer, and in the case the Tenderer is a JV, of each member of the JV, shall remain sound according to criteria established with respect to Financial Capability under Paragraph (i) above if all pending litigation will be resolved against the Tenderer. Tenderer shall provide information on pending litigations in the appropriate form.

- Litigation History

There shall be no consistent history of court/arbitral award decisions against the Tenderer, in the last 3 years. All parties to the contract shall furnish the information in the appropriate form about any litigation or arbitration resulting from contracts completed or ongoing under its execution over the years speciﬁed. A consistent history of awards against the Tenderer or any member of a JV may result in rejection of the tender.

EVALUATION /QUALIFICATION CRITERIA

To be qualified for award of Contract, the tenderer shall provide evidence satisfactory to the Employer of their eligibility and of their capability and adequacy of resources to effectively carry out the subject Contract. To this end, the tenderer shall be required to provide latest information set out below:

A. PRELIMINARY EXAMINATION

- Certified Certificate of Incorporation by Registrar of Companies.

- Current Certified CR12 Certificate (dated within 12 Months before date of opening) from the Registrar of Companies. This should be provided with Identification Documents of Directors and all individuals listed on the CR12. (ID or Passport). For Corporate Directors, CR12 or its equivalent for the corporate directors, Identification Documents for the corporate Director and its directors MUST be provided.

- Provide (where applicable) valid certified certificate of registration with National Treasury for Access to Government Procurement Opportunities (AGPO) in the relevant group (Women, Youth or Persons Living with Disability)-for PWD bidders should provide evidence of directors registration with the National Council of People Living with Disability (NCPWD).

- Valid Certified Current Single Business permit.

- Provide a Valid Tax Compliance Certificate or Tax exemption certificate issued by Kenya Revenue Authority in accordance with ITT 4.14.

- Tender Security (Where MANDATORY) in the amount specified in the tender Data Sheet. The Format to be as issued with this Tender and valid for the period specified under ITT 19. Alteration of the format shall lead to disqualification

- Duly filled Tender Securing Declaration Form (where applicable) shall be in the format issued in this Tender and validly executed in accordance with ITT 19. Alteration of the format shall lead to disqualification

- Current Certificate of Road works Registration with National Construction Authority in the Category NCA 8, 7 and 6 together with Road works valid NCA practicing license.

- Submit a written power of attorney authorizing the signatory of the bid to commit the Bidder Executed by a Commissioner for Oaths, in accordance with ITT 20.3.

- Declare conflicts of interest in accordance with ITT 3.3; provide a sworn-in affidavit executed by a Commissioner for Oaths specific to this tender.

- Dully filled, Signed and stamped (With Company Seal/ Rubber Stamp) ALL Tendering Forms and Schedules,

given under Section V of the tender document.

- Dully, Filled, Signed and Stamped Confidential Business Questionnaire

- Duly filled-in and Signed Form of Tender Pursuant to ITT 12

- Dully Priced Bills of Quantities Pursuant to ITT 14

- Pre-Tender Site Visit Certificate duly endorsed by the Authorized KeRRA Representative

- All pages of the Tender Document MUST be Chronologically Serialized (this should be sequential in the format of 1,2,3,4,5…….) from the first page to the last page. Pursuant to ITT 12.1.

- If Tender is a Joint Venture, should meet the requirements as given Pursuant to ITT3.5.

- Provide a Work Methodology relevant to the proposed works.

- Foreign Tenderers (where applicable) Must demonstrate and fulﬁl the 40% Local Content condition Pursuant to ITT 3.9.

- Bidders to demonstrate that Goods, equipment and services to be supplied under the contract to have their origin in any country that is not determined ineligible under ITT 4.1.

- Submission of Audited Accounts or equivalent acceptable to the Employer, for the last three [3] years to demonstrate: the current soundness of the applicant’s financial position and its prospective long-term profitability, and capacity to have a cash flow amount, turnover and working capital.

- Bidders must not have been declared ineligible by the PPRA as described in ITT 3.7.

- Declaration of Current ongoing Works.

- Attach proof of similar previous experience (attach award letters and completion certificates).

- Attach proof of equipment holding in the format provided.

- Bank statement in the name of the firm for at least Three (3) Months from the date of this tender.

- Authority to seek reference from Contractors’ bankers

QUALIFICATION FORM

| Item No. | Qualification Subject | Qualification Requirement | Document To be Completed/provided by Tenderer | For Procuring Entity's Use (Qualification met or Not Met) |
| --- | --- | --- | --- | --- |
| A. PRELIMINARY EVALUATION |  |  |  |  |
| 1. | Nationality | Nationality in accordance with ITT 4.5 | Forms ELI - 1.1, 1.2 and 1.3, with attachments |  |
| 2. | Tax Obligations for Kenyan Tenderers | Has produced a current tax clearance certificate or tax exemption certificate issued by Kenya Revenue Authority in accordance with ITT 3.14. | Form of Tender |  |
| 3. | Conflict of Interest | No conflicts of interest in accordance with ITT 4.3 | Form of Tender |  |
| 4. | PPRA Eligibility | Not having been declared ineligible by the PPRA as described in ITT 4.6 | Form of Tender - Form SD1 |  |
| 5. | State owned Enterprise | Meets conditions of ITT 4.7 | Forms ELI - 1.1 and 1.2, with attachments |  |
| 6. | Goods, equipment and services to be supplied under the contract | To have their origin in any country that is not determined ineligible under ITT 4.1 | Forms ELI - 1.4 |  |
| 7. | History of Non- Performing Contracts | Non-performance of a contract did not occur as a result of contractor default since 1st January {2020}. | Form CON-2 |  |
| 8. | Suspension Based on Execution of Tender / Proposal Securing Declaration by the Procuring Entity | Not under suspension based on- execution of a Tender/Proposal Securing Declaration pursuant to ITT4.8. | To be confirmed from Internal records by the procuring entity |  |
| 9. | Pending Litigation | Tender's financial position and prospective long- term profitability still sound according to criteria established in 3.1 and assuming that all pending litigation will NOT be resolved against the | Form CON - 1 |  |

| Item No. | Qualification Subject | Qualification Requirement | Document To be Completed/provided by Tenderer | For Procuring Entity's Use (Qualification met or Not Met) |
| --- | --- | --- | --- | --- |
|  |  | Tenderer |  |  |
| 10. | Litigation History | No consistent history of court/arbitral award decisions against the Tenderer for the last three (3) years. | Form CON - 1 |  |
| 11. | Declaration of Fair employment laws and practices | Bidders shall declare they are not guilty of any serious violation of fair employment laws and practices and will be bound to abide by the industry CBA at minimum | Form CON - 2 |  |
| 12. | Declaration of Knowledge of Site /Pre-Bid Conference | Attend Pre-Tender Site Visits as per TDS,ITT 8.1(mandatory) | Form CON - 3 |  |
| 13. | Tender Security | Tender Security document | Form in the Prescribed Format |  |
| 14. | Priced Bill of Quantities | - Fill all rates, and amounts, - NO Alterations of the Quantities accepted, - All bidders own Corrections must be Countersigned - NO Errors noted in the Bills of Quantities | Bills of Quantity in the Prescribed Format |  |
| 15. | Serialization of the Bid | Bidders shall sequentially serialize all pages of each tender submitted. Any written Pages or document attached or inserted Documents MUST be sequentially serialized. | The Serialization MUST be numerically sequential starting from Numeric 1. |  |
| 16. | Completeness of tender document | The person or persons signing the bid shall initial all pages of the bid where entries have been made. | All pages with entries (Typed or hand written) must be initialed. Any alterations made in the tender document must be countersigned. |  |

| Item No. | Qualification Subject | Qualification Requirement | Document To be Completed/provided by Tenderer | For Procuring Entity's Use (Qualification metor Not Met) |
| --- | --- | --- | --- | --- |
| B. TECHNICAL EVALUATION |  |  |  |  |
| 1. | History of Non- Performing Contracts | Non-performance of a contract did not occur as a result of contractor default for the last three (3) years. Non-performance shall be deemed to have occurred by evidence of: - Termination Letter - Liquidated Damages | Form CON-1 If a bidder fails to disclose, shall be disqualified Reference to be made to procuring Authority’s records A bidder with any history of non-performance earns zero(0) marks |  |
| 2. | Financial Capabilities | (i) The Tenderer shall demonstrate that it has access to, or has available, liquid assets, unencumbered real assets, lines of credit, and other financial means (independent of any contractual advance payment) sufficient to meet the construction cash flow requirements estimated as Kenya Shillings 3.3 M Tenderer's other commitments. (ii) The Tenderers shall also demonstrate, to the satisfaction of the Procuring Entity, that it has adequate sources of finance to meet the cash flow requirements on works currently in progress and for future contract commitments. The audited balance sheets or, if not required by the laws of the Tenderer' s country, other financial statements acceptable to the Procuring Entity, for the last 3 years shall be submitted and must demonstrate | Form FIN - 3.1, with attachments which includes: - Audited accounts All pages must be initialed and stamped by both a valid practicing Auditor registered with ICPAK and one of the Directors. - Line of Credit - Bank statements Etc. |  |

| ItemNo | Qualification Subject | Qualification Requirement | Document To be Completed/provided by Tenderer | For Procuring Entity's Use (Qualification metor Not Met) |
| --- | --- | --- | --- | --- |
|  |  | the current soundness of the Tenderer's financial position and indicate its prospective long-term profitability. |  |  |
|  | Average Annual Construction Turnover | (iii)Minimum average annual construction turnover of Kenya Shillings 30 Million, equivalent calculated as total certified payments received for contracts in progress and/or completed within the last 3 Years. | Form FIN - 3.2 Attachments include Financial Statements |  |
| 3. | Ongoing Works | Value of outstanding works shall not be more than the Engineer`s Estimate | Form FIN - 3.4 If the outstanding Works is more than the Engineer`s Estimate of this bid, the bidder loses |  |

| Item No. | Qualification Subject | Qualification Requirement | Document To be Completed/provided by Tenderer | For Procuring Entity's Use (Qualification metor Not Met) |
| --- | --- | --- | --- | --- |
|  | General Construction Experience | Experience under construction contracts in the role of prime contractor, JV member, sub- contractor, or management contractor, substantially completed in the last 3 years prior to the applications submission deadline. | Form EXP -4.1 Attach Letters of Award and Completion Certificates |  |
|  | Specific Construction &Contract Management Experience | Participation in at least Two similar Contract of minimum value of Kenya shillings 10 million for works equivalents in the last 3 years prior to the applications submission deadline. The similarity shall be based on the physical size, complexity, methods / technology or other characteristics | Form EXP 4.2(a)&(b) Provide Letters of Award and Completion Certificates For subcontracted works, the bidder should provide the following; - Award letter of the main contractor - Award letter of the subcontract. - Completion letter of the subcontract. - Subcontract approval from the Engineer/supervision Authority |  |

| Item No. | Qualification Subject | Qualification Requirement | Document To be Completed/provided by Tenderer | For Procuring Entity's Use (Qualification met or Not Met) |
| --- | --- | --- | --- | --- |
| 4. | Contractor's Representative and Key Personnel | Curriculum Vitae (CVs) of the Proposed Key Staff must be presented in the provided format and duly signed by the proposed individual. Copies of certificates and Annual Practicing Licenses (for Engineers) and Academic Certificates for all staff is mandatory; | Schedule F (Form PER. 1and PER. 2) |  |
|  |  | The site staff shall possess minimum levels set below; |  |  |
|  |  | Site Agent | Qualification = Degree/Diploma in Civil General experience =1 yr and above. Specific Experience =1 yr and above. |  |
|  |  | Senior Foreman | Qualification = Degree/Diploma in Civil Eng. General Experience =1 yr and above. Specific Experience = 1 yr and above. |  |
|  |  | Site Surveyor | Qualification = Degree/Diploma in Survey General Experience =1 yr and above. Specific Experience =1 yr and above. |  |
| 5. | Contractors key equipment | - Bidders shall declare they have possession/Ownership of various equipment as proposed to be used in the Project by providing Logbooks that demonstrate proof of ownership - For Bidders planning to hire, they shall provide an Active Lease Agreement in Place that can be used | FORM EQU: EQUIPMENT UNDER SECTION V – TENDERING FORMS |  |

| Item No. | Qualification Subject | Qualification Requirement |  |  | Document To be Completed/provided by Tenderer |  |  | For Procuring Entity's Use (Qualification metor Not Met) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | during the Project Life. The copy of logbooks of the lessor(s) shall also be provided. |  |  |  |  |  |  |  |  |
|  |  |  | Main Scope of Works of this Tender | Main Equipment |  | Quantity (No) (Minimum) | Marks (Score) |  |  |  |
|  |  |  |  |  |  |  | Owned |  | Hired/ leased |  |
|  |  |  | Earth Works, Drainage Works & Grading and Gravelling Works | Motor Grader |  | 1 |  |  |  |  |
|  |  |  |  | Water Bowser |  | 1 |  |  |  |  |
|  |  |  |  | Single Drum Roller |  | 1 |  |  |  |  |
|  |  |  |  | Excavator/Loader |  | 1 |  |  |  |  |
|  |  |  |  | Tippers payload 7-10 tonnes |  | 2 |  |  |  |  |
|  |  |  | Total |  |  |  |  |  |  |  |
| 6. | Proposed methodology | Adequacy and quality of the proposed methodology |  |  | a) Technical approach and methodology - Provided a detailed Work Methodology |  |  |  |  |  |
|  |  |  |  |  | - Provided a Methodology on safety during the construction period |  |  |  |  |  |
|  |  |  |  |  | - Provided a specific Quality management plan |  |  |  |  |  |

| Item No. | Qualification Subject | Qualification Requirement | Document To be Completed/provided by Tenderer | For Procuring Entity's Use (Qualification metor Not Met) |
| --- | --- | --- | --- | --- |
|  |  |  | b) Work plan/Program of Works (PoW) |  |
|  |  |  | - PoW Resourced with Equipment-Min. allocation pursuant to the Schedule …. of Technical Proposal - – To be submitted in well legible Fonts |  |
|  |  |  | - PoW captures Monthly outputs for each activity |  |
|  |  |  | - PoW details BoQ Quantities, Units and Rates |  |
|  |  |  | - PoW is superimposed with Cash flow Projections as detailed in Schedule ….. of the technical proposal |  |
|  |  |  | c) Site Organization |  |
|  |  |  | and stafﬁng |  |
|  |  |  | (Schedule of Technical proposal) |  |

| Item No. | Qualification Subject | Qualification Requirement | Document To be Completed/provided by Tenderer | For Procuring Entity's Use (Qualification met or Not Met) |
| --- | --- | --- | --- | --- |
| 7. | Knowledge Transfer [When transfer of knowledge is a particularly important component of the assignment, the following sub- criteria may be provided] | Transfer of knowledge (training) program (relevance of approach and methodology | - Relevance of training program [Not Applicable] - Training approach and methodology [Not Applicable] - Qualiﬁcations of experts and trainers [Not Applicable] |  |
| Tenderers who pass the technical evaluation will be evaluated further. |  |  |  |  |
| C. FINANCIAL EVALUATION: Lowest Evaluated Bidder is subjected to Post Qualification Evaluation |  |  |  |  |
| D. POST QUALIFICATION: The procuring entity shall verify the documents provided by the bidder with the issuing authority. In line with ITT 3.12 Confirm Bidders Proof of registration with the Roadworks National Construction Authority in Category 8,7 and 6 as Roads/Bridges Contractor (Attach copy of Current NCA Practicing License); Tax Compliance Certificate; bid security, AGPO (where applicable); specific experience; equipment ownership; credit facility (determine financial capacity/soundness),audited company’s details and on-going works (where applicable) The procuring entity shall verify the documents provided by the bidder with the issuing authority. |  |  |  |  |

APPENDIX TO THE QUALIFICATION CRITERIA

| ITEM | DESCRIPTION | Qualification Requirement | Document To be Completed/provided by Tenderer |
| --- | --- | --- | --- |
| 1 | HISTORY OF NON-PERFORMANCE |  |  |
|  | History of Non-Performance | Non-performance of a contract did not occur as a result of contractor default for the last three (3) years. Non-performance shall be deemed to have occurred by evidence of: - Termination Letter - Liquidated Damages | Form CON-1 If a bidder fails to disclose, shall be disqualified Reference to be made to procuring Authority’s records A bidder with any history of non-performance shall be disqualified |
| 2 | FINANCIAL CAPACITY |  |  |
|  | Audited Statements | - Bidders shall provide audited balance sheets or, if not required by the laws of the Tenderer's country, other financial statements acceptable to the Procuring Entity, for the shall be submitted and must demonstrate the current soundness of the Tenderer's financial position and indicate its prospective long-term profitability (as demonstrated by Financial Evaluation | - All pages must be initialed and stamped by both a practicing Auditor registered with ICPAK and one of the Directors. - Auditor’s practicing membership number from ICPAK must be indicated and a valid practicing license shall be provided |

| ITEM | DESCRIPTION | Qualification Requirement | Document To be Completed/provided by Tenderer |
| --- | --- | --- | --- |
|  | Computation of Financial Ratios | ratios). Firms incorporated within the last three years MUST demonstrate financial ability in the form of Line of Credit equivalent to required working capital. | Computation shall be made for the following Ratios and marks awarded to eachof the ratios: - Working Capital - Debt to Equity Ratio - Current ratio - Operating Cash Flowratio |
|  | Working capital | The Tenderer shall demonstrate that it has access to, or has available, liquid assets, unencumbered real assets, lines of credit, and other financial means (independent of any contractual advance payment) sufficient to meet the cash flow requirements estimated at a minimum of 35% of bid sum for the subject contract(s) net of the Tenderer's other commitments. The Tenderers shall also demonstrate, to the satisfaction of the Procuring Entity, that it has adequate sources of finance to meet the cash flow requirements on works currently in progress and for future contract commitments | - Line of Credit - Bank statements |
|  | Turnover | Minimum average annual construction turnover of Kenya Shillings 30 Million, equivalent calculated as total certified payments received for contracts in progress and/or completed within the last | Form FIN - 3.2 Attachments include Financial Statements |

| ITEM | DESCRIPTION |  |  |  | Qualification Requirement | Document To be Completed/provided by Tenderer |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 2 years. |  |
| 3 | EXPERIENCE |  |  |  |  |  |
|  | Workload Analysis |  |  |  |  |  |
|  | General Experience |  |  |  |  |  |
|  | Specific experience in related works |  |  |  |  |  |
| 4 | KEY PERSONNEL |  |  |  |  |  |
|  | Site Agent | Qualification |  | Degree |  |  |
|  |  |  |  | HND |  |  |
|  |  |  |  | Diploma |  |  |
|  |  | Relevant experience |  | 1yr and above |  |  |
| 5 | PLANT AND EQUIPMENT |  |  |  |  |  |
|  | Relevant Equipment (As Detailed in FORM EQU) |  | Owned |  |  |  |
|  |  |  | 100% Leased |  |  |  |
| 6 | PROGRAM OF WORKS AND WORK METHODOLOGY |  |  |  |  |  |
| 6a | Work Methodology | Provided a detailed Work Methodology |  |  |  |  |
|  |  | Provided a Methodology on safety during the construction period |  |  |  |  |
|  |  | Provided a specific Quality management plan |  |  |  |  |
| 6b | Program of Works | PoW Resourced with Equipment-Min. allocation pursuant to the Schedule E of Technical Proposal :- To be submitted in well legible Fonts |  |  |  |  |
|  |  | PoW captures Monthly outputs for each activity |  |  |  |  |
|  |  | PoW details BoQ Quantities, Units and |  |  |  |  |

| ITEM | DESCRIPTION |  | Qualification Requirement | Document To be Completed/provided by Tenderer |
| --- | --- | --- | --- | --- |
|  |  | Rates |  |  |
|  |  | PoW is superimposed with cash flow Projections as detailed in Schedule A of the technical proposal |  |  |
|  | Site base facilities | Site Organization and stafﬁng (Schedule B of Technical proposal). Proposal on office & stock yard facilities – identify location and condition of the planned site officeand stockyard. Attachment of map and photo preferably |  |  |
|  | Responsive/Non-responsive |  |  |  |

SECTION V - TENDERING FORMS

QUALIFICATION FORMS

- FORM EQU: EQUIPMENT

The Tenderer shall provide adequate information to demonstrate clearly that it has the capability to meet the requirements for the key equipment listed in Section III, Evaluation and Qualiﬁcation Criteria. A separate Form shall be prepared for each item of equipment listed, or for alternative equipment proposed by the Tenderer.

| Item of Equipment |  |  |
| --- | --- | --- |
| Equipment information | Name of manufacturer | Model and power rating |
|  | Capacity | Year of manufacture |
| Current Status | Current location |  |
|  | Details of Current Commitments |  |
| Source | Indicate source of the equipment Owned leased Rented Specially manufactured |  |
| Omit the following information for equipment owned by the Tenderer |  |  |
| Owner | Name of owner |  |
|  | Address of owner |  |
|  | Telephone | Contact name and tittle |
|  | Fax | Telex |
| Agreements | Details of rental/lease/ manufacturer agreements specific to the project |  |

- FORM PER -1

Contractor's Representative and Key Personnel Schedule

Tenderers should provide the names and details of the suitably qualiﬁed Contractor's Representative and Key Personnel to perform the Contract. The data on their experience should be supplied using the Form PER-2 below for each candidate.

Contractor' Representative and Key Personnel

| 1 | Title of position: Contractor’s Representative |  |
| --- | --- | --- |
|  | Name of Candidate: |  |
|  | Duration of appointment |  |
|  | Time commitment: for this position |  |
|  | Expected time schedule for this position |  |
| 2 | Title of position |  |
|  | Name of Candidate: |  |
|  | Duration of appointment |  |
|  | Time commitment: for this position |  |
|  | Expected time schedule for this position |  |
| 3 | Title of position: |  |
|  | Name of Candidate: |  |
|  | Duration of appointment |  |
|  | Time commitment: for this position |  |
|  | Expected time schedule for this position |  |
| 4 | Title of position: |  |
|  | Name of Candidate: |  |
|  | Duration of appointment |  |
|  | Time commitment: for this position |  |
|  | Expected time schedule for this position |  |
| 5 | Title of position: |  |
|  | Name of Candidate: |  |
|  | Duration of appointment |  |
|  | Time commitment: for this position |  |
|  | Expected time schedule for this position |  |

- FORM PER-2:

Resume and Declaration - Contractor's Representative and Key Personnel.

| Name of Tenderer |  |  |
| --- | --- | --- |
| Position[#]: [Tittle of position from Form PER-1] |  |  |
| Personnel Information | Name: | Date of birth |
|  | Address: | Email: |
|  | Professional qualifications: |  |
|  | Academic qualifications: |  |
|  | Language proficiency:[language and levels of speaking, reading and writing skills] |  |
| Details |  |  |
|  | Address of procuring Entity |  |
|  | Telephone: | Contact(manager/personnel officer): |
|  | Fax |  |
|  | Job Title | Years with present procuring Entity |

Summarize professional experience in reverse chronological order. Indicate particular technical and managerial experience relevant to the project.

| Project | Role | Duration of Involvement | Relevant Experience |
| --- | --- | --- | --- |
| [Main project details] | [role and responsibilities on the project] | [time in role] | [describe the experience relevant to this position] |

Declaration

I, the undersigned [insert either “Contractor's Representative” or “Key Personnel” as applicable], certify that to the best of my knowledge and belief, the information contained in this Form PER-2 correctly describes myself, my qualiﬁcations and my experience.

I conﬁrm that I am available as certiﬁed in the following table and throughout the expected time schedule for this position as provided in the Tender:

| Commitments | Details |
| --- | --- |
| Commitments to duration of contracts: |  |
| Time Commitment: |  |

I understand that any misrepresentation or omission in this Form may:

- be taken into consideration during Tender evaluation;

- result in my disqualiﬁcation from participating in the Tender;

- result in my dismissal from the contract.

Name of Contractor's Representative or Key Personnel: [insert name]

Name & Signature:

Date: (day month year):

Countersignature of authorized representative of the Tenderer:

Name & Signature:

Date: (day month year):

TENDERERS QUALIFICATION WITHOUT PRE-QUALIFICATION

To establish its qualiﬁcations to perform the contract in accordance with Section III, Evaluation and Qualiﬁcation Criteria the Tenderer shall provide the information requested in the corresponding Information Sheets included hereunder.

FORM ELI -1.1 Tenderer Information Form

Date:

ITT No.

Title:

Tenderer’s Name:

In case of Joint Venture (JV), Name of each member:

Tenderer’s actual or intended country of registration: [indicate country of constitution]

Tenderer’s actual or intended year of incorporation

Tenderer’s legal address [in country of registration]:

Tenderer ‘authorized representative information: Name: Address: Telephone / Fax numbers: E-mail address:

1. Attached are original documents of Articles of Incorporation (or equivalent document of constitution or association) and/or documents of registration of legal entity named above, in accordance with ITT 3.6 In case of JV, letter of intent to form JV or JV Agreement, in accordance with ITT 3.5 In case of state-owned enterprise or institution, in accordance with ITT3.8, documents establishing; - legal and financial autonomy, - Operation under commercial Law, - Establishing that the tenderer is not under the supervision of the Procuring Entity in accordance with ITT 3.8 2. Included are the organizational chart, a list of Board of Directors and the beneficial ownership.

FORM ELI -1.2

Tenderer's JV Information Form

(to be completed for each member of Tenderer's JV) Date:

ITT No.

Title:

1. Tenderer’s JV Name:

2. JV Member’s name:

3. JV Member’s country of registration

4. JV Member’s year of Constitution :

5. JV Member’s legal address in country of Constitution:

6. JV Member’s Authorized Representative Information Name: Address: Telephone/Fax numbers: Email Address:

- Attached are original documents of Articles of Incorporation( or equivalent document of constitution or association) and/or documents of registration of legal entity named above, in accordance with ITT 3.6 In case of state-owned enterprise or institution, documents establishing legal and financial autonomy, Operation under commercial Law, and that they are not under supervision of the Procuring Entity in accordance with ITT 3.8 - Included are the organizational chart, a list of Board of Directors and the beneficial ownership.

FORM CON – 2

Historical Contract Non-Performance, Pending Litigation and Litigation History

Tenderer's Name:  	                                                                                     Date:	 JV Member's Name

ITT No.

Title:

| Non-Performed contracts in accordance with section III, Evaluation and Qualifications |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Contract non-performance did not occur since 1st January [insert year] specified in section III |  |  |  |  |
| Evaluation and Qualification Criteria, Sub-Factor 2.1 Contract(s) not performed since 1st January [insert year] specified in section III Evaluation and Qualification Criteria, Sub-Factor 2.1 |  |  |  |  |  |  |
| Year |  |  | Non-performed portion of contract |  | Contract Identification | Total Contract Amount (current value, currency, exchange rate and Kenya Shilling equivalent) |
| [insert year] |  |  | [inset amount and percentage] |  | Contract Identification: [indicate complete contract name/ number, and any other identification] Name of Procuring Entity: [insert full name] Address of Procuring Entity: [insert street/city/country] Reason(s) for non-performance: [indicate main reason(s) |  |
| Pending Litigation, in accordance with section III, Evaluation and Qualification Criteria |  |  |  |  |  |  |
|  |  | No pending litigation in accordance with section III, Evaluation and Qualification Criteria, Sub-Factor 2.3 |  |  |  |  |
| Pending litigation in accordance with section III, Evaluation and Qualification Criteria, Sub-Factor 2.3 as indicated below. |  |  |  |  |  |  |
| Year of dispute |  |  |  | Amount in dispute (currency) | Contract Identification | Total Contract Amount (Currency), Kenya shillings Equivalent (Exchange Rate) |
|  |  |  |  |  | Contract Identification: Name of Procuring Entity: Address of Procuring Entity: Matter in dispute: Party who initiated the dispute: Status of dispute: |  |
|  |  |  |  |  | Contract Identification: Name of Procuring Entity: Address of Procuring Entity: Matter in dispute: Party who initiated the dispute: Status of dispute: |  |
| Litigation History in accordance with section III, Evaluation and Qualification Criteria. |  |  |  |  |  |  |
|  |  | No litigation History in accordance with section III, Evaluation and Qualification Criteria, Sub-Factor 2.4 |  |  |  |  |
| Litigation History in accordance with section III, Evaluation and Qualification Criteria, Sub-Factor 2.4 as indicated below. |  |  |  |  |  |  |
| Year of award |  |  |  | Outcome as percentage of Net Worth | Contract Identification | Total Contract Amount (Currency), Kenya shillings Equivalent (Exchange Rate) |
| [insert year] |  |  |  | [insert percentage] | Contract Identification: Name of Procuring Entity: Address of Procuring Entity: Matter in dispute: Party who initiated the dispute: Status of dispute: | [insert amount] |

- 58: FORM FIN – 3.1

Financial Situation and Performance

Tenderer's Name:  	                                                                                                     Date:		 JV Member's Name

ITT No. and title:

    - Financial Data

| Type of Financial Information in KShs (currency) | Historic information for previous (insert as appropriate) years, (Amount in currency (KShs)) |  |  |
| --- | --- | --- | --- |
|  | Year 1 | Year 2 | Year 3 |
| Statement of Financial Position (Information from Balance Sheet) |  |  |  |
| Total Assets(TA) |  |  |  |
| Total Liability (TL) |  |  |  |
| Total Equity/Net Worth (NW) |  |  |  |
| Current Assets (CA) |  |  |  |
| Current Liability (CL) |  |  |  |
| Working Capital (WC) |  |  |  |
| Information from Income Statement |  |  |  |
| Total Revenue (TR) |  |  |  |
| Profits Before Taxes (PBT) |  |  |  |
| Cash Flow Information |  |  |  |
| Cash Flow from Operating Activities |  |  |  |

Sources of Finance

Specify sources of ﬁnance to meet the cash ﬂow requirements on works currently in progress and for future contract commitments.

| No | Source of finance | Amount (Kenya shillings equivalent) |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |

Financial documents

The Tenderer and its parties shall provide copies of ﬁnancial statements for	years pursuant Section III, Evaluation and Qualiﬁcations Criteria, Sub-factor 3.1. The ﬁnancial statements shall:

      - reﬂect the ﬁnancial situation of the Tenderer or in case of JV member, and not an afﬁliated entity (such as parent company or group member).

      - be independently audited or certiﬁed in accordance with local legislation.

      - be complete, including all notes to the ﬁnancial statements.

      - correspond to accounting periods already completed and audited.

Attached are copies of ﬁnancial statements1 for the 	years	required above; and complying with the requirements

5.5 FORM FIN – 3.2

Average Annual Construction Turnover

Tenderer's Name:

Date:

JV Member's Name

ITT No.

Title:

| Annual Turnover data ( Construction only) |  |
| --- | --- |
| Year | Kenya Shillings |
| [indicate year] |  |
| Average Annual Construction Turnover |  |

* See Section III, Evaluation and Qualiﬁcation Criteria, Sub-Factor 3.2.

1If the most recent set of ﬁnancial statements is for a period earlier than 12 months from the date of Tender, the reason for this should be justiﬁed.

# 5. 60 FORM FIN – 3.3:

Financial Resources

Specify proposed sources of ﬁnancing, such as liquid assets, unencumbered real assets, lines of credit, and other ﬁnancial means, net of current commitments, available to meet the total construction cash ﬂow demands of the subject contract or contracts as speciﬁed in Section III, Evaluation and Qualiﬁcation Criteria

| Financial Resource |  |  |
| --- | --- | --- |
| No. | Source of financing | Amount (Kenya Shilling Equivalent) |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |

  - FORM FIN – 3.4:

Current Contract Commitments / Works in Progress

Tenderers and each member to a JV should provide information on their current commitments on all contracts that have been awarded, or for which a letter of intent or acceptance has been received, or for contracts approaching completion, but for which an unqualiﬁed, full completion certiﬁcate has yet to be issued.

| Current Contract Commitments |  |  |  |  |
| --- | --- | --- | --- | --- |
| No. | Contract Description | Name of Procurement entity | Value of outstanding work in Kshs. | Estimated completion date |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

  - FORM EXP - 4.1

General Construction Experience

Tenderer's Name:  	                                                                                     Date:	 JV Member's Name

ITT No.

Title:

Page	of	pages

| Starting Year | Ending Year | Contract Identification | Role of Tenderer |
| --- | --- | --- | --- |
|  |  | Contract name: Brief Description of the work performed by the tenderer: Amount of contract: Name of Procuring Entity: Address: |  |
|  |  | Contract name: Brief Description of the work performed by the tenderer: Amount of contract: Name of Procuring Entity: Address: |  |
|  |  | Contract name: Brief Description of the work performed by the tenderer: Amount of contract: Name of Procuring Entity: Address: |  |

  - FORM EXP - 4.2(a)

Speciﬁc Construction and Contract Management Experience

Tenderer's Name:  	                                               Date:		 JV Member's Name 		 ITT No. and title:

| Contract No. | Information |  |  |  |
| --- | --- | --- | --- | --- |
| Contract Identification |  |  |  |  |
| Award Date |  |  |  |  |
| Completion Date |  |  |  |  |
| Role in Contract | Prime Contractor | Member in JV | Management Contractor | Sub- contractor |
| Total Contract Amount |  |  |  | Kenya Shillings |
| If a member in JV or Sub- contractor specify participation in total contract amount |  |  |  |  |
| Procuring Entity Name |  |  |  |  |
| Address: Telephone/Fax Number: E-mail: |  |  |  |  |
| Description of the similarity in with Sub-Factor 4.2 (a) of Section III: |  |  |  |  |
| 1. Amount |  |  |  |  |
| 2. Physical size of required works items |  |  |  |  |
| 3. Complexity |  |  |  |  |
| 4. Methods/Technology |  |  |  |  |
| 5. Construction rate for key activities |  |  |  |  |
| 6. Other Characteristics |  |  |  |  |

10.1 FORM EXP- 4.2(b)

Construction Experience in Key Activities

Tenderer's Name:  		                                                                                    Date:		 Tenderer's JV Member Name: 	  Sub-contractor's Name2 (as per ITT 34): 	 ITT No.

Title:

All Sub-contractors for key activities must complete the information in this form as per ITT 34 and Section III, Evaluation and Qualiﬁcation Criteria, Sub-Factor 4.2.

KeyActivity No One: -

|  | Information |  |  |  |
| --- | --- | --- | --- | --- |
| Contract Identification |  |  |  |  |
| Award Date |  |  |  |  |
| Completion Date |  |  |  |  |
| Role in Contract | Prime Contractor | Member in JV | Management Contractor | Sub- contractor |
| Total Contract Amount | Kenya Shillings |  |  |  |
| Quantity (Volume, Number or rate of production, as applicable) Performed under the contract per year | Total Quantity in the contract (i) | Percentage participate (ii) | Actual Quantity Performed (i) x (ii) |  |
| Year 1 |  |  |  |  |
| Year 2 |  |  |  |  |
| Year 3 |  |  |  |  |
| Year 4 |  |  |  |  |
| Procuring Entity’s Name: |  |  |  |  |
| Address: Telephone/ Fax number E-mail: |  |  |  |  |
| Description of the key activities in according with Sub-Factor 4.2 (b) of Section III |  |  |  |  |

- Activity No. Two 3. …………………

If applicable

OTHER FORMS

  - FORM OFTENDER

INSTRUCTIONS TO TENDERERS

    - The Tenderer must prepare this Form of Tender on stationery with its letterhead clearly showing the Tenderer's complete name and business address.

    - All italicized text is to help Tenderer in preparing this form.

    - Tenderer must complete and sign CERTIFICATE OF INDEPENDENT TENDER DETERMINATION and the SELFDECLARATION OF THE TENDERER attached to this Form of Tender.

    - The Form of Tenders hall include the following Forms duly completed and signed by the Tenderer.

      - Tenderer's Eligibility- Conﬁdential Business Questionnaire

      - Certiﬁcate of Independent Tender Determination

      - Self-Declaration of the Tenderer

Date of this Tender submission:

Request for Tender No.:

Name and description of Tender

Alternative No.: N/A

To:	The Regional Director,

Kenya Rural Roads Authority,

Box 213-30070 ITEN

Dear Sirs,

- In accordance with the Conditions of Contract, Speciﬁcations, Drawings and Bills of Quantities for the execution of the above named Works, we, the undersigned offer to construct and complete the Works and remedy any defects therein for the sum of Kenya Shillings [[Amount in ﬁgures]	Kenya Shillings [amount in words]

- We under take if our tender is accepted, to commence the Works as soon as is reasonably possible after the receipt of the notice to commence, and to complete the whole of the Works comprised in the Contract within the time stated in the Special Conditions of Contract.

- We agree to adhere by this tender until	[Insert dates], and it shall remain binding upon us and may be accepted at any time before that date.

- Unless and until a formal Agreement is prepared and executed, this tender together with your written acceptance thereof, shall constitute a binding Contract between us. We further understand that you are not bound to accept the lowest or any tender you may receive.

- We, the undersigned, further declare that:

  - No reservations: We have examined and have no reservations to the tender document, including Addenda issued in accordance with ITT 28;

  - Eligibility: We meet the eligibility requirements and have no conﬂict of interest in accordance with ITT 3 and 4;

  - Tender-Securing Declaration: We have not been suspended nor declared ineligible by the Procuring Entity based on execution of a Tender-Securing or Proposal-Securing Declaration in the Procuring Entity's Country in accordance with ITT 19.8;

  - Conformity: We offer to execute in conformity with the tendering documents and in accordance with the implementation and completion speciﬁed in the construction schedule, the following Works: [insert a brief description of the Works];

  - Tender Price: The total price of our Tender, excluding any discounts offered in item 1 above is: [Insert one of the options below as appropriate]

- Tender Validity Period: Our Tender shall be valid for the period speciﬁed in TDS 18.1 (as amended, if applicable) from the date ﬁxed for the Tender submission deadline speciﬁed in TDS 22.1 (as amended, if applicable), and it shall remain binding upon us and may be accepted at any time before the expiration of that period;

- Performance Security: If our Tender is accepted, we commit to obtain a Performance Security in accordance with the Tendering document;

- One Tender Per Tender: We are not submitting any other Tender(s) as an individual Tender, and we are not participating in any other Tender(s) as a Joint Venture member or as a subcontractor, and meet the requirements of ITT 3.4, other than alternative Tenders submitted in accordance with ITT 13.3;

- Suspension and Debarment: We, along with any of our subcontractors, suppliers, Project Manager, manufacturers, or service providers for any part of the contract, are not subject to, and not controlled by any entity or individual that is subject to, a temporary suspension or a debarment imposed by the Public Procurement Regulatory Authority or any other entity of the Government of Kenya, or any international organization.

- State-owned enterprise or institution: [select the appropriate option and delete the other] [We are not a state-owned enterprise or institution] / [We are a state-owned enterprise or institution but meet the requirements of ITT 3.8];

- Commissions, gratuities, fees: We have paid, or will pay the following commissions, gratuities, or fees with respect to the tender process or execution of the Contract: [insert complete name of each Recipient, its full address, the reason for which each commission or gratuity was paid and the amount and currency of eachsuchcommission or gratuity].

| Name | Address | Reason | Amount |
| --- | --- | --- | --- |

(If none has been paid or is to be paid, indicate “none.”)

- Binding Contract: We understand that this Tender, together with your written acceptance thereof included in your Letter of Acceptance, shall constitute a binding contract between us, until a formal contract is prepared and executed;

- Not Bound to Accept: We understand that you are not bound to accept the lowest evaluated cost Tender, the Most Advantageous Tender or any other Tender that you may receive;

- Fraud and Corruption: We hereby certify that we have taken steps to ensure that no person acting for us or on our behalf engages in any type of Fraud and Corruption;

- Collusive practices: We hereby certify and conﬁrm that the tender is genuine, non-collusive and made with the intention of accepting the contract if awarded. To this effect we have signed the “Certiﬁcate of Independent Tender Determination” attached below.

- We undertake to adhere by the Code of Ethics for Persons Participating in Public Procurement and Asset Disposal, copy available from	(specify website) during the procurement process and the execution of any resulting contract.

- We, the Tenderer, have completed fully and signed the following Forms as part of our Tender:

  - Tenderer's Eligibility; Conﬁdential Business Questionnaire – to establish we are not in any conﬂict to interest.

  - Certiﬁcate of Independent Tender Determination – to declare that we completed the tender without colluding with other tenderers.

  - Self-Declaration of the Tenderer – to declare that we will, if awarded a contract, not engage in any form of fraud and corruption.

  - Declaration and commitment to the Code of Ethics for Persons Participating in Public Procurement and Asset Disposal

Further, we conﬁrm that we have read and understood the full content and scope of fraud and corruption as informed in “Appendix 1- Fraud and Corruption” attached to the Form of Tender.

Name of the Tenderer:

Name of the person duly authorized to sign the Tender on behalf of the Tenderer

Title of the person signing the Tender:

Signature of the person named above:

Notes

Date signed	day of	,

* In the case of the Tender submitted by joint venture specify the name of the Joint Venture as Tenderer

** Person signing the Tender shall have the power of attorney given by the Tenderer to be attached with the Tender.

- TENDERER’S ELIGIBILITY- CONFIDENTIALBUSINESS QUESTIONNAIRE

Instruction to Tenderer

Tender is instructed to complete the particulars required in this Form, one form for each entity if Tender is a JV. Tenderer is further reminded that it is an offence to give false information on this Form.

(a)	Tenderer's details

| NO | ITEM | DESCRIPTION |
| --- | --- | --- |
| 1 | Name of the procuring entity | Kenya Rural Road Authority |
| 2 | Reference Number of the Tender |  |
| 3 | Date and Time of Tender Opening |  |
| 4 | Name of the tenderer |  |
| 5 | Full Address and Contract Detail of the Tenderer | Country City Location Building Floor Postal Address Name and email of contact person |
| 6 | Current Trade License Registration Number and Expiring date |  |
| 7 | Name, country and full address( postal and physical addresses, email and telephone number) of Registration Body Agency |  |
| 8 | Description of Nature of Business |  |
| 9 | Maximum value of business which the Tenderer handles |  |
| 10 | State if Tenders Company is listed in stock exchange, give name and full address (postal and physical addresses, email, and telephone number) of state which stock exchange |  |

General and Speciﬁc Details

- Sole Proprietor, provide the following details.

Name in full	Age	Nationality		 Country of Origin		Citizenship

- Partnership, provide the following details.

| No. | Names of Partners | Nationality | Citizenship | % shares owned |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |

- Registered Company, provide the following details.

  - Private or public Company

  - State         the         nominal        and        issued         capital         of         the         Company Nominal Kenya Shillings (Equivalent)	Issued

Kenya Shillings (Equivalent).....................................................................................

Give details of Directors as follows.

| No. | Names of Directors | Nationality | Citizenship | % shares owned |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |

- DISCLOSURE OF INTEREST- Interest of the Firm in the Procuring Entity.

  - Are there any person/persons in Kenya Rural Roads Authority who has/have an interest or relationship in this ﬁrm? Yes/No………………………

If yes, provide details as follows.

| No. | Names of person | Designation in the Procuring Entity | Interest or Relationship with Tenderer |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |

  - Conﬂict of interest disclosure

| No. | Type of Conflict | Disclosure YES OR NO | If YES provide details of the relationship with Tenderer |
| --- | --- | --- | --- |
| 1 | Tenderer is directly or indirectly controls, is controlled by or is under common control with another tenderer. |  |  |
| 2 | Tenderer receives or has received any direct or indirect subsidy from another tenderer. |  |  |
| 3 | Tenderer has the same legal representative as another tenderer |  |  |
| 4 | Tender has a relationship with another tenderer, directly or through common third parties that puts it in a position to influence the tender of another tenderer, or influence the decisions of the Procuring Entity regarding this tendering process. |  |  |
| 5 | Any of the Tenderer's affiliates participated as a consultant in the preparation of the design or technical specifications of the works that are the subject of the tender. |  |  |
| 6 | Tenderer would be providing goods, works, non- consulting services or consulting services during implementation of the contract Specified in this Tender Document. |  |  |
| 7 | Tenderer has a close business or family relationship with a professional staff of the Procuring Entity who are directly or indirectly involved in the preparation of the Tender document or specifications of the Contract, and/or the Tenderer Evaluation process of such contract. |  |  |
| 8 | Tenderer has a close business or family relationship with a professional staff of the Procuring Entity who would be involved in the implementation or supervision of the Contract. |  |  |
| 9 | Has the conflict stemming from such relationship stated in item 7 and 8 above been resolved in a manner acceptable to the Procuring Entity throughout the tendering process and execution of the Contract. |  |  |

f)	Certiﬁcation

On behalf of the Tenderer, I certify that the information given above is complete, current and accurate as at the date of submission.

Full Name

Title or Designation

(Signature)	(Date)

- CERTIFICATE OF INDEPENDENT TENDER DETERMINATION

I, the undersigned, in submitting the accompanying Letter of Tender to the Kenya Rural Roads Authority for:		 [Name and number of tender] in response to the request for tenders made by: 		                                                   [Name of Tenderer] do hereby make the following statements that I certify to be true and complete in every respect: I certify, on behalf of	[Name of Tenderer] that:

- I have read and I understand the contents of this Certiﬁcate;

- I understand that the Tender will be disqualiﬁed if this Certiﬁcate is found not to be true and complete in every respect;

- I am the authorized representative of the Tenderer with authority to sign this Certiﬁcate, and to submit the Tender on behalf of the Tenderer;

- For the purposes of this Certiﬁcate and the Tender, I understand that the word “competitor” shall include any individual or organization, other than the Tenderer, whether or not afﬁliated with the Tenderer, who:

  - has been requested to submit a Tender in response to this request for tenders;

  - could potentially submit a tender in response to this request for tenders, based on their qualiﬁcations, abilities or experience;

- The Tenderer discloses that [check one of the following, as applicable:

  - The Tenderer has arrived at the Tender independently from, and without consultation, communication, agreement or arrangement with, any competitor;

  - the Tenderer has entered into consultations, communications, agreements or arrangements with one or more competitors regarding this request for tenders, and the Tenderer discloses, in the attached document(s), complete details thereof, including the names of the competitors and the nature of, and reasons for, such consultations, communications, agreements or arrangements;

- In particular, without limiting the generality of paragraphs (5)(a) or (5)(b) above, there has been no consultation, communication, agreement or arrangement with any competitor regarding:

  - prices;

  - methods, factors or formulas used to calculate prices;

  - the intention or decision to submit, or not to submit, a tender; or

  - the submission of a tender which does not meet the speciﬁcations of the request for Tenders; except as speciﬁcally disclosed pursuant to paragraph (5)(b) above;

- In addition, there has been no consultation, communication, agreement or arrangement with any competitor regarding the quality, quantity, speciﬁcations or delivery particulars of the works or services to which this request for tenders relates, except as speciﬁcally authorized by the procuring authority or as speciﬁcally disclosed pursuant to paragraph (5)(b) above;

- the terms of the Tender have not been, and will not be, knowingly disclosed by the Tenderer, directly or indirectly, to any competitor, prior to the date and time of the ofﬁcial tender opening, or of the awarding of the Contract, whichever comes ﬁrst, unless otherwise required by law or as speciﬁcally disclosed pursuant to paragraph (5)(b) above.

Name _	Title	Date

[Name, title and signature of authorized agent of Tenderer and Date].

- SELF - DECLARATION FORMS FORM SD1

SELF DECLARATION THAT THE PERSON/TENDERER IS NOT DEBARRED IN THE MATTER OF THE PUBLIC PROCUREMENTAND ASSET DISPOSALACT 2015.

I ……………………………………., of Post Ofﬁce Box	being a resident of

…………………..…………….. in the Republic of	do hereby make a statement

as follows: -

- THAT I am the Company Secretary/ Chief Executive/Managing Director/Principal Ofﬁcer/Director of

………....………………………………………………………........................ (insert name of the Company)

who is a Bidder in respect of Tender No.	for

…………………………………………………………………………...…. (insert tender name/description)

for Kenya Rural Roads Authority and duly authorized and competent to make this statement.

- THAT the aforesaid Bidder, its Directors and subcontractors have not been debarred from participating in procurement proceeding under Part IV of the Act.

- THATwhat is deponed to herein above is true to the best of my knowledge, information and belief.

| …………………………. | ………………………………. | ……………………… |
| --- | --- | --- |
| (Signature) | (Date) | (Title) |

Bidder Ofﬁcial Stamp

SELF DECLARATION THAT THE PERSON/TENDERER WILL NOT ENGAGE IN ANY CORRUPT OR FRAUDULENT PRACTICE

I, ………………………………………. of P. O. Box	being a resident

of ……………………… in the Republic of	do hereby make a statement as follows: -

- THAT I am the Chief Executive/Managing Director/Principal Ofﬁcer/Director of

………………………………………………...................... (insert name of the Company) who is a Bidder in respect of Tender No.	for

……………………………………………………….…..…………….. (insert tender name/description) for

Kenya Rural Roads Authority and duly authorized and competent to make this statement.

- THAT the aforesaid Bidder, its servants and/or agents /subcontractors will not engage in any corrupt or fraudulent practice and has not been requested to pay any inducement to any member of the Board, Management, Staff and/or employees and/or agents of Kenya Rural Roads Authority which is the procuring entity.

- THAT the aforesaid Bidder, its servants and/or agents /subcontractors have not offered any inducement to any member of the Board, Management, Staff and/or employees and/or agents of Kenya Rural Roads Authority THAT the aforesaid Bidder will not engage /has not engaged in any corrosive practice with other bidders participating in the subject tender

- THATwhat is deponed to herein above is true to the best of my knowledge information and belief.

………………………………… …………………….....	................……………………… (Title)	(Signature)		(Date)

Bidder's Ofﬁcial Stamp

I	(person) on behalf of (Name of the Business/

Company/Firm)	declare that I have read and fully

understood the contents of the Public Procurement & Asset Disposal Act, 2015, Regulations and the Code

of Ethics for persons participating in Public Procurement and Asset Disposal and my responsibilities under the Code.

I do hereby commit to abide by the provisions of the Code of Ethics for persons participating in Public Procurement and Asset Disposal.

Name of Authorized signatory…………………............................ Sign…………….....................

Position………………………………………….............................................................................

Ofﬁce address ……………………………… Telephone……………......................……………..

E-mail: ………………............................................................….........................................………

Name of the Firm/Company: ……………......................................................................................

Date………………………………………………………..............................................…………

(Company Seal/ Rubber Stamp where applicable)

Witness

Name	……………………………………………………….......	Sign...................................................

Date……………………………………………………

- APPENDIX 1- FRAUD AND CORRUPTION

(Appendix 1 shall not be modiﬁed)

  - Purpose

The Government of Kenya's Anti-Corruption and Economic Crime laws and their sanction's policies and procedures, Public Procurement and Asset Disposal Act (no. 33 of 2015) and its Regulation, and any other Kenya's Acts or Regulations related to Fraud and Corruption, and similar offences, shall apply with respect to Public Procurement Processes and Contracts that are governed by the laws of Kenya.

  - Requirements

The Government of Kenya requires that all parties including Procuring Entities, Tenderers, (applicants/proposers), Consultants, Contractors and Suppliers; any Sub-contractors, Sub-consultants, Service providers or Suppliers; any Agents (whether declared or not); and any of their Personnel, involved and engaged in procurement under Kenya's Laws and Regulation, observe the highest standard of ethics during the procurement process, selection and contract execution of all contracts, and refrain from Fraud and Corruption and fully comply with Kenya's laws and Regulations as per paragraphs 1.1 above.

Kenya's public procurement and asset disposal act (no. 33 of 2015) under Section 66 describes rules to be followed and actions to be taken in dealing with Corrupt, Coercive, Obstructive, Collusive or Fraudulent practices, and Conﬂicts of Interest in procurement including consequences for offences committed. A few of the provisions noted below highlight Kenya's policy of no tolerance for such practices  and behavior: -

    - a person to whom this Act applies shall not be involved in any corrupt, coercive, obstructive, collusive or fraudulent practice; or conﬂicts of interest in any procurement or asset disposal proceeding;

    - A person referred to under subsection (1) who contravenes the provisions of that sub-section commits an offence;

    - Without limiting the generality of the subsection (1) and (2), the person shall be: -

      - disqualiﬁed from entering into a contract for a procurement or asset disposal proceeding; or

      - if a contract has already been entered into with the person, the contract shall be voidable;

    - The voiding of a contract by the procuring entity under subsection (7) does not limit any legal remedy the procuring entity may have;

    - An employee or agent of the procuring entity or a member of the Board or committee of the procuring entity who has a conﬂict of interest with respect to a procurement: -

      - shall not take part in the procurement proceedings;

      - shall not, after a procurement contract has been entered into, take part in any decision relating to the procurement or contract; and

      - shall not be a subcontractor for the bidder to whom was awarded contract, or a member of the group of bidders to whom the contract was awarded, but the subcontractor appointed shall meet all the requirements of this Act.

    - An employee, agent or member described in subsection (1) who refrains from doing anything prohibited under that subsection, but for that subsection, would have been within his or her duties shall disclose the conﬂict of interest to the procuring entity;

    - If a person contravenes subsection (1) with respect to a conﬂict of interest described in subsection (5)(a) and the contract is awarded to the person or his relative or to another person in whom one of them had a direct or indirect pecuniary interest, the contract shall be terminated and all costs incurred by the public entity shall be made good by the awarding ofﬁcer. Etc.

In compliance with Kenya's laws, regulations and policies mentioned above, the Procuring Entity:

      - Deﬁnes broadly, for the purposes of the above provisions, the terms set forth below as follows:

        - “corrupt practice” is the offering, giving, receiving, or soliciting, directly or indirectly, of anything of value to inﬂuence improperly the actions of another party;

        - “fraudulent practice” is any act or omission, including misrepresentation, that knowingly or recklessly misleads, or attempts to mislead, a party to obtain ﬁnancial or other beneﬁt or to avoid an obligation;

        - “collusive practice” is an arrangement between two or more parties designed to achieve an improper purpose, including to inﬂuence improperly the actions of another party;

        - “coercive practice” is impairing or harming, or threatening to impair or harm, directly or indirectly, any party or the property of the party to inﬂuence improperly the actions of a party;

        - “obstructive practice” is:

          - deliberately destroying, falsifying, altering, or concealing of evidence material to the investigation or making false statements to investigators in order to materially impede investigation by Public Procurement Regulatory Authority (PPRA) or any other appropriate authority appointed by Government of Kenya into allegations of a corrupt, fraudulent, coercive, or collusive practice; and/or threatening, harassing, or intimidating any party to prevent it from disclosing its knowledge of matters relevant to the investigation or from pursuing the investigation; or

          - acts intended to materially impede the exercise of the PPRA's or the appointed authority's inspection and audit rights provided for under paragraph 2.3 e. below.

      - Deﬁnes more speciﬁcally, in accordance with the above procurement Act provisions set forth for fraudulent and collusive practices as follows:

"fraudulent practice" includes a misrepresentation of fact in order to inﬂuence a procurement or disposal process or the exercise of a contract to the detriment of the procuring entity or the tenderer or the contractor, and includes collusive practices amongst tenderers prior to or after tender submission designed to establish tender prices at artiﬁcial non-competitive levels and to deprive the procuring entity of the beneﬁts of free and open competition.

      - Rejects a proposal for award1 of a contract if PPRA determines that the ﬁrm or individual recommended for award, any of its personnel, or its agents, or its sub-consultants, sub-contractors, service providers, suppliers and/ or their employees, has, directly or indirectly, engaged in corrupt, fraudulent, collusive, coercive, or obstructive practices in competing for the contract in question;

      - Pursuant to the Kenya's above stated Acts and Regulations, may sanction or recommend to appropriate authority (ies) for sanctioning and debarment of a ﬁrm or individual, as applicable under the Acts and Regulations;

      - Requires that a clause be included in Tender documents and Request for Proposal documents requiring (i) Tenderers (applicants/proposers), Consultants, Contractors, and Suppliers, and their Sub-contractors, Sub-consultants, Service providers, Suppliers, Agents personnel, permit the PPRA or any other appropriate authority appointed by Government of Kenya to inspect2 all accounts, records and other documents relating to the procurement process, selection and/or contract execution, and to have them audited by auditors appointed by the PPRA or any other appropriate authority appointed by Government of Kenya; and

      - Pursuant to Section 62 of the above Act, requires Applicants/Tenderers to submit along with their Applications/Tenders/Proposals a “Self-Declaration Form” as included in the procurement document declaring that they and all parties involved in the procurement process and contract execution have not engaged/will not engage in any corrupt or fraudulent practices.

1For the avoidance of doubt, a party's ineligibility to be awarded a contract shall include, without limitation, (i) applying for pre-qualiﬁcation, expressing interest in a consultancy, and tendering, either directly or as a nominated sub-contractor, nominated consultant, nominated manufacturer or supplier, or nominated service provider, in respect of such contract, and (ii) entering into an addendum or amendment introducing a material modiﬁcation to any existing contract.

- Inspections in this context usually are investigative (i.e., forensic) in nature. They involve fact-ﬁnding activities undertaken by the Investigating Authority or persons appointed by the Procuring Entity to address speciﬁc matters related to investigations/audits, such as evaluating the veracity of an allegation of possible Fraud and Corruption, through the appropriate mechanisms. Such activity includes but is not limited to: accessing and examining a ﬁrm's or individual's ﬁnancial records and information, and making copies thereof as relevant; accessing and examining any other documents, data and information (whether in hard copy or electronic format) deemed relevant for the investigation/audit, and making copies thereof as relevant; interviewing staff and other relevant individuals; performing physical inspections and site visits; and obtaining third party veriﬁcation of information.

[The Surety shall ﬁll in this Tender Bond Form in accordance with the instructions indicated.]

BOND NO.

- BY THIS BOND [name of tenderer] as Principal (hereinafter called “the Principal”), and [name, legal title, and address of surety],authorized to transact business in [name of country of Procuring Entity], as Surety (hereinafter called “the Surety”), are held and ﬁrmly bound unto [name of Procuring Entity] as Obligee (hereinafter called “the Procuring Entity”) in the sum of [amount of Bond][amount in words], for the payment of which sum, well and truly to be made, we, the said Principal and Surety, bind ourselves, our successors and assigns, jointly and severally, ﬁrmly by these presents.

- WHEREAS the Principal has submitted or will submit a written Tender to the Procuring Entity dated the	day of	, 20 , for the supply of [name of Contract] (hereinafter called the “Tender”).

- NOW, THEREFORE, THE CONDITION OF THIS OBLIGATION is such that if the Principal:

  - has withdrawn its Tender during the period of Tender validity set forth in the Principal's Letter of Tender (“the Tender Validity Period”), or any extension thereto provided by the Principal; or

  - having been notiﬁed of the acceptance of its Tender by the Procuring Entity during the Tender Validity

Period or any extension thereto provided by the Principal; (i) failed to execute the Contract agreement; or

(ii) has failed to furnish the Performance Security, in accordance with the Instructions to tenderers (“ITT”) of the Procuring Entity's Tendering document.

Then the Surety undertakes to immediately pay to the Procuring Entity up to the above amount upon receipt of the Procuring Entity's ﬁrst written demand, without the Procuring Entity having to substantiate its demand, provided that in its demand the Procuring Entity shall state that the demand arises from the occurrence of any of the above events, specifying which event(s) has occurred.

- The Surety hereby agrees that its obligation will remain in full force and effect up to and including the date 30 days after the date of expiration of the Tender Validity Period set forth in the Principal's Letter of Tender or any extension thereto provided by the Principal.

- IN TESTIMONY WHEREOF, the Principal and the Surety have caused these presents to be executed in their respective names this	day of	20	.

Principal:	 Corporate Seal

Surety

(Signature)	(Signature)

(Printed nameandtitle)	(Printed nameand title)

[The Bidder shall complete this Form in accordance with the instructions indicated]

Date:	[insert date (as day, month and year) of Tender Submission]

Tender No.	[insert number of tendering process]

To:	[insert complete name of Purchaser] I/We, the undersigned, declare that:

- I/We understand that, according to your conditions, bids must be supported by a Tender-Securing Declaration.

- I/We accept that I/we will automatically be suspended from being eligible for tendering in any contract with the Purchaser for the period of time of twenty-four (24) months starting on the date of tender award or date of termination, if we are in breach of our obligation(s) under the bid conditions, because we –

  - have withdrawn our tender during the period of tender validity speciﬁed by us in the Tendering Data Sheet; or

  - having been notiﬁed of the acceptance of our Bid by the Purchaser during the period of bid validity,

    - fail or refuse to execute the Contract, if required, or

    - fail or refuse to furnish the Performance Security, in accordance with the instructions to tenders.

- I/We understand that this Tender Securing Declaration shall expire if we are not the successful Tenderer(s), upon the earlier of:

- our receipt of a copy of your notiﬁcation of the name of the successful Tenderer; or

- Thirty (30)days after the expiration of our Tender.

- I/We understand that if I am/we are/in a Joint Venture, the Tender Securing Declaration must be in the name of the Joint Venture that submits the bid, and the Joint Venture has not been legally constituted at the time of bidding, the Tender Securing Declaration shall be in the names of all future partners as named in the letter of intent.

Signed:……………………………………………………………………..…….....…..

Capacity / title (director or partner or sole proprietor, etc.) ……….………….........…….

Name: …………………………………………………………………………………..

Duly authorized to sign the bid for and on behalf of: [insert complete name of Tenderer]

Dated on …………………. day of	[Insert date of signing] Seal or stamp

Schedule of Currency requirements

Summary of currencies of the Tender for

[insert name of Section of the Works]

| Name of Currency | Amounts payable |
| --- | --- |
| Local Currency (KShs) |  |
| Provisional sums expressed in local currency | [to be entered by the Procuring Entity] |

PART II - WORK REQUIREMENTS

## SECTION V – DRAWINGS

## List of Drawings

- Figure C.1-Cross Section A (Minor Standard Cross-Section)

- Figure C2- Cross Section B (Reduced Cross-Section)

- Figure C3-Cross- Section C (Black Cotton Soil)

- Figure C4-Cross- Section D (Sidelong Ground)

- Figure C5- Standard Cross- Section (Super-elevation)

- Figure C6 -Mitre Drains

- Figure C7 -Scour Checks

- Figure C8-Dimensions Of Scour Checks For Standard Drain

- Figure C9a-Masonry Scour Checks

- Figure C9b- Concrete Scour Checks

- Figure C10- Culvert Entry/Exit Structure Types

- Figure C11 Headwall Type 1 (Head And Wing walls)

- Figure C12. Headwall Type 2 (Drop Inlet)

- Figure C.13-Headwall Type 3a (Concrete/ Block Headwalls)

- Figure C14-Headwall Type 3b (Stone Masonry Headwalls)

- Figure C15-Headwall Type For Access Culverts)

- Figure C16-Bedding And Haunch Profile Types I & II

- Figure C17-Bedding And Surround Profile Types III & IV

- Figure C18-Access Drift

- Figure C19-Level Drift

- Figure C20-Vented Drift

- Figure C21-Hand Packed Stone Pavement

- Figure C22-Trafflc Signs

- Figure C23-Publicity Signboard

FIGURE C.1-CROSS SECTION A (MINOR STANDARD CROSS-SECTION)

# FIGURE C2- CROSS SECTION B (REDUCED CROSS-SECTION)

FIGURE C3-CROSS- SECTION C (BLACK COTTON SOIL)

FIGURE C4-CROSS- SECTION D (SIDELONG GROUND)

FIGURE C5- STANDARD CROSS- SECTION (SUPERELEVATION)

# FIGURE C6 -MITRE DRAINS

# FIGURE C7 -SCOUR CHECKS

## FIGURE C8-DIMENSIONS OF SCOUR CHECKS FOR STANDARD DRAIN

### FIGURE C9a-MASONRY SCOUR CHECKS

2400

Note

# I.Dimensio»s in mm

| Cross- Section | Sizes In (mm) |  |  | Exc. (m3) | Concrete (m3) | Apron stone pitching (m3) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Width | Depth |  |  |  |

SECTION OF MASONRY SCOUR CHECK

# PLAN OF DRAIN WITH SCOUR CHECK

### FIGURE C9b- CONCRETE SCOUR CHECKS

2400

SECTION OF CONCRETE SCOUR CHECK

NOT

I. Dimensions in mm

PLAN OF DRAIN WITH SCOUR CHECK

| Cross Section | Sizes in mm |  |  | Excavation(m3) | Concrete m3) | Apron stone pitching |
| --- | --- | --- | --- | --- | --- | --- |
|  | Length | Width | Depth |  |  |  |
|  | 2400 | 100 | 550 |  |  | 0.18 |
|  | 2000 | 100 | 500 | 0.10 | 0.09 | 0.14 |

## FIGURE C10- CULVERT ENTRY/EXIT STRUCTURE TYPES

| TYPEI(ENTRY AND EXIT) | TYPE3(ENTRY AND EXIT) |
| --- | --- |

| TYPE 2(ENTRY ONLY) | TYPE 4(ENTRY AND EXIT ON ACCESS) |
| --- | --- |

NOTE

I. The code numbers specify the shape and function and the code letter denotes the materiai;

- =Concrete block

- =Stone masonry

- —Concrete

C-10

FIGURE C11 HEADWALL TYPE 1 (HEAD AND WINGWALLS)

FIGURE C12. HEADWALL TYPE 2 (DROP INLET)

FIGURE C.13-HEADWALL TYPE 3A (CONCRETE/ BLOCK HEADWALLS

FIGURE C14-HEADWALL TYPE 3B (STONE MASONRY HEADWALLS)

FIGURE C15-HEADWALL TYPE FOR ACCESS CULVERTS)

FIGURE C16-BEDDING AND HAUNCH PROFILE TYPES I & II

# PROFILE I                                                      PROFILE II

### (CONCRETE)	(CONCRETE)

| a; a. |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  |  | c. |
| 450 (mm); 600 (mm); 900 (mm). Concrete Class 15/20; Volume in (m3/m). 0.24; 0.24; 0.24. Application; -Fair subgrade condition -Overfill > 75% of the pipe diameter Seasonal water now only. Remarks; Material for back/overfill shall be approved by the Engineer. | 450 (mm); 600 (mm); 900 (mm). Volume in (m3/m). 0.20; 0.37; 0.56. -Fair to poor subgrade condition -Overfill > 75% of the pipe diameter -Seasonal water flow only. Material for back/over/filI shall be approved by the Engineer. |

FIGURE C17-BEDDING AND SORROND PROFILE TYPES III & IV

# PROFILE III	PROFILE IV

(CONCRETE)	(CONCRETE)

|  | 450 (mm) | 600 (mm) | 900 (mm) |  | 450 (mm) | 600 (mm) | 900 (mm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class 15/20 |  |  |  |  |  |  |  |
| Application | -Fair subgrade condition -Overfill > 75% of the pipe diameter -Seasonal water now only |  |  |  | -Fair to poor subgrade condition -Overfill > 75% of the pipe diameter -Seasonal water flow only |  |  |
| Remarks | Material for back/overfill shall approved by the Engineer |  |  |  | Material for back/over/filI shall approved by the Engineer |  |  |

FIGURE C18-ACCESS DRIFT

pattern

150mm Grouted Stone Pitching

(Cement mortar 1

Stone masonry toes

on four sides of structure

SECTION X-X	(200*300mm)

| QUANTITIES TABLE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cross section | DIMENSONS |  |  |  |  | Excavation(m3) | Stone masonry (m3) | 150mm Grouted stone pitching(m3) |
|  | 4000 | 1800 | 600 | 1800 | 4200 | 7.5 | 1.30 |  |
|  | 6000 | 1800 |  | 1800 | 4200 | 10.00 |  |  |
|  | 4000 | 1400 | 400 | 1800 | 3600 | 7.00 |  |  |
|  | 6000 | 1400 | 400 | 180 | 3600 | 9.00 | t .50 | 25.50 |

C-18

FIGURE C19-LEVEL DRIFT

FIGURE C20-VENTED DRIFT

FIGURE C21-HAND PACKED STONE PAVEMENT

FIGURE C22-TRAFFlC SIGNS

600

nd

Road

sign

KEY

1. T lie type of sign required and their location shall be as shown on the Road Plan or as directed by 1112 Engineer.

2.Sign plate to be 2mm thick mild steel plate

3.Sign post to be 50mm fixing clamps/brackets.

4.Sign plate to be fixed to steel tube by $ Nos MIO bolts and 50mm fixing clamps/brackets.

5.sign paints shall be reflective.

6.The sign plate and post shall be treated by applying two coats of lead red oxide paint before applying n priming and two finish coats of approved paints. Paints used Shull have a hard, durable and glossy finish.

FIGURE C23-PUBLlCITY SIGNBOARD

—25mm

WHITE

BLACK

NOTES

- The wording of the project sign board and the location to be as directed by the Engineer.

- Materials to be used for fabrication or signboard shall be pressure impregnated treated softwood timber

3.Wording board posts to be attached to the posts with galvanised nails

4.Project board posts and struts to be embedded in concrete class 20/20(1:2:4)

SECTION VI – SPECIFICATIONS FOR ROUTINE MAINTENANCE /SPOT IMPROVEMENT WORKS

Standard Specification

Whenever reference is made to “The Engineer” in the specification, it shall be construed to be synonymous with “Employer’s duly authorized Representative” as referred to in the Conditions of Contract.

SECTION 01	:	PRELIMINARY AND GENERAL ITEMS

Scope:

This section refers to those items that are needed at the start and end of the Works or are provisional items applicable for the duration of the Works.

01-40- 001: Mobilization and Establishment of Site

The Contractor shall provide all equipment, tools, material and temporary stores required to carry out the required Works.

The Contractor shall ensure that all possible means of protection are given to the staff at all times. Such protection shall include provision of high visibility clothing or vests for the workforce, in potentially dangerous locations. The Contractor shall also maintain first aid kits with items included on the advice of the local Medical Officer, or as directed by the Engineer.

Measurement and Payment

No separate payment shall be made for this item. The Contractor shall include the costs in the other rates for other measured items.

01-40-002: Clearance on Completion

On Completion of the Works, all temporary stores, equipment, signs and tools shall be removed from the site, and the Site left in good order to the satisfaction of the Engineer.

Measurement and Payment

The Lump Sum payment shall be made upon the approval of the Engineer that the Clearance has been satisfactorily carried out.

01-40-006: Traffic Control

The Contractor shall provide warning signs, fences, barriers, detours, which shall be properly positioned well in advance so that all traffic is well and safely accommodated for the duration of the Works.

Traffic signs and other traffic control facilities shall be kept in good condition and located in positions where they are visible to road users.

Work Method

The Contractor shall use Labour to carry out this item of workQuality Control The Engineer shall check regularly that traffic control measures are satisfactory.

Measurement and Payment

A Lump Sum shall be paid on a Monthly basis upon the approval of the Engineer that adequate Traffic Control is in place

01–40–007: Drinking Water

The Contractor shall provide safe drinking water on site for workers at a reasonable distance from all worklocations, for the duration of the Works.

Quality Control

The Engineer shall check regularly that adequate supplies of water are available throughout the Site. Measurement and Payment

A Lump Sum shall be paid on a Monthly basis upon the approval of the Engineer that adequate supplies have been provided

SECTION 04: ROADSIDE CLEARANCE

Scope

This section covers all routine maintenance works within the road reserve and includes items such as bush clearing, pruning of tree branches, grass cutting, and removal of litter and any other debris.

04–50–002: Grass Cutting (Manual)

Grass shall be defined as any form of plant growth including small shrubs having a girth of not more than 100mm measured at height of 200mm above ground level.

The grass shall be cut to height of not more than 50 mm above the ground. The width limits shall be as instructed by the Engineer. All cut grass shall be removed from the carriageway, side drains, mitre drains and inlets and outlet drains of structures/culverts and deposited in approved spoil dumps

EA Burning of the grass shall not be allowed and care shall be taken not to damage roadsidefixtures such as signs and marker posts.

This activity shall be carried out twice, each time before the rainy season or as shall be instructed by the Engineer. Work Method

The Contractor shall use Labour to carry out this item of work. Quality Control

The road width for grass cutting shall be measured at 50-m intervals and shall be free of grass after the operation. Measurement:	m2

The measurement shall be area of grass cut, based on the standard width and measured length of clearing.

Payment

The unit rate shall be full compensation, for labour, materials, tools, and incidental costs required to carry out the work.

04-50-003	Bush Clearing - heavy04-50-

004	Bush Clearing - light

04-50-005: Pruning Tree Branches

This activity involves the removal of small trees, shrubs and bushes all including their root systems, In addition, the Engineer may order the trimming of branches of large trees to improve visibility. The width limits shall be as instructed by the Engineer.

The cut material shall be collected into heaps away from the side drains and where it shall not block or interfere with visibility. The burning of cut bush and removed trees or branches shall not be allowed.

Work Method

The Contractor shall use Labour to carry out this item of work Quality Control

The width for bush clearing shall be measured at 50-m intervals and shall be free of trees or bushes. Measurement Unit: m2The measurement shall be the area cleared according to the specified widths and measured length of clearance.

Payment

The unit rate shall be the full compensation, for labour, materials, tools, and incidental costs required to carry out the work.

04–50–008: Clearing of Obstructions

This activity shall involve the following tasks:-

- Inspection of the road section(s) regularly

- Removal of all obstructions such as fallen trees/ branches, rock fall, landslides and broken signs away from the road, side drains, mitre drains and other drains, inlets and outlets of drifts, culverts and other structures and the safe disposal thereof outside the road formation width.

- Removal of dead animals’ carcasses away for the carriageway and disposing of them as directed by the Engineer. Liaison with the Police may be necessary

Quality Control

The road section shall be free of any obstruction.

Measurement Unit: Provisional Sum

The measurement for this item shall be a Provisional Sum paid as necessary on a Dayworks basis.

Payment

The unit rate shall be the full compensation for the provision of labour, tools and incidental costs necessary to carry out the tasks.

SECTION 05:	EARTHWORKS

05-70- 001: Grassing

This activity involves the protection / repair of erosion on embankment slopes, cut faces, shoulders, and side slopes by filling with suitable soils and compacting using appropriate tamping tools as instructed by the Engineer.

The Contractor shall plant sprigs of approved indigenous ‘runner’ type grass. The Contractor shallcare for and water the grass until it is firmly established.

Work Method

The Contractor shall use Labour to carry out this item of work.Quality Control

- The width of the slope shall be measured at 50m intervals and shall have maximum tolerances of + / - 100mm.

- The quality of grass and spacing of the sprigs shall be as directed by the Engineer Measurement Unit	m²

The unit of measurement shall be area calculated as the net area, measured on the slope

Payment

The unit rate shall be full compensation, for labour, materials, tools, and incidental costs required to carry out the work.

SECTION 08:	CULVERT AND DRAINAGE WORKS

Scope:

This section covers all routine maintenance of the drainage system including the cleaning or desilting of the side drains, mitre drains, catch water drains, culverts, inlets and outlets, and scour checks.

    - Ditch Cleaning (Manual) - Partially Silted08-50- 003		- Fully Silted

      - Partially silted

Partially silted drains are those that are less than half silted and require only cleaning.

All deposited material, debris, and vegetation shall be removed and the drain shaped to the original cross- section and left in a free- draining condition. Suitable material may be used to fill depressions and potholes on the carriageway. All debris and other unsuitable material removed from the side drains shall be disposed of well clear of the drainage system in approved spoil dumps where it will not cause any obstruction or be washed back.

The side drains, mitre drains and catchwater drains shall be cleaned before the onset of the rains or asdirected by the Engineer. Work Method

The Contractor shall use Labour to carry out this item of work Quality Control

- Appropriate drain templates shall be used to check and control the dimensions of the drains.

- The longitudinal profile of the drains shall be checked using boning rods, to ensure free flow. Measurement Unit:	m

The measurement shall be the length of drain desilted or cleaned to the specified cross-section. Payment

The unit rate shall be full compensation, for labour, tools, and incidental costs required to carry out the work.

i.	Fully silted

Fully silted drains shall be those that are greater than half-silted and require re-excavation or reshaping

All deposited material, debris, and vegetation shall be removed and the drain shaped to the original cross- section and left in a free- draining condition. Suitable material may be used to fill depressions and potholes on the carriageway. All debris and other unsuitable material removed from the side drains shall be disposed of well clear of the drainage system in approved spoil dumps where it will not cause any obstruction or be washed back

The side drains shall be desilted or re-excavated before the onset of the rains, or as directed by the Engineer.

Work Method

The Contractor shall use Labour to carry out this item of work

Quality Control

- Appropriate drain templates shall be used to check and control the dimensions of the drains.

- The longitudinal profile of the drains shall be checked using boning rods, to ensure free flow. Measurement Unit:	m3

The measurement shall be the volume of drain re-excavated or re-shaped to the specified cross-section. Payment The unit rate shall be full compensation for labour, tools, and incidental costs required to carry out the work.

08- 60 - 001/2/3/4/5: Culverts Cleaning (Partially blocked)

08 - 60 - 001	300mm dia;

08 - 60 - 002	450mm dia;

08 - 60 - 003	600mm dia;

08 - 60 - 004	900mm dia;

08 - 60 - 005	1200mm dia

This activity involves the cleaning of culverts of specified sizes including pipe barrels, the outlet/inlet structures, and the outlet drains, keeping them free of all debris, weed, silt and any obstruction to ensure free passage of water at all times. The debris shall be deposited in approved spoil dumps as directed by the Engineer

Partially blocked culverts shall be those with less than half of the barrel blocked.

Correct widths and slopes of the outlet drains shall be maintained. The gradient of the outlet drain shall be not less than 2 %. All broken culvert barrels discovered in the course carrying out this activity shall be reported to the Engineer.

This activity shall be carried out before the rains, or as directed by the Engineer.

Work Method

The Contractor shall use Labour to carry out this item of work

Quality Control

The culverts shall be checked as free of debris to the satisfaction of the Engineer.Measurement Unit: m

The measurement shall be the length of culvert, including the outlet drain, cleaned Payment

The unit rate shall be full compensation for labour, tools and incidental costs required to carry out the work.

08- 60 - 006/7/8/9/10 : Culvert Cleaning (Fully blocked):

08 - 60 - 006	300mm dia;

08 - 60 - 007	450mm dia;

08 - 60 - 008	600mm dia;

08 - 60 - 009	900mm dia;

08 - 60 - 010	1200mm dia

This activity involves the cleaning of culverts of specified sizes including pipe barrels, the outlet/inlet structures, and the outlet drains, keeping them free of all debris, weed, silt and any obstruction to ensure free passage of water at all times. The debris shall be deposited in approved spoil dumps as directed by the Engineer

Fully blocked culvert shall be those with greater than half of the barrel blocked.

Correct widths and slopes of the outlet drains shall be maintained. The gradient of the outlet drain shall be not less than 2 %. All broken culvert barrels discovered in the course of carrying out this activity shall be reported to the Engineer.

This activity should be carried out before the onset of the rains, or as directed by the Engineer. Work Method The Contractor shall use Labour to carry out this item of work

Quality Control

The culverts shall be checked as free from debris, to the satisfaction of the Engineer.

Measurement Unit:	m

The measurement shall be the length of culvert, including the outlet drain cleaned.Payment

The unit rate shall be full compensation for labour, tools and incidental costs required to carry out the work.

Supply and Installation of Concrete Pipe Culverts08-60-031

600	mm sorrounds

08 – 60-33 `	900 mm surrounds

The Contractor shall supply, lay and join concrete pipes to form culverts, including the concrete bedding;haunching or surrounds; and backfilling, in accordance with the Drawings for the Type and diameter specified in the Contract or directed by the Engineer.

The pipes shall be of Class 20/20 concrete, at least 28 days cured, and manufactured on site or procured from a supplier approved by the Engineer and preferably ogee jointed. The pipes shall be laid on a bedding of Class 15/20 concrete of dimensions as shown on the Drawings and jointed with cement mortar 1:4.

The culvert gradient including the outlet shall be a minimum 2%.

The pipes shall be surrounded with Class 15/20 concrete to the dimensions shown on the Drawings or as directed by the Engineer.

Backfilling shall be carried with approved material and compacted in layers not exceeding 150 mm loosedepth and placed evenly on each side of the pipe. Ramps shall be shaped to achieve a minimum overfill of 75% of the pipe diameter, and shall be tapered back on the carriageway to provide a gradual approach, as directed by the Engineer.

If the Contractor wishes to construct culverts on site, using inflatable or collapsible forms the Engineer’sapproval shall first be sought for the proposed working method.

On completion the inside of the culvert shall be smooth, without displaced joints or other obstructions and true to line and level. The Contractor shall use Labour and appropriate compaction Equipment to carry out this item work

Work Method:	LM-MB

Quality Control

- Concrete quality shall be checked for cracks, honey combing, and other defects.

- Before the pipes are laid, the gradient of the concrete bedding shall be checked and shall not be lessthan 2%

- The joints shall be checked to see that they have been properly made.

Measurement Unit:	m

The measurement shall be in linear metres of the installed Type and size of culvert specified, measured net according to the Drawings. Payment

The unit rate shall be the full compensation for labour, tools, materials, equipment and any other incidentals that may be required in carrying out the work.

08-60-030	Excavate in soft material for culverts

These activities should be done in accordance with Bill 8, sub clauses 8.01 to 8.20 in the StandardSpecifications for Roads and Bridges 1986.

The Contractor shall use both Labour and appropriate Equipment to carry out this item. Work Method:	LM-MB

Quality Control

- The workability and mix of concrete for the classes 15/20 and 20/25 shall be checked using the slumptest and shall have a slump limit as directed by the Engineer.

- The laying and joining of the culverts shall be subject to the approval of the engineer. Measurement Unit:	m

The measurement shall be the length of culvert laid.

Payment

The unit rate shall be the full compensation for labour, tools, materials, formwork, equipment and otherincidentals that may be required in carrying out the work.

- 70- 001: Head Wall Repair - Masonry

This activity involves the repairs to damaged head walls and wing walls built in masonry.

Where directed by the Engineer, the masonry walls shall be inspected and loose or missing stone re- secured or replaced. Damaged pointing shall be repaired with cement mortar 1:4 and finished flush with the stonework.

Work Method

The Contractor shall use Labour to carry out this item of work

Quality Control

  - The stability of the walls and the pointing shall be to the satisfaction of the Engineer.

Measurement Unit:	No

The measurement shall be the number of walls repaired as directed by the Engineer.Payment

The unit rate shall be full compensation for labour, materials, tools, and incidental costs required to carry out the work.

08-70-002: Headwall Repair - Concrete

The activity involves the repairs to damaged concrete headwalls and wingwalls, and to inlet/outlet concrete aprons. Concrete walls shall be inspected and repair works carried out as instructed by the Engineer to include breaking out and replacement of damaged concrete with similar material, and the rendering of open texture areas with cement mortar 1:4. Broken wall sections shall be re-built in 20/20 (1:2:4) concrete within formwork erected on the correct lines and levels in accordance with the Standard Drawings. Areas of new concrete and mortar shall be protected from direct sunlight and kept moist for 3 days.

Quality Control

The work shall be carried out to the satisfaction of the Engineer. Measurement Unit:	No

The measurement shall be the number of walls/aprons repaired. Payment

The unit shall be full compensation for labour, materials, tools, and incidental costs required to carry out the work.

08 - 70 - 008:	Scour Check Repair - masonry08 - 70 -

009:	Scour Check Repair - wooden 08 - 70 -

010:	Scour Check Repair - concrete

This activity involves the repair of Scour Checks using stones or wooden stakes or concrete. The construction details shall be shown in the Drawings or as instructed by the Engineer.

Scour checks shall be inspected and the repairs carried out as directed by the Engineer, which shall include replacement of missing or broken stonework and stakes; and the repair of damaged concrete, to the original lines, levels, and Specifications.

Work Method

The Contractor shall use Labour to carry out this item work. Quality Control

EB

The sizes of the stakes and stones used shall be as the original construction. The shape of the scour check shall be checked using the scour check template.

Measurement: No.

The unit rate of measurement shall be the number of scour checks repaired.Payment

The unit rate shall be full compensation, for labour, tools, materials, and incidental costs required for carrying out the work.

08- 70- 011: Stone Pitching Repair

This activity involves the repair of stone pitching on slopes, in inlet/outlet aprons and access drifts The stone pitching shall be inspected and repairs carried out as directed by the Engineer, including the replacement and re-bedding of missing or loose stones; the repair of mortar jointing: and the cleaning out of weep holes, as required. All work shall be to the lines and levels of the original construction with new stonework and mortar being flush with the adjacent materials.

Work Method

The Contractor shall use Labour to carry out this work

Quality Control

The work shall be carried out to the satisfaction of the Engineer. Measurement Unit:	m2

The measurement shall be the net surface area of the repairs.Payment Payment

The unit rate shall be full compensation, for labour, tools, materials, and incidental costs required to carry out the work.

08- 80- 004	Drift Maintenance – desilting

This activity involves the removal of debris, silt and any vegetation from drifts and causeways. The debris shall be deposited away from the drift in approved spoil dumps.

This activity shall be carried before the rains, or as directed by the Engineer.Work Method The Contractor shall use Labour to carry out this item of work

Quality Control

The work shall be carried out to the satisfaction of the Engineer. Measurement Unit:	m3

The measurement shall be the volume of debris or silt removed calculated as the product of length , width and measured depth of the affected section of drift.

Payment

The unit rate shall be full compensation for labour, tools, and incidental costs required to carry out the work.

08-80-005	Drift Repairs – Concrete

This activity involves the repair of concrete drifts, including the removal of loose or broken concrete, cutting back damaged areas to sound surfaces and repairing with concrete of similar Class to the original.

The drift shall be inspected and necessary repairs shall be instructed by the Engineer. Holes and voids shall be cleared of debris, loose material and dust, and shall be well watered before the new concrete is placed. The new concrete shall be firmly rammed against the existing surfaces and finished flush with the surrounding materials. The surface of the repair shall be protected from direct sunlight and kept moist for 3 days. Concrete shall be Class 20/20 unless otherwise directed by the Engineer.

The work shall be carried out to the satisfaction of the Engineer. Measurement Unit:	m3

The measurement shall be the volume of concrete used for the repair.Payment:

The unit rate shall be the full compensation for labour, tools, materials and incidental costs required to carry out the work

08-80-003: Gabions

The Contractor shall provide and install Gabions as retaining walls and anti-erosion structures at locations shown on the Drawings or as directed by the Engineer.

Gabions shall include mattresses and boxes and for purposes of construction, measurement and payment, no distinction shall be made between them.

Gabions shall be ‘Maccaferi” boxes or ‘Reno’ mattresses or equivalent approved by the Engineer.

The surfaces on which the Gabions are to be laid prior to being filled with rock shall be levelled to the depthsand dimensions shown on the Drawings or as directed by the Engineer.

Gabion boxes shall be tied together with 3 mm galvanized binding wire securing all edges at 150mm intervals.Work Method The Contractor shall use Labour to carry out this item.

Quality Control

The placing and tying of the Gabions shall be approved by the Engineer before filling commences. Measurement Unit:	No

The measurement shall be the number of Gabion boxes installed

Payment

The unit rate shall be the full compensation for labour, materials, and any incidental item costs necessary to carry out the work.

08-80-003 : Rockfill to Gabions

The Contractor shall provide selected rock, crushed if necessary, and carry out the packing and compacting of the rock inside the Gabion boxes.

The boxes shall be filled in layers from the sides towards the middle in an interlocking stone matrix to prevent deformation and bulging. The interior and top layers of the boxes shall be hand packed with smaller stone to form a tightly compact structure and rammed in place. Care shall be taken to ensure that each layer of boxes is filled evenly and to a level surface before the next course of boxes is placed.

Work method

The Contractor shall use Labour to carry out this activity.

| BEARING STRENGTH REQUIREMENTS |  |  |
| --- | --- | --- |
| Traffic Commercial | CBR | DCP |
| VPD |  | Equivalent |
|  |  | mm/Blow |
| >15 | 20 | 11 |
| <15 | 15 | 14 |

The filling and compaction of the stones in the Gabion boxes shall be approved by the Engineer. Measurement Unit	m³ Rockfill to Gabions shall be the volume of Gabions filled.

Payment

The unit rate shall be the full compensation for labour, tools, materials and incidental costs required for carrying out the work.

SECTION 10:

BILL 10: GRADING AND GRAVELLING

Scope:

Grading covers the work of reinstating the road carriageway to the correct camber by removing the highpoints and filling gullies, corrugations, and wheel ruts to restore a smooth running surface.

Grading can either be done by labour (Manual Reshaping) or by Machine (Motorized grading or towed grading).

Manual reshaping is preferable where there is sufficient labour. For existing roads with side drains light manual reshaping should be used as defined in 10-50-004. Heavy manual reshaping should be used forroads that have deteriorated to such an extent that the drains and carriageway need to be re-instated. Heavy Manual Reshaping is defined under Bill 5 – Earth Works.

Light grading is carried out on good and fair roads as a maintenance activity while heavy grading is for re- establishing a road in poor or very poor condition.

Gravelling consists of the excavation; loading, hauling, dumping, spreading and compacting using approved equipment of gravel wearing course material on the formation of the road carriageway. Gravel shall include lateritic gravel, quartzitic gravel, calcareous gravel, decomposed rock, soft stone coral rag, clayey sand and crushed rock.

The material may be obtained from quarries, borrow pits or excavation in cuttings as directed by the Engineer.Gravel material shall conform to the requirement given in Table 10.1

| GRADING REQUIREMENTS |  |
| --- | --- |
| Sieve (mm) | % by Weight Passing |
| 40 | 100 |
| 28 | 95 – 100 |
| 20 | 85 – 100 |
| 14 | 65 – 100 |
| 10 | 55 – 100 |
| 5 | 35 – 92 |
| 2 | 23 – 77 |
| 1 | 18 – 62 |
| 0.425 | 14 – 50 |
| 0.075 | 10 – 40 |

| PLASTICITY INDEX REQUIREMENTS PI |  |  |
| --- | --- | --- |
| Zone | Min | Max |
| WET: Mean annual rainfall | 5 | 20 |
| DRY: Mean annual rainfall < 500mm | 10 | 30 |

Table 10.1:   Requirement for Gravel Wearing Course

| For “Quarry Waste” gravel stones of maximum dimension 80mm may be permitted | CBR at 95 % MDD, Modified AASHTO and 4 days soaking |
| --- | --- |
|  | Lower quality material (CBR 15) may be accepted if no better material can be found |

The Engineer shall approve quarries and the extent of their exploitation. The possible quarries shall be shown to the Contractor prior to commencement of the Works. The Contractor shall be responsible for the acquisitionof the quarry rights and shall conduct respective negotiations with landowners and affected communities.

Alternative sources of gravel material whose quality can be shown to be in compliance with the specification requirements may be used, with the approval of the Engineer. The Contractor is deemed to have included in his rates for the provision of the gravel material.

10-60-001:	Provide Gravel Wearing Course (Excavation, Free haul, spreading and Compaction ofGravel)

Excavation of Gravel

Gravel shall be excavated from quarries approved by the Engineer, and the Contractor shall inform the Engineer if the quality/availability of the gravel changes during the course of excavation.

Stones and boulders with one dimension greater than 80mm shall be removed from the excavated gravel and deposited outside the quarry at locations approved by the Engineer. Such stones and boulders may be reused in other parts of Works with the approval of the Engineer.

Excavation and loading shall normally be by labour unless, at the request of the Contractor, the Engineerallows the use of equipment. The Contractor shall use Labour and/or Equipment to carry out this work, as directed by the Engineer.

Work Method:	LB or MB

Quality Control:

- Oversize stones and boulders shall not be loaded for haulage to the road.

- Areas containing deleterious material shall not be excavated.

Free haul, spreading and Compaction of Gravel

Free haul involves the transportation of gravel material for the first 1.5 km from the quarry. The Contractorshall spread and compact gravel material, in a manner to ensure a uniform thickness of the layer across the

full width of the carriageway and shaped to the specified camber. Spreading also includes the removal of any oversized stones or boulders, which cannot be broken down to the required size, to spoil dumps. Gravel shall be spread within 24 hours of off-loading.

Compaction of the gravel material shall be carried out from the carriageway edges to the centerline by overlapping passes of the compaction equipment. The number of passes shall be as directed by the Engineer dependent upon the equipment used and the material being compacted. Unless otherwise instructed the moisture content of the material shall be within + 2% of optimum

Where additional moisture is required water shall be applied in an even manner and the rate of application shall be such that no transverse or longitudinal flows occur.

The Engineer may instruct the Contractor to carry out density tests on the compacted material to ensure that an acceptable standard has been achieved.

The Contractor shall use Equipment for haulage and Labour for spreading unless the Engineer instructsotherwise. Work Method:	LB-MB

Quality Control:

- The gravel surface width shall be checked at 100m intervals and shall have a tolerance of

+50mm

- Trial holes shall be dug as directed by the Engineer to check the gravel thickness and shall have atolerance of + 5mm / - 0mm

- The camber shall be checked at 50m intervals and the maximum tolerance shall be +1 %

- The longitudinal profile shall be checked after the compaction of each load to ensure a smooth surface with no corrugations or depressions, tolerance of + 10mm.

- Compaction shall show no movement of material under the roller, minimum of 6 passes.

- Compaction test standard shall be 95% MDD (AASHTO T180) Measurement Unit:	m3

The measurement shall be the volume of compacted gravel surfacing measured net according to theDrawings and shall include the excavation and the 1.5km ‘free’ haul distance

Payment

The unit rate shall be the full compensation for labour, tools, equipment and incidental costs required forcarrying out the work.

10 CARRIAGEWAY REPAIR WORKS (GRAVEL)

Scope:

This section covers all routine maintenance works on the gravel carriageway and includes items such as pothole patching, and reshaping of the carriageway, using labour.

10- 50- 006	Light Manual Reshaping (Potholes, Ruts and Gullies)

Description:

This activity involves the removal of all unsuitable/degraded material from the pothole, rut or gully until firm ground is reached, and filling with approved material and ensuring that the area is free draining.

For earth roads material from the side drains may be used.

For gravel roads the gravel shall be obtained from the stacks placed at intervals along the road for this purpose.

The fill material shall be watered, mixed and compacted using suitable tampers to a finished level 25mm above the surrounding road surface.

This activity shall be carried out before and after the rains, or as directed by the Engineer. Work Method The Contractor shall use Labour to carry out this workQuality Control

- The quality of fill material shall be approved by the Engineer

- The minimum compaction to be applied shall be such that no rammer imprint on the surface shall be seen. Measurement Unit:	m3

- The measurement shall be total volume of materials used for the repairs.Payment

Payment

The unit rate shall include full compensation for labour, tools, materials, and incidental costs necessary to carry out the work.

10- 50- 009:	Light Manual Reshaping (Grub edge and Reshape Carriageway)

Description:

This activity involves trimming the edge of the carriageway, grubbing grass from the carriageway and reshaping of the camber of the road to the original standard and shape. No grass shall be grubbed from theshoulders, but it shall be cut to a maximum height of 50mm.

For earth roads materials from the side drains may be used to reshape the carriageway. Where additional suitable material is required to reinstate the camber to the required shape, this material shall be obtained from approved sources nearest to the final deposition area.

Work Method

The Contractor shall apply Labour methods to carry out this item.

Quality Control

- The width of the carriageway including the shoulders shall be checked at 100m intervals with tolerance of +50mm or - 20mm

- The camber shall be checked using camber board at 50m intervals and shall have a tolerance of +/- 1%

Measurement Unit:	m2

The measurement shall be the area of carriageway shaped

Payment

The unit rate shall be the full compensation for labour, tools and incidental costs required for carrying out the work.

SECTION 11:	SHOULDER MAINTENANCE AND REPAIR

# 11-50-001: Shoulder Grading

Where directed by the Engineer the Contractor shall reinstate or re-form the shoulders of the road using either a Towed or Motor grader.

Pegs 400 to 500mm long shall be placed at 10 to 20 m intervals on the edge of the carriageway. Suitable material from the front slope of the side drain shall be bladed and spread on the shoulders (and, if appropriate the carriageway), unless otherwise directed by the Engineer, and the shoulders re-formed to the cross-section dimensions, as shown in Table 10.2. Compaction shall be achieved with the wheels of the equipment used, or by other approved means.

Table 10.2: Overall width ditch

| Road Category | Carriageway width | Overall width to front of Ditches |
| --- | --- | --- |
| A/B/C + Secondary Roads | 6.0 m | 8.4 m |
| D/E + Minor Roads | 5.4 m | 7.8 m |
| RAR Roads | 4.5 m | 6.5 m |
| Minor/RAR roads with insufficient widths or Temporary | 3.5 m | 5.5 m |

Work Method

The Contractor shall use Equipment to carry out this item. Measurement Unit:	km

The measurement shall be the length of shoulder reformed.Payment

The unit rate shall be the full compensation for labour, tools, equipment and incidental costs required for carrying out the work.

SECTION 15:	PAVED CARRIAGEWAY MAINTENANCE

# 15 – 50 – 001:	Pothole Patching - hot mix 15 – 50 –

002:	Pothole Patching - cold mix

This activity involves the repair of potholes, which have developed on the surface of paved roads where there is no evidence of base failure. Potholes shall be defined as local failures usually round or oval in shape covering less than 4.0 sq. m and less than 100mm in depth. Repair of larger areas shall be defined as surface repairs.

The areas to be repaired shall be marked out into rectangular shapes. All failed areas shall be cut back to sound road material. The hole shall extend to the bottom of the base layer, and the sides shall be cut to form a vertical face. All unsuitable material shall be removed and deposited away from the road to the approval of the Engineer.

A prime coat of 60% cationic spray grade emulsion, shall be applied on all the sides and on the bottom of the prepared hole

Premixed bituminous material, similar to the existing surfacing shall be placed in the hole and compacted in two or more layers of regular thickness depending on the depth involved.

The final layer, prior to compaction, shall be overfilled by 20% of its depth. Compaction shall be carried out using a pedestrian vibrating roller, plate compactor or hand rammer as appropriate until the surface is level, with the surrounding material.

A minimum depth of 50mm shall be provided for the cationic premix bituminous backfill.

Premix proportions shall be as follows:

- Sand	38%

- 6.7mm aggregate	57%

- Bitumen emulsion	5% (bitumen content 60-65%)

Work Method

The Contractor shall use Labour and Equipment to carry out this item.Quality Control

- The quality of the repair material shall be to the approval of the Engineer.

- The patch shall be checked using a straight edge and shall be flush with other parts of thecarriageway

- Compaction of the patch shall be checked such that no imprint of the compaction equipment is visible.

Measurement Unit:	m3

The unit of measurement shall be the volume of material used in the repair.Payment

The unit rate for this item shall include the full compensation, labour, tools, materials , equipment and incidental costs required to carry out the work.

# 15 – 50 – 004:	Road Edge Repairs

This activity involves the repair to the edges of paved carriageways, including the rebuilding of pavement layers and the shoulders.

All soft material along the surface edge shall first be removed. The pavement layers shall be rebuilt and thoroughly compacted, in similar materials to the existing pavement. The pavement shall be similarly rebuilt against sound material of the existing pavement, and thoroughly compacted. The top of the compacted fill shall be between 30mm to 50mm below the estimated new surface level.

A trench (100 mm wide x 100mm deep) shall be dug along the surface edge in order to reinforce the newedge. The trench shall be swept clean of all loose material and primed with bitumen emulsion.

A trench shall be filled with cold premix and compacted in layers of a 50mm maximum thickness, finishing flush with the existing surface and maintaining the cross fall. The repaired edge shall be finished off by sheet patching with a light sand layer to absorb surplus bitumen.

Work Method

The Contractor shall use Labour and Equipment to carry out this item.

Quality Control

- The edge repair shall be checked using straight edge to be flush with other parts of thecarriageway

- Compaction of the edge repair shall be such that no imprint of the compaction equipment is visible.

Measurement Unit:	m2

The measurement shall be the area of the repair including the bituminous edge and shoulder

Payment

The unit rate shall be full compensation for labour, tools, materials, equipment, and incidental costs required to carry out the work.

# 15 – 60 – 001: Spot Sealing - (Fine cracks)

This activity involves the repair of alligator cracking (maximum width 3mm) on the carriageway. The area to be sealed shall be marked in chalk and be clean and dry. The bituminous binder shall be as specified by the Engineer and shall be applied with a spray lance or other approved method. Distribution rates shall be 1.5kg/m2 for bitumen emulsion.

Coarse sand, up to 5mm, shall then be spread over the whole surface.Crack sealing shall be carried out during the warm and dry weather.

Work Method

The Contractor shall use Labour and Equipment to carry out this item.Quality Control

Quality Control

- The quality of the repair work shall be to the approval of the Engineer.

- The area of spot sealing shall be checked to be free of cracks.

Measurement	Unit:	m2

The measurement for this item will be the area of fine cracks repaired.Payment

The unit rate shall include full compensation for materials, labour, tools, equipment, and incidental costs required to carry out the work.

15-60-002	Crack Sealing

This activity covers the repair of transverse or longitudinal cracks wider than 3mm on the surface of a surface dressed or bituminous carriageway or shoulder.

The cracks and the surrounding road surface shall be clean and dry. The cracks shall be filled with hot cut back bitumen.

The bitumen shall be distributed using a watering can and shall follow the line of the crack, with the nozzle or spout held close to the road surface, keeping the width of the spread as narrow as possible. Coarse sand, up to 5mm, or crusher dust shall be spread over the strip of the binder.

All crack sealing shall be carried out during the warm and dry weather.

Work Method

The Contractor shall use Labour and Equipment to carry out this item. Quality Control

- The sealing shall be carried out to the satisfaction of the Engineer.

- The area of sealing shall be checked to be free of cracks.

Measurement Unit:	m

The measurement shall be the length of cracks sealed.Payment

The unit rate shall include full compensation for materials, labour, tools and incidental costs required to carry out the work.

15 – 80 – 001 to 005	Base repairs

This activity involves the repair of localised failures of the pavement layers, including the removal of the deformed areas and reconstructing the pavement and surfacing layers including treatment of the bottom of the excavation prior to backfilling.

The areas to be repaired shall be marked and cut out into rectangular shapes. All failed area shall be cut back to sound road material. The sides shall be dressed so that they are at 60o to the horizontal, and the bottom of the hole shall be compacted to a density of 93 % AASHTO T180. All unsuitable material shall be removed and deposited away from the road as directed by the Engineer.

For a bituminous base a prime coat of 60% cationic spray grade emulsion shall be applied to all verticalsurfaces and on the bottom of the prepared hole.

The material for the base repair shall be graded crushed stone, dense bitumen macadam or neat gravel, according to the original base material, and shall be compacted in layers as directed by the Engineer.

15-80-01	:	Graded Crushed Stone

16	“Graded crushed stone” shall be crushed stone with a smooth grading curve, which is within the specified envelope. The stone class shall be given in the special Specifications or as directed by the Engineer.

Work Method

The Contractor shall use Labour and Equipment to carry out this item of work.Quality Control

- The repair shall be carried out to the approval of the Engineer.

- The repair shall be checked with a straight edge and shall be flush with other parts of thecarriageway laterally and longitudinally.

- Compaction of the patch shall be checked such that no imprint of the compaction equipment is visible Measurement Unit		m3

The measurement shall be the volume of graded crushed stone compacted in the repair measured in-site. Payment

The unit rate for this item shall include the full compensation for the materials, tools, labour, equipment and incidental costs, required to carry out the work.

    - :	Dense Bitumen Macadam

Dense bitumen macadam shall be a hot-mixed, hot-laid plant mixture of well-graded aggregate and penetration grade bitumen, as specified for road-base material.

Work method

The Contractor shall use Labour and Equipment to carry out this item.Quality Control

      - The repair shall be carried out to the approval of the Engineer

      - The repair shall be checked with a straight edge and shall be flush with other parts of thecarriageway laterally and longitudinally.

      - Compaction of the patch shall be checked such that no imprint of the compaction equipment is visible.

Measurement Unit:	m3

The measurement shall be the volume of dense bitumen macadam compacted on the road measured in- situ. Payment

The unit rate for this item shall include the full compensation for the materials, tools, labour, equipment, and incidental costs required to carry out the work.

15-80-005: Neat Gravel

“Gravel “ includes lateritic gravel, quartzitic gravel, calcareous gravel, soft stone, coral rag, clayey sands, decomposed rock, crushed rock or a combination of any of these materials.

Work method

The Contractor shall use Labour and Equipment to carry out this item of workQuality Control

- The repair shall be carried out to the approval of the Engineer.

- The repair shall be checked with a straight edge and shall be flush with other parts of thecarriageway laterally and longitudinally.

- Compaction of the patch shall be checked such that no imprint of the compaction equipment shall be visible.

Measurement Unit:	m3

The measurement shall be the volume of gravel compacted on the road measured in-situ.Payment

The unit rate for this item shall include the full compensation for the materials, labour, tools, equipment, and incidental costs required to carry out the work.

SECTION 17: BRIDGE MAINTENANCE

17 - 50 - 001:	Cleaning and Clearing - Deck

# 17 - 50 - 002:	Cleaning and Clearing - Riverbed

Description

This activity involves the following tasks,

- Brooming of the deck and sidewalks to clear all loose soil, dirt, aggregate, debris, and removal of the same from site

- Removal of all dirt and stones lodged between deck planking

- Clearance of weep holes of dirt and debris

- Removal of all dirt and debris in joints between beams and abutments or any point under bridge

- Removal of debris lodged at pier and abutments or any point under the bridge

- Removal of trees and bushes growing under the bridge and directly upstream and downstream and disposing the material away from the bridge and stream

- Cleaning of any signs or reflectors on or at the bridge.

- Removal of termites tunnels in the vicinity of timber bridges and soaking the ground with chemical solutions against wood destroying insects.

Work Method

The Contractor shall use Labour to carry out this item.

Quality Control

- The bridge deck shall be checked to be clean and free of dirt and debris The passage below the deck shall be checked to be free of obstructions

Payment

Payment shall be made for the labour and material instructed by the Engineer to be used on the work

SECTION 20: ROAD FURNITURE REPAIR AND MAINTENANCE

20-50- 001: Traffic Sign Maintenance

This activity involves all the tasks required to ensure that the road signs and signposts are in a clean, properly aligned, vertical and secure condition; the replacement of missing or broken bolts, nuts or other fixings and the tightening of the same. The maintenance shall also extend to securing any loose posts by the re-compacting or removal of any unsuitable material surrounding the posts, importing and compacting of suitable material to render the post secure

Work Method

The Contractor shall use Labour to carry out this item.

Quality Control

- The signs shall be clean and in vertical position

- The fixings shall be hand checked to be tight Measurement Unit:	No.

The measurement shall be number of signs maintained.Payment

The unit rate shall be the full compensation for labour, tools, material and incidental costs required to carry out the work.

20-50- 003:	Guardrail Repair

This activity involves the repair of Guardrails (including rails, posts and fixings) to a properly aligned, vertical and secure condition.

The repair shall include securing any loose posts by re-compaction or removal of any unsuitable material surrounding the post, importing and compaction of suitable materials to render the posts secure, and the re-fixing of the rails

Work Method

The Contractor shall use Labour to carry out this item.

Quality Control

- The guardrails shall be checked as being properly aligned secure and in a vertical position

- The fixings shall be hand checked to be firmly fixed Measurement Unit:	m

The measurement shall be the length of Guardrail repairedPayment

The unit rate shall be the full compensation for labour, material, tools, and incidental costs required to carry out the work.

- 50- 004: Marker Posts Replacement

This activity involves the replacement of kilometre stones and culvert marker posts

The Engineer shall determine the location of the marker stones and posts. They shall be set in a simple excavation and backfilled with soil. The depth of the excavation shall be determined on the site, depending on the size and shape of the marker stone or post.

Work Method

The Contractor shall use Labour to carry out this item.

Quality Control

  - The posts shall be vertical and firmly bedded to the approval of the Engineer

  - Measurement Unit:	No

  - The measurement shall be in number of marker posts. Payment

The unit rate shall be the full compensation for labour, tools, posts, materials and incidental costs required to carry out the work.

20-50-006: Sign cleaning and repainting

This activity involves the cleaning of road signs, reflectors and guideposts and repainting of sign supporting structures, rear panels of signs, kilometre posts and culvert markers. Surfaces to be painted shall be clean, dry and free from loose material.

Work Method

The Contractor shall use Labour to carry out this item of work

Quality Control

- The cleaning and painting shall be carried out to the approval of the Engineer Measurement Unit:	No

The measurement shall be in number of signs, reflectors, or guideposts cleaned or painted. Payment

The unit rate shall be the full compensation for labour, materials, tools and incidental costs required to carry out the work

SECTION 25:	HIV/AIDS AWARENESS AND PREVENTION CAMPAIGN

Scope:

This section covers the Contractors obligations with regard to on-site HIV / AIDS awareness campaign and preventive measures that are to be instituted.

    - HIV / AIDS Awareness Campaign

The Contractor shall institute an HIV / AIDS awareness campaign amongst the workers for the duration ofthe Contract.

As part of the campaign the Contractor shall display AIDS awareness posters in all buildings frequented by workers employed on the Contract, where such buildings fall under the control of the Contractor.

In addition at least two of the Contractors vehicles regularly used on site shall display HIV / AIDS awareness posters. The posters shall be printed on gloss paper and shall be at least A1 size on buildings and A3 size or other approved size on vehicles. The message on the posters shall be supplied by the Employer through the Engineer.

Aids awareness shall also be included in the orientation process of all workers employed on the Contract.

Measurement Unit:	month

The measurement shall be the calendar month or part thereof, measured over the duration of thecampaign

Payment

The unit rate shall include full compensation for labour and material required for the provision of the item.

    - AIDS Prevention Campaign

The Contractor shall institute an HIV / AIDS prevention campaign amongst the workers for the duration ofthe Contract

As part of the campaign the Contractor shall make condoms available to the workers. The Employerthrough the Engineer shall supply the condoms.

Measurement Unit:	month

The measurement shall be the calendar month measured over the duration of the campaign. Payment

The unit rate shall include full compensation for labour and material including the distribution of condoms, required for the provision of the item.

SECTION VII- BILLS OF QUANTITIES

- Objectives

The objectives of the Bill of Quantities are:

  - to provide sufﬁcient information on the quantities of Works to be performed to enable tenders to be

prepared efﬁciently and accurately; and

  - when a Contract has been entered into, to provide a priced Bill of Quantities for use in the periodic valuation of Works executed.

In order to attain these objectives, Works should be itemized in the Bill of Quantities in sufﬁcient detail to distinguish between the different classes of Works, or between Works of the same nature carried out in different locations or in other circumstances which may give rise to different considerations of cost. Consistent with these requirements, the layout and contents of the Bill of Quantities should be as simple and brief as possible.

- Day work Schedule

ADay work Schedule should be included only if the probability of unforeseen work, outside the items included in the Bill of Quantities, is high. To facilitate checking by the Procuring Entity of the realism of rates quoted by the Tenderers, the Day work Schedule should normally comprise the following:

  - A list of the various classes of labor, materials, and Constructional Plant for which basic day work rates or prices are to be inserted by the Tenderer, together with a statement of the conditions under which the Contractor shall be paid for work executed on a day work basis.

  - Nominal quantities for each item of day work, to be priced by each Tenderer at day work rates as Tender. The rate to be entered by the Tenderer against each basic day work item should include the Contractor's proﬁt, overheads, supervision, and other charges.

- Provisional Sums

Ageneral provision for physical contingencies (quantity overruns) may be made by including a provisional sum in the Summary Bill of Quantities. Similarly, a contingency allowance for possible price increases should be provided as a provisional sum in the Summary priced Bill of Quantities. The inclusion of such provisional sums often facilitates budgetary approval by avoiding the need to request periodic supplementary approvals as the future need arises. Where such provisional sums or contingency allowances are used, the Special Conditions of Contract should state the manner in which they shall be used, and under whose authority (usually the Project Manager's).

The estimated cost of specialized work to be carried out, or of special goods to be supplied, by other contractors should be indicated in the relevant part of the Bill of Quantities as a particular provisional sum with an appropriate brief description. Aseparate procurement procedure is normally carried out by the Procuring Entity to select such specialized contractors. To provide an element of competition among the Tenderers in respect of any facilities, amenities, attendance, etc., to be provided by the successful Tenderer as prime Contractor for the use and convenience of the specialist contractors, each related provisional sum should be followed by an item in the Bill of Quantities inviting the Tenderer to quote a sum for such amenities, facilities, attendance, etc.

These Notes for Preparing a Bill of Quantities are intended only as information for the Procuring Entity or the person drafting the tendering document. They should not be included in the ﬁnal tendering document.

- The Bills of Quantities

The APPLICABLE Bills of Quantities shall be divided generally as below:

  - Preambles

  - Preliminary items

  - Work Items

- Daywork Schedule and

- Provisional items

Summary.

SECTION VII - BILLS OF QUANTITIES

| Road Code |  | UK28/904 | Section Name |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Package: KeRRA/008/KEMA/KS/GOK/039/106/22¦23 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Contractor |  | 0 |  |  |  |  |  |  |  |  |  |  |  |
| Bill of Quantities |  |  |  |  |  |  |  |  |  | Page: 1 |  |  |  |  |  |
| Bill No.1 |  | General: Office administration and overheads/Preliminaries |  |  |  |  |  |  |  | Project: |  |  |  |
| Item No. |  | Description | Units | Quantity |  |  | Unit Bid Rate(Ksh) |  |  | Amount KSh |  | Technology |  |
| 01-50-014 |  | Field supervision | KS | 200000 |  |  |  |  |  | - |  | LB-MB |  |
| 01-60-005 |  | Publicity Sign Boards | KS | 2 |  |  |  |  |  | - |  | LB-MB |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  | Total Carried Forward to Summary: |  |  |  |  |  |  |  | - |  |  |  |
| Road Code |  | UK28/904 | Section Name |  |  |  |  |  |  |  |  |  |  |
| Package: KeRRA/008/KEMA/KS/GOK/039/106/22¦23 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Contractor |  | 0 |  |  |  |  |  |  |  |  |  |  |  |
| Bill of Quantities |  |  |  |  |  |  |  |  |  |  | Page: 2 |  |  |  |
| Bill No.8 | CULVERT AND DRAINAGE WORKS |  |  |  |  |  |  |  |  |  | Project: |  |  |  |
| Item No. | Description |  | Units |  | Quantity |  |  | Unit Bid Rate(Ksh) |  |  | Amount KSh |  | Technology |  |
| 08-60-027 | Culvert Installation 900 mm with surround |  | METERS |  | 21 |  |  |  |  |  | - |  | LB |  |
|  |  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | - |  |  |  |
|  | Total Carried Forward to Summary: |  |  |  |  |  |  |  |  |  | - |  |  |  |

| Road Code | UK28/904 |  | Section Name |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Package: KeRRA/008/KEMA/KS/GOK/039/106/22¦23 |  |  |  |  |  |  |  |  |  |  |  |  |
| Contractor | 0 |  |  |  |  |  |  |  |  |  |  |  |
| Bill of Quantities |  |  |  |  |  |  |  | Page: 3 |  |  |
| Bill No.10 | GRADING AND GRAVELLING WORKS |  |  |  |  |  |  |  |  | Project: |  |  |
| Item No. | Description |  | Units | Quantity |  |  | Unit Bid Rate(Ksh) |  |  | Amount KSh |  | Technology |
| 10-50-001 | Heavy grading without watering or compaction instructed by the Engineer |  | M² | 38400 |  |  |  |  |  | - |  | MB |
| 10-60-003 | Gravel Patching |  | M³ | 4998.59 |  |  |  |  |  | - |  | MB |
|  |  |  |  |  |  |  |  |  |  | - |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |
|  |  |  |  |  |  |  |  |  |  | - |  |  |
|  | Total Carried Forward to Summary: |  |  |  |  |  |  |  |  | - |  |  |

| Road Code | UK28/904 | Section Name |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Package: KeRRA/008/KEMA/KS/GOK/039/106/22¦23 |  |  |  |  |  |  |  |  |  |  |  |
| Contractor | 0 |  |  |  |  |  |  |  |  |  |  |
| Bill of Quantities |  |  |  |  |  |  |  |  | Page: 4 |  |
| Bill No.22 | DAYWORKS |  |  |  |  |  | Project: |  |  |  |  |
| Item No. | Description | Units | Quantity |  | Unit Bid Rate(Ksh) |  | Amount KSh |  |  | Technology |  |
| 22-63-009 | Crawler Tractor (e.g. D6) : 140 - 160 HP | EH | 72.5 |  |  |  | - |  |  | LB-MB |  |
|  |  |  |  |  |  |  | - |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  |  |
|  |  |  |  |  |  |  | - |  |  |  |  |
|  | Total Carried Forward to Summary: |  |  |  |  |  | - |  |  |  |  |

| RoadCode UK28/904 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Structure: Package: KeRRA/008/KEMA/KS/GOK/039/106/22¦23 |  | Section Name UFANISI-ENDO KABOR |  |  |
| Contractor 0 |  |  |  |  |
| Bill of Quantities |  | Page: 1 |  |
|  | Summary | Project: |  |  |
| Item No. | Description | Amount (KShs) |  |  |
| 1 | General: Office administration and overheads/Preliminaries | - |  |  |
| 8 | CULVERT AND DRAINAGE WORKS | - |  |  |
| 10 | GRADING AND GRAVELLING WORKS | - |  |  |
| 22 | DAYWORKS | - |  |  |
|  | Sub Total | - |  |  |
|  | VAT @ 16 % | - | 0.16 |  |
|  | Total | - |  |  |
|  | Contingencies ( @ 0 %) | - |  |  |

PART III - CONDITIONS OF CONTRACT AND CONTRACT FORMS

SECTION VIII - GENERAL CONDITIONS OF CONTRACT

These General Conditions of Contract (GCC), read in conjunction with the Special Conditions of Contract (SCC) and other documents listed therein, should be a complete document expressing fairly the rights and obligations of both parties.

These General Conditions of Contract have been developed on the basis of considerable international experience in the drafting and management of contracts, bearing in mind a trend in the construction industry towards simpler, more straightforward language.

The GCC can be used for both smaller admeasurement contracts and lump sum contracts.

General Conditions of Contract

- General

- Deﬁnitions

Boldface type is used to identify deﬁned terms.

  - The Accepted Contract Amount means the amount accepted in the Letter of Acceptance for the execution and completion of the Works and the remedying of any defects.

  - The Activity Schedule is a schedule of the activities comprising the construction, installation, testing, and commissioning of the Works in a lump sum contract. It includes a lump sum price for each activity, which is used for valuations and for assessing the effects of Variations and Compensation Events.

  - The Adjudicator is the person appointed jointly by the Procuring Entity and the Contractor to resolve disputes in the ﬁrst instance, as provided for in GCC 23.

  - Bill of Quantities means the priced and completed Bill of Quantities forming part of the Bid.

  - Compensation Events are those deﬁned in GCC Clause 42 hereunder.

  - The Completion Date is the date of completion of the Works as certiﬁed by the Project Manager, in accordance with GCC Sub-Clause 53.1.

  - The Contract is the Contract between the Procuring Entity and the Contractor to execute, complete, and maintain the Works. It consists of the documents listed in GCC Sub-Clause 2.3 below.

  - The Contractor is the party whose Bid to carry out the Works has been accepted by the Procuring Entity.

  - The Contractor's Bid is the completed bidding document submitted by the Contractor to the Procuring Entity.

  - The Contract Price is the Accepted Contract Amount stated in the Letter of Acceptance and thereafter as adjusted in accordance with the Contract.

  - Days are calendar days; months are calendar months.

  - Day works are varied work inputs subject to payment on a time basis for the Contractor's employees and Equipment, in addition to payments for associated Materials and Plant.

  - ADefect is any part of the Works not completed in accordance with the Contract.

  - The Defects Liability Certiﬁcate is the certiﬁcate issued by Project Manager upon correction of defects by the Contractor.

  - The Defects Liability Period is the period named in the SCC pursuant to Sub-Clause 34.1 and calculated from the Completion Date.

  - Drawings means the drawings of the Works, as included in the Contract, and any additional and modiﬁed drawings issued by (or on behalf of) the Procuring Entity in accordance with the Contract, include calculations and other information provided or approved by the Project Manager for the execution of the Contract.

  - The Procuring Entity is the party who employs the Contractor to carry out the Works, as specﬁed in the SCC, who is also the Procuring Entity

  - Equipment is the Contractor's machinery and vehicles brought temporarily to the Site to construct the Works.

  - “In writing” or “written” means hand-written, type-written, printed or electronically made, and resulting in a permanent record;

  - The Initial Contract Price is the Contract Price listed in the Procuring Entity's Letter of Acceptance.

  - The Intended Completion Date is the date on which it is intended that the Contractor shall complete the Works. The Intended Completion Date is speciﬁed in the SCC. The Intended Completion Date may be revised only by the Project Manager by issuing an extension of time or an acceleration order.

  - Materials are all supplies, including consumables, used by the Contractor for incorporation in the Works.

  - Plant is any integral part of the Works that shall have a mechanical, electrical, chemical, or biological function.

  - The Project Manager is the person named in the SCC (or any other competent person appointed by the Procuring Entity and notiﬁed to the Contractor, to act in replacement of the Project Manager) who is responsible for supervising the execution of the Works and administering the Contract.

  - SCC means Special Conditions of Contract.

  - The Site is the area of the works as deﬁned as such in the SCC.

aa) Site Investigation Reports are those that were included in the bidding document and are factual and interpretative reports about the surface and subsurface conditions at the Site.

bb) Speciﬁcation means the Speciﬁcation of the Works included in the Contract and any modiﬁcation or addition made or approved by the Project Manager.

cc) The Start Date is given in the SCC. It is the latest date when the Contractor shall commence execution of the Works. It does not necessarily coincide with any of the Site Possession Dates.

dd) A Subcontractor is a person or corporate body who has a Contract with the Contractor to carry out a part of the work in the Contract, which includes work on the Site.

ee) Temporary Works are works designed, constructed, installed, and removed by the Contractor that are needed for construction or installation of the Works.

ff) A Variation is an instruction given by the Project Manager which varies the Works.

gg) The Works are what the Contract requires the Contractor to construct, install, and turn over to the Procuring Entity, as deﬁned in the SCC.

- Interpretation

In interpreting these GCC, words indicating one gender include all genders. Words indicating the singular also include the plural and words indicating the plural also include the singular. Headings have no signiﬁcance. Words have their normal meaning under the language of the Contract unless speciﬁcally deﬁned. The Project Manager shall provide instructions clarifying queries about these GCC.

If sectional completion is speciﬁed in the SCC, references in the GCC to the Works, the Completion Date, and the Intended Completion Date apply to any Section of the Works (other than references to the Completion Date and Intended Completion Date for the whole of the Works).

The documents forming the Contract shall be interpreted in the following order of priority:

  - Agreement,

  - Letter of Acceptance,

  - Contractor's Bid,

  - Special Conditions of Contract,

  - General Conditions of Contract, includingAppendices,

  - Speciﬁcations,

  - Drawings,

  - Bill of Quantities6, and

  - any other document listed in the SCC as forming part of the Contract.

6In lump sum contrcts, delete “Bill of Quantities” and replace with “Activity Schedule.”

- Language andLaw

The language of the Contract is English Language and the law governing the Contract are the Laws of Kenya.

Throughout the execution of the Contract, the Contractor shall comply with the import of goods and services prohibitions in the Procuring Entity's Country when

  - as a matter of law or ofﬁcial regulations, Kenya prohibits commercial relations with that country; or

  - by an act of compliance with a decision of the United Nations Security Council taken under Chapter VII of the Charter of the United Nations, Kenya prohibits any import of goods from that country or any payments to any country, person, or entity in that country.

- Project Manager's Decisions

Except where otherwise speciﬁcally stated, the Project Manager shall decide contractual matters between the Procuring Entity and the Contractor in the role representing the Procuring Entity.

- Delegation

Otherwise speciﬁed in the SCC, the Project Manager may delegate any of his duties and responsibilities to other people, except to the Adjudicator, after notifying the Contractor, and may revoke any delegation after notifying the Contractor.

- Communications

Communications between parties that are referred to in the Conditions shall be effective only when in writing. A notice shall be effective only when it is delivered.

- Subcontracting

The Contractor may subcontract with the approval of the Project Manager, but may not assign the Contract without the approval of the Procuring Entity in writing. Subcontracting shall not alter the Contractor's obligations.

- Other Contractors

The Contractor shall cooperate and share the Site with other contractors, public authorities, utilities, and the Procuring Entity between the dates given in the Schedule of Other Contractors, as referred to in the SCC. The Contractor shall also provide facilities and services for them as described in the Schedule. The Procuring Entity may modify the Schedule of Other Contractors, and shall notify the Contractor of any such modiﬁcation

- Personnel and Equipment

The Contractor shall employ the key personnel and use the equipment identiﬁed in its Bid, to carry out the Works or other personnel and equipment approved by the Project Manager. The Project Manager shall approve any proposed replacement of key personnel and equipment only if their relevant qualiﬁcations or characteristics are substantially equal to or better than those proposed in the Bid.

If the Project Manager asks the Contractor to remove a person who is a member of the Contractor's staff or work force, stating the reasons, the Contractor shall ensure that the person leaves the Site within seven days and has no further connection with the work in the Contract.

If the Procuring Entity, Project Manager or Contractor determines, that any employee of the Contractor be determined to have engaged in Fraud and Corruption during the execution of the Works, then that employee shall be removed in accordance with Clause 9.2 above.

- Procuring Entity's and Contractor's Risks

The Procuring Entity carries the risks which this Contract states are Procuring Entity's risks, and the Contractor carries the risks which this Contract states are Contractor's risks.

- Procuring Entity's Risks

From the Start Date until the Defects Liability Certiﬁcate has been issued, the following are Procuring Entity's risks:

  - The risk of personal injury, death, or loss of or damage to property (excluding the Works, Plant, Materials, and Equipment), which are due to

    - use or occupation of the Site by the Works or for the purpose of the Works, which is the unavoidable result of the Works or

    - negligence, breach of statutory duty, or interference with any legal right by the Procuring Entity or by any person employed by or contracted to him except the Contractor.

  - The risk of damage to the Works, Plant, Materials, and Equipment to the extent that it is due to a fault of the Procuring Entity or in the Procuring Entity's design, or due to war or radioactive contamination directly affecting the country where the Worksare to be executed.

From the Completion Date until the Defects Liability Certiﬁcate has been issued, the risk of loss of or damage to the Works, Plant, and Materials is a Procuring Entity's risk except loss or damage due to

- a Defect which existed on the Completion Date,

- an event occurring before the Completion Date, which was not itself a Procuring Entity's risk, or

- the activities of the Contractor on the Site after the Completion Date.

- Contractor's Risks

From the Starting Date until the Defects Liability Certiﬁcate has been issued, the risks of personal injury, death, and loss of or damage to property (including, without limitation, the Works, Plant, Materials, and Equipment) which are not Procuring Entity's risks are Contractor's risks.

- Insurance

The Contractor shall provide, in the joint names of the Procuring Entity and the Contractor, insurance cover from the Start Date to the end of the Defects Liability Period, in the amounts and deductibles stated in the SCC for the following events which are due to the Contractor's risks:

  - loss of or damage to the Works, Plant, and Materials;

  - loss of or damage to Equipment;

  - loss of or damage to property (except the Works, Plant, Materials, and Equipment) in connection with the Contract; and

  - personal injury or death.

Policies and certiﬁcates for insurance shall be delivered by the Contractor to the Project Manager for the Project Manager's approval before the Start Date. All such insurance shall provide for compensation to be payable in the types and proportions of currencies required to rectify the loss or damage incurred.

If the Contractor does not provide any of the policies and certiﬁcates required, the Procuring Entity may effect the insurance which the Contractor should have provided and recover the premiums the Procuring Entity has paid from payments otherwise due to the Contractor or, if no payment is due, the payment of the premiums shall be a debt due.

Alterations to the terms of an insurance shall not be made without the approval of the Project Manager. Both parties shall comply with any conditions of the insurance policies.

- Site Data

The Contractor shall be deemed to have examined any Site Data referred to in the SCC, supplemented by any information available to the Contractor.

- Contractor to Construct the Works

The Contractor shall construct and install the Works in accordance with the Speciﬁcations and Drawings.

- The Works to Be Completed by the Intended Completion Date

The Contractor may commence execution of the Works on the Start Date and shall carry out the Works in accordance with the Program submitted by the Contractor, as updated with the approval of the Project Manager, andcomplete them by the Intended Completion Date.

- Approval by the Project Manager

The Contractor shall submit Speciﬁcations and Drawings showing the proposed Temporary Works to the Project Manager, for his approval.

The Contractor shall be responsible for design of Temporary Works.

The Project Manager's approval shall not alter the Contractor's responsibility for design of the Temporary Works. The Contractor shall obtain approval of third parties to the design of the Temporary Works, where required.

All Drawings prepared by the Contractor for the execution of the temporary or permanent Works, are subject to prior approval by the Project Manager before this use.

- Safety

The Contractor shall be responsible for the safety of all activities on the Site.

- Discoveries

Anything of historical or other interest or of signiﬁcant value unexpectedly discovered on the Site shall be the property of the Procuring Entity. The Contractor shall notify the Project Manager of such discoveries and carry out the Project Manager's instructions for dealing with them.

- Possession of the Site

The Procuring Entity shall give possession of all parts of the Site to the Contractor. If possession of a part is not given by the date stated in the SCC, the Procuring Entity shall be deemed to have delayed the start of the relevant activities, and this shall be a Compensation Event.

- Access to the Site

The Contractor shall allow the Project Manager and any person authorized by the Project Manager access to the Site and to any place where work in connection with the Contract is being carried out or is intended to be carried out.

- Instructions, Inspections and Audits

The Contractor shall carry out all instructions of the Project Manager which comply with the applicable laws where the Site is located.

The Contractor shall keep, and shall make all reasonable efforts to cause its Subcontractors and sub-consultants to keep, accurate and systematic accounts and records in respect of the Works in such form and details as will clearly identify relevant time changes and costs.

The Contractor shall permit and shall cause its subcontractors and sub-consultants to permit, the Procuring Entity and/or persons appointed by the Public Procurement Regulatory Authority to inspect the Site and/or the accounts and records relating to the procurement process, selection and/or contract execution, and to have such accounts and records audited by auditors appointed by the Public Procurement Regulatory Authority. The Contractor's and its Subcontractors' and sub-consultants' attention is drawn to Sub-Clause 25.1 (Fraud and Corruption) which provides, inter alia, that acts intended to materially impede the exercise of the Public Procurement Regulatory Authority's inspection and audit rights constitute a prohibited practice subject to contract termination (as well as to a determination of ineligibility pursuant to the Public Procurement Regulatory Authority's prevailing sanctions procedures).

- Appointment of the Adjudicator

The Adjudicator shall be appointed jointly by the Procuring Entity and the Contractor, at the time of the Procuring Entity's issuance of the Letter of Acceptance. If, in the Letter of Acceptance, the Procuring Entity does not agree on the appointment of the Adjudicator, the Procuring Entity will request the Appointing Authority designated in the SCC, to appoint the Adjudicator within 14 days of receipt of such request.

Should the Adjudicator resign or die, or should the Procuring Entity and the Contractor agree that the Adjudicator is not functioning in accordance with the provisions of the Contract, a new Adjudicator shall be jointly appointed by the Procuring Entity and the Contractor. In case of disagreement between the Procuring Entity and the Contractor, within 30 days, the Adjudicator shall be designated by the Appointing Authority designated in the SCC at therequest of either party, within 14 days of receipt of such request.

- Settlement of Claims and Disputes

Contractor's Claims

If the Contractor considers itself to be entitled to any extension of the Time for Completion and/or any additional payment, under any Clause of these Conditions or otherwise in connection with the Contract, the Contractor shall give Notice to the Project Manager, describing the event or circumstance giving rise to the claim. The notice shall be given as soon as practicable, and not later than 30 days after the Contractor became aware, orshould have become aware, of the event or circumstance.

If the Contractor fails to give notice of a claim within such period of 30 days, the Time for Completion shall not be extended, the Contractor shall not be entitled to additional payment, and the Procuring Entity shall be discharged from all liability in connection with the claim. Otherwise, the following provisions of this Sub- Clause shall apply.

The Contractor shall also submit any other notices which are required by the Contract, and supporting particulars for the claim, all as relevant to such event or circumstance.

The Contractor shall keep such contemporary records as may be necessary to substantiate any claim, either on the Site or at another location acceptable to the Project Manager. Without admitting the Procuring Entity's liability, the Project Manager may, after receiving any notice under this Sub-Clause, monitor the record- keeping and/or instruct the Contractor to keep further contemporary records. The Contractor shall permit the Project Manager to inspect all these records, and shall (if instructed) submit copies to the Project Manager.

Within 42 days after the Contractor became aware (or should have become aware) of the event or circumstance giving rise to the claim, or within such other period as may be proposed by the Contractor and approved by the Project Manager, the Contractor shall send to the Project Manager a fully detailed claim which includes full supporting particulars of the basis of the claim and of the extension of time and/or additional payment claimed. If the event or circumstance giving rise to the claim has a continuing effect:

  - this fully detailed claim shall be considered as interim;

  - the Contractor shall send further interim claims at monthly intervals, giving the accumulated delay and/or amount claimed, and such further particulars as the Project Manager may reasonably require; and

  - the Contractor shall send a ﬁnal claim within 30 days after the end of the effects resulting from the event or circumstance, or within such other period as may be proposed by the Contractor and approved by the Project Manager.

Within 42 days after receiving a Notice of a claim or any further particulars supporting a previous claim, or within such other period as may be proposed by the Project Manager and approved by the Contractor, the Project Manager shall respond with approval, or with disapproval and detailed comments. He may also request any necessary further particulars, but shall nevertheless give his response on the principles of the claim within the above deﬁned time period.

Within the above deﬁned period of 42 days, the Project Manager shall proceed in accordance with Sub-Clause

3.5 [Determinations] to agree or determine (i) the extension (if any) of the Time for Completion (before or after its expiry) in accordance with Sub-Clause 8.4 [Extension of Time for Completion], and/or (ii) the additional payment (if any) to which the Contractor is entitled under the Contract.

Each Payment Certiﬁcate shall include such additional payment for any claim as has been reasonably substantiated as due under the relevant provision of the Contract. Unless and until the particulars supplied are sufﬁcient to substantiate the whole of the claim, the Contractor shall only be entitled to payment for such part of the claim as he has been able to substantiate.

If the Project Manager does not respond within the timeframe deﬁned in this Clause, either Party may consider that the claim is rejected by the Project Manager and any of the Parties may refer to Arbitration in accordance with Sub-Clause 24.4 [Arbitration].

The requirements of this Sub-Clause are in addition to those of any other Sub-Clause which may apply to a claim. If the Contractor fails to comply with this or another Sub-Clause in relation to any claim, any extension of time and/or additional payment shall take account of the extent (if any) to which the failure has prevented or prejudiced proper investigation of the claim, unless the claim is excluded under the second paragraph of this Sub-Clause 24.3.

Amicable Settlement

Where a notice of a claim has been given, both Parties shall attempt to settle the dispute amicably before the commencement of arbitration. However, unless both Parties agree otherwise, the Party giving a notice of a claim in accordance with Sub-Clause 24.1 above should move to commence arbitration after the ﬁfty-sixth day from the day on which a notice of a claim was given, even if no attempt at an amicable settlement has been made.

Matters thatmay be referred to arbitration

Notwithstanding anything stated herein the following matters may be referred to arbitration before the practical completion of the Works or abandonment of the Works or termination of the Contract by either party:

- The appointment of a replacement Project Manager upon the said person ceasing to act.

- Whether or not the issue of an instruction by the Project Manager is empowered by these Conditions.

- Whether or not a certiﬁcate has been improperly withheld or is not in accordance with these Conditions.

- Any dispute arisingin respect of war risks or war damage.

- All other matters shall only be referred to arbitration after the completion or alleged completion of the Works or termination or alleged termination of the Contract, unless the Procuring Entity and the Contractor agree otherwise in writing.

Arbitration

Any claim or dispute between the Parties arising out of or in connection with the Contract not settled amicably in accordance with Sub-Clause 24.3 shall be ﬁnally settled by arbitration.

No arbitration proceedings shall be commenced on any claim or dispute where notice of a claim or dispute has not been given by the applying party within ninety days of the occurrence or discovery of the matter or issue giving rise to the dispute.

Notwithstanding the issue of a notice as stated above, the arbitration of such a claim or dispute shall not commence unless an attempt has in the ﬁrst instance been made by the parties to settle such claim or dispute amicably with or without the assistance of third parties. Proof of such attempt shall be required.

The Arbitrator shall, without prejudice to the generality of his powers, have powers to direct such measurements, computations, tests or valuations as may in his opinion be desirable in order to determine the rights of the parties and assess and award any sums which ought to have been the subject of or included in any certiﬁcate.

The Arbitrator shall, without prejudice to the generality of his powers, have powers to open up, review and revise any certiﬁcate, opinion, decision, requirement or notice and to determine all matters in dispute which shall be submitted to him in the same manner as if no such certiﬁcate, opinion, decision requirement or notice had been given.

The arbitrators shall have full power to open up, review and revise any certiﬁcate, determination, instruction, opinion or valuation of the Project Manager, relevant to the dispute. Nothing shall disqualify representatives of the Parties and the Project Manager from being called as a witness and giving evidence before the arbitrators on any matter whatsoever relevant to the dispute.

Neither Party shall be limited in the proceedings before the arbitrators to the evidence, or to the reasons for dissatisfaction given in its Notice of Dissatisfaction.

Arbitration may be commenced prior to or after completion of the Works. The obligations of the Parties, and the Project Manager shall not be altered by reason of any arbitration being conducted during the progress of the Works.

    - The terms of the remuneration of each or all the members of Arbitration shall be mutually agreed upon by the Parties when agreeing the terms of appointment. Each Party shall be responsible for paying one-half of this remuneration.

Arbitration with National Contractors

If the Contract is with national contractors, arbitration proceedings will be conducted in accordance with the Arbitration Laws of Kenya. In case of any claim or dispute, such claim or dispute shall be notiﬁed in writing by either party to the other with a request to submit it to arbitration and to concur in the appointment of an Arbitrator within thirty days of the notice. The dispute shall be referred to the arbitration and ﬁnal decision of a person to be agreed between the parties. Failing agreement to concur in the appointment of anArbitrator, the Arbitrator shall be appointed, on the request of the applying party, by the Chairman or Vice Chairman of any of the following professional institutions;

      - Architectural Association of Kenya

      - Institute of Quantity Surveyors of Kenya

      - Association of Consulting Engineers of Kenya

      - Chartered Institute of Arbitrators (Kenya Branch)

      - Institution of Engineers of Kenya

The institution written to ﬁrst by the aggrieved party shall take precedence over all other institutions.

Alternative Arbitration Proceedings

Alternatively, the Parties may refer the matter to the Nairobi Centre for International Arbitration (NCIA) which offers a neutral venue for the conduct of national and international arbitration with commitment to providing institutional support to the arbitral process.

Failure to Comply with Arbitrator's Decision

The award of such Arbitrator shall be ﬁnal and binding upon the parties.

In the event that a Party fails to comply with a ﬁnal and binding Arbitrator's decision, then the other Party may, without prejudice to any other rights it may have, refer the matter to a competent court of law.

Contract operations to continue

Notwithstanding any reference to arbitration herein,

- the parties shall continue to perform their respective obligations under the Contract unless they otherwise agree; and

- the Procuring Entity shall pay the Contractor any monies due the Contractor.

- Fraud and Corruption

The Government requires compliance with the country's Anti-Corruption laws and its prevailing sanctions policies and procedures as set forth in the Constitution of Kenya and its Statutes.

The Procuring Entity requires the Contractor to disclose any commissions or fees that may have been paid or are to be paid to agents or any other party with respect to the bidding process or execution of the Contract. The information disclosed must include at least the name and address of the agent or other party, the amount and currency, and the purpose of the commission, gratuity or fee.

- Time Control

- Program

Within the time stated in the SCC, after the date of the Letter of Acceptance, the Contractor shall submit to the Project Manager for approval a Program showing the general methods, arrangements, order, and timing for all the activities in the Works. In the case of a lump sum contract, the activities in the Program shall be consistent with those in the Activity Schedule.

An update of the Program shall be a program showing the actual progress achieved on each activity and the effect of the progress achieved on the timing of the remaining work, including any changes to the sequence of the activities.

The Contractor shall submit to the Project Manager for approval an updated Program at intervals no longer than the period stated in the SCC. If the Contractor does not submit an updated Program within this period, the Project Manager may withhold the amount stated in the SCC from the next payment certiﬁcate and continue to withhold this amount until the next payment after the date on which the overdue Program has been submitted. In the case of a lump sum contract, the Contractor shall provide an updated Activity Schedule within 14 days of being instructed to by the Project Manager.

The Project Manager's approval of the Program shall not alter the Contractor's obligations. The Contractor may revise the Program and submit it to the Project Manager again at any time. A revised Programshall show the effect of Variations and Compensation Events.

- Extension of the Intended Completion Date

The Project Manager shall extend the Intended Completion Date if a Compensation Event occurs or a Variation is issued which makes it impossible for Completion to be achieved by the Intended Completion Date without the Contractor taking steps to accelerate the remaining work, which would cause the Contractor to incur additional cost.

The Project Manager shall decide whether and by how much to extend the Intended Completion Date within 21 days of the Contractor asking the Project Manager for a decision upon the effect of a Compensation Event or Variation and submitting full supporting information. If the Contractor has failed to give early warning of a delay or has failed to cooperate in dealing with a delay, the delay by this failure shall not be considered in assessing the new Intended Completion Date.

- Acceleration

When the Procuring Entity wants the Contractor to ﬁnish before the Intended Completion Date, the Project Manager shall obtain priced proposals for achieving the necessary acceleration from the Contractor. If the Procuring Entity accepts these proposals, the Intended Completion Date shall be adjusted accordingly and conﬁrmed by both the Procuring Entity and the Contractor.

If the Contractor's priced proposals for an acceleration are accepted by the Procuring Entity, they are incorporated in the Contract Price and treated as a Variation.

- Delays Ordered by the Project Manager

The Project Manager may instruct the Contractor to delay the start or progress of any activity within the Works.

- Management Meetings

Either the Project Manager or the Contractor may require the other to attend a management meeting. The business of a management meeting shall be to review the plans for remaining work and to deal with matters raised in accordance with the early warning procedure.

The Project Manager shall record the business of management meetings and provide copies of the record to those attending the meeting and to the Procuring Entity. The responsibility of the parties for actions to be taken shall be decided by the Project Manager either at the management meeting or after the management meeting and stated in writing to all who attended the meeting.

- Early Warning

The Contractor shall warn the Project Manager at the earliest opportunity of speciﬁc likely future events or circumstances that may adversely affect the quality of the work, increase the Contract Price, or delay the execution of the Works. The Project Manager may require the Contractor to provide an estimate of the expected effect of the future event or circumstance on the Contract Price and Completion Date. The estimate shall be provided by the Contractor as soon as reasonably possible.

The Contractor shall cooperate with the Project Manager in making and considering proposals for how the effect of such an event or circumstance can be avoided or reduced by anyone involved in the work and in carrying out any resulting instruction of the Project Manager.

- Quality Control

  - Identifying Defects

The Project Manager shall check the Contractor's work and notify the Contractor of any Defects that are found. Such checking shall not affect the Contractor's responsibilities. The Project Manager may instruct the Contractor to search for a Defect and to uncover and test any work that the Project Manager considers may have a Defect.

  - Tests

If the Project Manager instructs the Contractor to carry out a test not speciﬁed in the Speciﬁcation to check whether any work has a Defect and the test shows that it does, the Contractor shall pay for the test and any samples. If there is no Defect, the test shall be a Compensation Event.

  - Correction of Defects

The Project Manager shall give notice to the Contractor of any Defects before the end of the Defects Liability Period, which begins at Completion, and is deﬁned in the SCC. The Defects Liability Period shall be extended for as long as Defects remain to be corrected.

Every time notice of a Defect is given, the Contractor shall correct the notiﬁed Defect within the length of time speciﬁed by the Project Manager's notice.

  - Uncorrected Defects

If the Contractor has not corrected a Defect within the time speciﬁed in the Project Manager's notice, the Project Manager shall assess the cost of having the Defect corrected, and the Contractor shall pay this amount.

- Cost Control

- Contract Price7

The Bill of Quantities shall contain priced items for the Works to be performed by the Contractor. The Bill of Quantities is used to calculate the Contract Price. The Contractor will be paid for the quantity of the work accomplished at the rate in the Bill of Quantities for each item.

- Changes in the Contract Price8

If the ﬁnal quantity of the work done differs from the quantity in the Bill of Quantities for the particular item by more than 25 percent, provided the change exceeds 1 percent of the Initial Contract Price, the Project Manager shall adjust the rate to allow for the change. The Project Manager shall not adjust rates from changes in quantities if thereby the Initial Contract Price is exceeded by more than 15 percent, except with the prior approval of the Procuring Entity.

If requested by the Project Manager, the Contractor shall provide the Project Manager with a detailed cost breakdown of any rate in the Bill of Quantities.

- Variations

All Variationsshall beincluded in updated Programs9 produced bythe Contractor.

The Contractor shall provide the Project Manager with a quotation for carrying out the Variation when requested to do so by the Project Manager. The Project Manager shall assess the quotation, which shall be given within seven (7) days of the request or within any longer period stated by the Project Manager and before the Variation is ordered.

If the Contractor's quotation is unreasonable, the Project Manager may order the Variation and make a change to the Contract Price, which shall be based on the Project Manager's own forecast of the effects of the Variation on the Contractor's costs.

If the Project Manager decides that the urgency of varying the work would prevent a quotation being given and considered without delaying the work, no quotation shall be given and the Variation shall be treated as a Compensation Event.

The Contractor shall not be entitled to additional payment for costs that could have been avoided by giving early warning.

If the work in the Variation corresponds to an item description in the Bill of Quantities and if, in the opinion of the Project Manager, the quantity of work above the limit stated in Sub-Clause 39.1 or the timing of its execution do not cause the cost per unit of quantity to change, the rate in the Bill of Quantities shall be used to calculate the value of the Variation. If the cost per unit of quantity changes, or if the nature or timing of the work in the Variation does not correspond with items in the Bill of Quantities, the quotation by the Contractor shall be in the form of new rates for the relevant items of work10.

Value Engineering: The Contractor may prepare, at its own cost, a value engineering proposal at any time during the performance of the contract. The value engineering proposal shall, at a minimum, include the following;

  - the proposed change(s), and a description of the difference to the existing contract requirements;

  - a full cost/beneﬁt analysis of the proposed change(s) including a description and estimate of costs (including life cycle costs) the Procuring Entity may incur in implementing the value engineering proposal; and

  - a description of any effect(s) of the change on performance/functionality.

The Procuring Entity may accept the value engineering proposal if the proposal demonstrates beneﬁts that:

- accelerate the contract completion period; or

- reduce the Contract Price or the life cycle costs to the Procuring Entity; or

- improve the quality, efﬁciency, safety or sustainability of the Facilities; or

- yield any other beneﬁts to the Procuring Entity, without compromising the functionality of the Works.

If the value engineering proposal is approved by the Procuring Entity and results in:

- a reduction of the Contract Price; the amount to be paid to the Contractor shall be the percentage speciﬁed in the SCC of the reduction in the Contract Price; or

- an increase in the Contract Price; but results in a reduction in life cycle costs due to any beneﬁt described in

  - to (d) above, the amount to be paid to the Contractor shall be the full increase in the Contract Price.

- Cash Flow Forecasts

When the Program11, is updated, the Contractor shall provide the Project Manager with an updated cash ﬂow forecast. The cash ﬂow forecast shall include different currencies, as deﬁned in the Contract, converted as necessary using the Contract exchange rates.

- Payment Certiﬁcates

The Contractor shall submit to the Project Manager monthly statements of the estimated value of the work executed less the cumulative amount certiﬁed previously.

The Project Manager shall check the Contractor's monthly statement and certify the amount to be paid to the

The value of work executed shall be determined by the Project Manager.

The value of work executed shall comprise the value of the quantities of work in the Bill of Quantities that have been completed12.

The value of work executed shall include the valuation of Variations and Compensation Events.

The Project Manager may exclude any item certiﬁed in a previous certiﬁcate or reduce the proportion of any item previously certiﬁed in any certiﬁcate in the light of later information.

Where the contract price is different from the corrected tender price, in order to ensure the contractor is not paid less or more relative to the contract price (which would be the tender price), payment valuation certiﬁcates and variation orders on omissions and additions valued based on rates in the Bill of Quantities or schedule of rates in the Tender, will be adjusted by a plus or minus percentage. The percentage already worked out during tender evaluation is worked out as follows: (corrected tender price – tender price)/tender price X 100.

- Payments

Payments shall be adjusted for deductions for advance payments and retention. The Procuring Entity shall pay the Contractor the amounts certiﬁed by the Project Manager within 30 days of the date of each certiﬁcate. If the Procuring Entity makes a late payment, the Contractor shall be paid interest on the late payment in the next payment. Interest shall be calculated from the date by which the payment should have been made up to the date when the late payment is made at the prevailing rate of interest for commercial borrowing for each of the currencies in which payments are made.

If an amount certiﬁed is increased in a later certiﬁcate or as a result of an award by the Adjudicator or an Arbitrator, the Contractor shall be paid interest upon the delayed payment as set out in this clause. Interest shall be calculated from the date upon which the increased amount would have been certiﬁed in the absence of dispute.

Unless otherwise stated, all payments and deductions shall be paid or charged in the proportions of currencies comprising the Contract Price.

Items of the Works for which no rate or price has been entered in shall not be paid for by the Procuring Entity and	shall be deemed covered by other rates and prices in the Contract.

- Compensation Events

The following shall be Compensation Events:

  - The Procuring Entity does not give access to a part of the Site by the Site Possession Date pursuant to GCC Sub-Clause 20.1.

  - The Procuring Entity modiﬁes the Schedule of Other Contractors in a way that affects the work of the Contractor under the Contract.

  - The Project Manager orders a delay or does not issue Drawings, Speciﬁcations, or instructions required for execution of the Works on time.

  - The Project Manager instructs the Contractor to uncover or to carry out additional tests upon work, which is then found to have no Defects.

  - The Project Manager unreasonably does not approve a subcontract to be let.

  - Ground conditions are substantially more adverse than could reasonably have been assumed before issuance of the Letter of Acceptance from the information issued to bidders (including the Site Investigation Reports), from information available publicly and from a visual inspection of the Site.

  - The Project Manager gives an instruction for dealing with an unforeseen condition, caused by the Procuring Entity, or additional work required for safety or other reasons.

  - Other contractors, public authorities, utilities, or the Procuring Entity does not work within the dates and other constraints stated in the Contract, and they cause delay or extra cost to the Contractor.

  - The advance payment is delayed.

  - The effects on the Contractor of any of the Procuring Entity's Risks.

If a Compensation Event would cause additional cost or would prevent the work being completed before the Intended Completion Date, the Contract Price shall be increased and/or the Intended Completion Date shall be extended. The Project Manager shall decide whether and by how much the Contract Price shall be increased and whether and by how much the Intended Completion Date shall be extended.

As soon as information demonstrating the effect of each Compensation Event upon the Contractor's forecast cost has been provided by the Contractor, it shall be assessed by the Project Manager, and the Contract Price shall be adjusted accordingly. If the Contractor's forecast is deemed unreasonable, the Project Manager shall adjust the Contract Price based on the Project Manager's own forecast. The Project Manager shall assume that the Contractor shall react competently and promptly to the event.

The Contractor shall not be entitled to compensation to the extent that the Procuring Entity's interests are adversely affected by the Contractor's not having given early warning or not having cooperated with the Project Manager.

- Tax

The Project Manager shall adjust the Contract Price if taxes, duties, and other levies are changed between the date 30 days before the submission of bids for the Contract and the date of the last Completion certiﬁcate. The adjustment shall be the change in the amount of tax payable by the Contractor, provided such changes are not already reﬂected in the Contract Price or are a result of GCC Clause 44.

- Currency y of Payment

All payments under the contract shall be made in Kenya Shillings

- Price Adjustment

Prices shall be adjusted for ﬂuctuations in the cost of inputs only if provided for in the SCC. If so provided, the amounts certiﬁed in each payment certiﬁcate, before deducting for Advance Payment, shall be adjusted by applying the respective price adjustment factor to the payment amounts due in each currency. A separate formula of the type speciﬁed below applies:

P = A + B Im/Io

where: P is the adjustment factor for the portion of the Contract Price payable.

A  and  B  are  coefﬁcients13  speciﬁed  in  the  SCC,  representing  the  non-adjustable  and  adjustable portions, respectively, of the Contract Price payable and Im is the index prevailing at the end of the month being invoiced and IOC is the index prevailing 30 days before Bid opening for inputs payable.

If the value of the index is changed after it has been used in a calculation, the calculation shall be corrected and an adjustment made in the next payment certiﬁcate. The index value shall be deemed to take account of all changes in cost due to ﬂuctuations in costs.

- Retention

The Procuring Entity shall retain from each payment due to the Contractor the proportion stated in the SCC until Completion of the whole of the Works.

Upon the issue of a Certiﬁcate of Completion of the Works by the Project Manager, in accordance with GCC 53.1, half the total amount retained shall be repaid to the Contractor and half when the Defects Liability Period has passed and the Project Manager has certiﬁed that all Defects notiﬁed by the Project Manager to the Contractor before the end of this period have been corrected. The Contractor may substitute retention money with an “on demand” Bank guarantee.

- Liquidated Damages

The Contractor shall pay liquidated damages to the Procuring Entity at the rate per day stated in the SCC for each day that the Completion Date is later than the Intended Completion Date. The total amount of liquidated damages shall not exceed the amount deﬁned in the SCC. The Procuring Entity may deduct liquidated damages

from payments due to the Contractor. Payment of liquidated damages shall not affect the Contractor's liabilities.

If the Intended Completion Date is extended after liquidated damages have been paid, the Project Manager shall correct any overpayment of liquidated damages by the Contractor by adjusting the next payment certiﬁcate. The Contractor shall be paid interest on the overpayment, calculated from the date of payment to the date of repayment, at the rates speciﬁed in GCC Sub-Clause 41.1.

- Bonus

The Contractor shall be paid a Bonus calculated at the rate per calendar day stated in the SCC for each day (less any days for which the Contractor is paid for acceleration) that the Completion is earlier than the Intended Completion Date. The Project Manager shall certify that the Works are complete, although they may not be due to be complete.

- Advance Payment

The Procuring Entity shall make advance payment to the Contractor of the amounts stated in the SCC by the date stated in the SCC, against provision by the Contractor of an Unconditional Bank Guarantee in a form and by a bank acceptable to the Procuring Entity in amounts and currencies equal to the advance payment. The Guarantee shall remain effective until the advance payment has been repaid, but the amount of the Guarantee shall be progressively reduced by the amounts repaid by the Contractor. Interest shall not be charged on the advance payment.

The Contractor is to use the advance payment only to pay for Equipment, Plant, Materials, and mobilization expenses required speciﬁcally for execution of the Contract. The Contractor shall demonstrate that advance payment has been used in this way by supplying copies of invoices or other documents to the Project Manager.

The advance payment shall be repaid by deducting proportionate amounts from payments otherwise due to the Contractor, following the schedule of completed percentages of the Works on a payment basis. No account shall be taken of the advance payment or its repayment in assessing valuations of work done, Variations, priceadjustments, Compensation Evenc ts, Boc   nuses, or Liquidated Damages.

- Securities

The Performance Security shall be provided to the Procuring Entity no later than the date speciﬁed in the Letter of Acceptance and shall be issued in an amount speciﬁed in the SCC, by a bank or surety acceptable to the Procuring Entity, and denominated in the types and proportions of the currencies in which the Contract Price is payable. The Performance Security shall be valid until a date 28 day from the date of issue of the Certiﬁcate of Completion in the case of a Bank Guarantee, and until one year from the date of issue of the Completion Certiﬁcate in the case of a Performance Bond.

- Dayworks

If applicable, the Dayworks rates in the Contractor's Bid shall be used only when the Project Manager has given written instructions in advance for additional work to be paid for in that way.

All work to be paid for as Dayworks shall be recorded by the Contractor on forms approved by the Project Manager. Each completed form shall be veriﬁed and signed by the Project Manager within two days of the work being done.

The Contractor shall be paid for Dayworks subject to obtaining signed Dayworks forms.

13The sum of the two coefﬁcients A and B should be 1 (one) in the formula for each currency. Normally, both coefﬁcients shall be the same in the formulae for all currencies, since coefﬁcient A, for the non-adjustable portion of the payments, is a very approximate ﬁgure (usually 0.15) to take account of ﬁxed cost elements or othernon-adjustable components. Thesum of the adjustmentsfor eachcurrency areadded to the Contract Price.

- Cost of Repairs

Loss or damage to the Works or Materials to be incorporated in the Works between the Start Date and the end of the Defects Correction periods shall be remedied by the Contractor at the Contractor's cost if the loss or damage arises from the Contractor's acts or omissions.

- Finishing the Contract

- Completion

The Contractor shall request the Project Manager to issue a Certiﬁcate of Completion of the Works, and the Project Manager shall do so upon deciding that the whole of the Works is completed.

- Taking Over

The Procuring Entity shall take over the Site and the Works within seven days of the Project Manager's issuing a certiﬁcate of Completion.

- Final Account

The Contractor shall supply the Project Manager with a detailed account of the total amount that the Contractor considers payable under the Contract before the end of the Defects Liability Period. The Project Manager shall issue a Defects Liability Certiﬁcate and certify any ﬁnal payment that is due to the Contractor within 56 days of receiving the Contractor's account if it is correct and complete. If it is not, the Project Manager shall issue within 56 days a schedule that states the scope of the corrections or additions that are necessary. If the Final Account is still unsatisfactory after it has been resubmitted, the Project Manager shall decide on the amount payable to the Contractor and issue a payment certiﬁcate.

- Operating and Maintenance Manuals

If “as built” Drawings and/or operating and maintenance manuals are required, the Contractor shall supply them by the dates stated in the SCC.

If the Contractor does not supplythe Drawings and/or manuals bythe dates stated in the SCC pursuant to GCC Sub- Clause 56.1, or they do not receive the Project Manager's approval, the Project Manager shall withholdthe amount stated in the SCC from payments due to the Contractor.

- Termination

The Procuring Entity or the Contractor may terminate the Contract if the other party causes a fundamental breach of the Contract.

Fundamental breaches of Contract shall include, but shall not be limited to, the following:

  - the Contractor stops work for 30 days when no stoppage of work is shown on the current Program and the stoppage has not been authorized by the Project Manager;

F.

- the Project Manager instructs the Contractor to delay the progress of the Works, and the instruction is not withdrawn within 30 days;

- the Procuring Entity or the Contractor is made bankrupt or goes into liquidation other than for a reconstruction oramalgamation;

- a payment certiﬁed by the Project Manager is not paid by the Procuring Entity to the Contractor within 84 days of the date of the Project Manager's certiﬁcate;

- the Project Manager gives Notice that failure to correct a particular Defect is a fundamental breach of Contract and the Contractor fails to correct it within a reasonable period of time determined by the Project Manager;

- the Contractor does not maintain a Security, which is required;

- the Contractor has delayed the completion of the Works by the number of days for which the maximum amount of liquidated damages can be paid, as deﬁned in the SCC; or

- if the Contractor, in the judgment of the Procuring Entity has engaged in Fraud and Corruption, as deﬁned in paragraph 2.2 a of the Appendix A to the GCC, in competing for or in executing the Contract, then the Procuring Entity may, after giving fourteen (14) days written notice to the Contractor, terminate the Contract and expel him from the Site.

Notwithstanding the above, the Procuring Entity may terminate the Contract for convenience.

If the Contract is terminated, the Contractor shall stop work immediately, make the Site safe and secure, and leave the Site as soon as reasonably possible.

When either party to the Contract gives notice of a breach of Contract to the Project Manager for a cause other than those listed under GCC Sub-Clause 56.2 above, the Project Manager shall decide whether the breach is fundamental or not.

- Payment upon Termination

If the Contract is terminated because of a fundamental breach of Contract by the Contractor, the Project Manager shall issue a certiﬁcate for the value of the work done and Materials ordered less advance payments received up to the date of the issue of the certiﬁcate and less the percentage to apply to the value of the work not completed, as speciﬁed in the SCC. Additional Liquidated Damages shall not apply. If the total amount due to the Procuring Entity exceeds any payment due to the Contractor, the difference shall be a debt payable to the Procuring Entity.

If the Contract is terminated for the Procuring Entity's convenience or because of a fundamental breach of Contract by the Procuring Entity, the Project Manager shall issue a certiﬁcate for the value of the work done, Materials ordered, the reasonable cost of removal of Equipment, repatriation of the Contractor's personnel employed solely on the Works, and the Contractor's costs of protecting and securing the Works, and less advance payments received up to the date of the certiﬁcate.

- Property

All Materials on the Site, Plant, Equipment, Temporary Works, and Works shall be deemed to be the property of the Procuring Entity if the Contract is terminated because of the Contractor's default.

- Release from Performance

If the Contract is frustrated by the outbreak of war or by any other event entirely outside the control of either the Procuring Entity or the Contractor, the Project Manager shall certify that the Contract has been frustrated. The Contractor shall make the Site safe and stop work as quickly as possible after receiving this certiﬁcate and shall be paid for all work carried out before receiving it and for any work carried out afterwards to which a commitment was made.

FORM No 1: NOTIFICATION OF INTENTION TO AWARD

This Notiﬁcation of Intention to Award shall be sent to each Tenderer that submitted a Tender. Send this Notiﬁcation to the Tenderer's Authorized Representative named in the Tender Information Form on the format below.

FORMAT

- For the attention of Tenderer's Authorized Representative

  - Name: [insert Authorized Representative's name]

  - Address: [insert Authorized Representative's Address]

  - Telephone: [insert Authorized Representative's telephone/fax numbers]

  - Email Address: [insert Authorized Representative's email address]

[IMPORTANT: insert the date that this Notiﬁcation is transmitted to Tenderers. The Notiﬁcation must be sent to all Tenderers simultaneously. This means on the same date and as close to the same time as possible.]

- Date of transmission: [email] on [date] (local time)

This Notiﬁcation is sent by (Name and designation)

- Notiﬁcation of Intention to Award

  - Procuring Entity: [insert the name of the Procuring Entity]

  - Project: [insert name of project]

  - Contract title: [insert the name of the contract]

  - Country: [insert country where ITT is issued]

  - ITT No: [insert ITT reference number from Procurement Plan]

This Notiﬁcation of Intention to Award (Notiﬁcation) notiﬁes you of our decision to award the above contract. The transmission of this Notiﬁcation begins the Standstill Period. During the Standstill Period, you may:

- Request a debrieﬁng in relation to the evaluation of your tender

Submit a Procurement-related Complaint in relation to the decision to award the contract.

- The successful tenderer

  - Name of successful Tender:

  - Address of the successful Tender :

  - Contract	price	of	the	successful	Tender	Kenya	Shillings

(in	words)

- Other Tenderers

Names of all Tenderers that submitted a Tender. If the Tender's price was evaluated include the evaluated price as well as the Tender price as read out. For Tenders not evaluated, give one main reason the Tender was unsuccessful.

| S/No. | Name of Tenderer | Tender Price as read out | Evaluated Tender price (Note a | One reason why not evaluated |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

(Note a) State NE if not evaluated

- How to request a debrieﬁng

- DEADLINE: The deadline to request a debrieﬁng expires at midnight on [insert date] (local time).

- You may request a debrieﬁng in relation to the results of the evaluation of your Tender. If you decide to request a debrieﬁng your written request must be made within three (5) Business Days of receipt of this Notiﬁcation of Intention toAward.

- Provide the contract name, reference number, name of the Tenderer, contact details; and address the request for debrieﬁng as follows:

  - Attention: [insert fullname ofperson, if applicable]

  - Title/position: [insert title/position]

- Agency: [insertname of Procuring Entity]

- Email address: [insertemailaddress]

- If your request for a debrieﬁng is received within the 3 Days deadline, we will provide the debrieﬁng within ﬁve (3) Business Days of receipt of your request. If we are unable to provide the debrieﬁng within this period, the Standstill Period shall be extended by ﬁve (3) Days after the date that the debrieﬁng is provided. If thishappens, we will notify you and conﬁrmthe date that the extended Standstill Period will end.

- The debrieﬁng may be in writing, by phone, video conference call or in person. We shall promptly advise you in writing how the debrieﬁng will take place and conﬁrmthe date and time.

- If the deadline to request a debrieﬁng has expired, you may still request a debrieﬁng. In this case, wewill provide the debrieﬁng as soon as practicable, and normally no later than ﬁfteen (15) Days from the date of publication of the ContractAward Notice.

- How to make a complaint

- Period: Procurement-related Complaint challenging the decision to award shall be submitted by midnight, [insert date] (local time).

- Provide the contract name, reference number, name of the Tenderer, contact details; and address the Procurement-related Complaint as follows:

  - Attention: [insert fullname ofperson, if applicable]

  - Title/position: [insert title/position]

  - Agency: [insertname of Procuring Entity]

  - Email address: [insertemailaddress]

- At this point in the procurement process, you may submit a Procurement-related Complaint challenging the decision to award the contract. You do not need to have requested, or received, a debrieﬁng before making this complaint. Your complaint must be submitted within the Standstill Period and received by us before the Standstill Period ends.

- Further information: For more information refer to the Public Procurement and Disposals Act 2015 and its Regulations available fromthe Website info@ppra.go.ke or complaints@ppra.go.ke.

You should read these documents before preparing and submitting your complaint.

- There are four essential requirements:

  - You must be an 'interested party'. In this case, that means a Tenderer who submitted a Tender in this tendering process, and is the recipient of a Notiﬁcation of Intention toAward.

  - The complaint can onlychallenge the decision to award the contract.

  - Youmust submit the complaint within the period stated above.

  - You must include, in your complaint, all of the information required to support your complaint.

- Standstill Period

  - DEADLINE: The Standstill Period is due to end at midnight on [insert date] (local time).

  - The Standstill Period lasts ten (14) Days after the date of transmission of this Notiﬁcation of Intention to Award.

  - The Standstill Period maybe extended as stated in paragraph Section 5 (d) above.

If you have any questions regarding this Notiﬁcation please do not hesitate to contact us. On behalf of the Procuring Entity:

Signature:	Name:

Title/position:	Telephone:	Email:

[letterhead paperofthe Procuring Entity] [date]

To: [name and address of the Contractor]

This is to notify you that your Tender dated [date] for execution of the [name of the Contract and identiﬁcation number, as given in the Contract Data] for the Accepted Contract Amount [amount innumbers andwords] [name of currency], as corrected and modiﬁed in accordance with the Instructions to Tenderers, is hereby accepted by	(name of

Procuring Entity).

You are requested to furnish the Performance Security within 30 days in accordance with the Conditions of Contract, using, for that purpose, one of the Performance Security Forms included in Section VIII, Contract Forms, of the Tender Document.

Authorized Signature:.................................................................................................................

Name and Title of Signatory:........................................................................................................

Name of Procuring Entity.............................................................................................................

Attachment: Contract Agreement..............................................................................................

FORM NO. 3 : CONTRACT AGREEMENT

AGREEMENT BETWEEN

KENYA RURAL ROADS AUTHORITY

&

Contractor…………………………………………………………………………..

CONTRACT No. …………………………………………………………..…………

This Contract is made this……………………….day of……………………….20………

between Kenya Rural Roads Authority, represented by the Director General of P.O. Box

48151 – 00100, NAIROBI Telephone 020 – 801346 (Hereinafter called the Employer)

acting through …his representative The Regional Director – KEIYO MARAKWET REGION, of P.O. Box 213-30700, ITEN of the one part and

…………………………………………………………………. (Hereinafter

referred to as “the Contractor”) of the other part).

Whereas the Employer is desirous of ………………………………………..…………….

Road Number: …………………………………………………………………………..

Region: …………………………………………………………………………………

Constituency: ………………………………………………………………………….

From Km/Length of road ……………..……………………….............................

and has accepted the Offer of the Contractor to execute road maintenance Works as defined hereinafter, and Whereas the Contractor represented to the Employer that he has the required capacity and ability, and has agreed to execute the Works on the Terms and Conditions set forth in this Contract for the Tender Sum of

Kshs ………………… (Kenya Shillings, …………………………………………………………………………………………………………… Only) to execute the Improvement/Rehabilitation Works on the road sections as defined

NOW THIS AGREEMENT WITNESSETH AS FOLLOWS:

In this Agreement words and expressions shall have the same meanings as are respectively assigned to them in the Conditions of Contract.

The following documents form part of this Agreement, viz:

- Letter of Acceptance;

- Form of Tender

- Appendix to the Form of  Agreement

- Conditions of Contract; Parts I and II

- Specifications;

- Drawings;

- Priced Bill of Quantities;

- Other Documents/Materials/Conditions agreed and documented.

Article 1:	GENERAL PROVISIONS

1.1      The documents forming the Contract are to be taken as mutually explanatory.  If ambiguity or discrepancy is found in the documents the Employer shall issue necessary instructions to the Contractor.

  - The language of the Contract shall be English.  The law of the Contract is the Republic of Kenya.

  - This contract has no defects liability period and therefore any retention.

Article 2:	OBLIGATIONS OF THE EMPLOYER

  - The Employer shall provide the Site and right of access thereto at the date of Commencement.

  - The Employer shall ensure that the Contractor’s claims for payment are settled timeously.

  - The Employer shall inspect the Contractor’s work regularly.

  - No approval or consent or absence of comment by the Employer shall affect the Contractor’s obligations.

Article 3:	EMPLOYER’S REPRESENTATIVE

  - The Director General of KeRRA will have overall responsibility for the Contract, and will be represented by the Keiyo Marakwet Regional Director for the day-to-day management of the Contract.

  - The Regional Director shall appoint and provide a Road Supervisor (CRO/SARO) by the name of ……...............................................................who shall from time to time assign work to the Contractor.  The Engineer shall notify the Contractor in writing of any change in the Road Supervisor.

3.3.	The Regional Director shall be entitled to vary the Work plan according to circumstances and to issue instructions to the Contractor accordingly.

Article 4:	CONTRACTOR’S OBLIGATIONS

  - The Contractor shall carry out the Works described in Annex 1and the Work Plan Annex 3 properly and in accordance with the Contract.  The Contractor shall provide all supervision, labour and tools which may be required.

  - The Contractor shall submit to the Employer for consent the name and particulars of the person authorized to receive instructions on behalf of the Contractor.

  - The Contractor shall comply with all instructions given by the Employer in respect of the Works.

  - The Contractor shall not subcontract any part of the Works without the consent of the Employer.

  - The Contractor shall, in the implementation of this contract, use labour based methods whenever possible.

  - The Contractor shall recruit labour for the Works from the local communities and shall take particular note of the Government policy on the recruitment of women and youth.  The target for each group shall be 30% of the labour force, which the Contractor shall take all possible measures to attain.

Article 5:	DESIGN BY CONTRACTOR

  - The Contractor shall not be responsible for any Works design.

Article 6:	EMPLOYER’S RISKS

6.1	Any failure by the Employer and any delay or disruption caused by any Variation are the relevant Employer’s Risks.

Article 7:	TIME FOR COMPLETION

  - The Contractor shall commence the Works on the Commencement Date….................... and shall proceed expeditiously and without delay and shall complete the Works within the Time for Completion, given as…….…………… (Weeks/Months).

7.2	The Contractor shall be entitled to an extension of the Time for Completion where delay is caused by an action, or failure of action of the Employer and/or any other unforeseen circumstance. All such extension of time must be requested by the contractor timeously and approved by the Employer.

Article 8:	TAKING OVER

8.1.	The Contractor may notify the Employer when the Works are considered to be complete.

  - The Employer shall notify the Contractor when he considers that the Contractor has completed the Works and stating the date accordingly.  Alternatively the Employer may notify the Contractor that the Works, although not fully complete, are ready for taking over, stating the date accordingly.  The Contractor shall promptly complete any outstanding work.

Article 9:	REMEDYING DEFECTS

9.1.	The Employer may, within one month after the Completion date in 8.2, notify the Contractor of any defects.  The Contractor shall remedy at no cost to the Employer any defects due to the Contractor’s workmanship not being in accordance with the Contract.

Article 10	VARIATIONS AND CLAIMS

  - The Employer may instruct Variations.

  - No variations for the contract sum.

Article 11:	CONTRACT PRICE AND PAYMENT

  - At not less than monthly intervals, the Contractor shall measure the Works in accordance with the Bill of Quantities and shall submit a statement to the Road Supervisor.  No Payment shall be made for Mobilisation and Establishment of the Site. The Road Supervisor shall certify on the statement to the Engineer that the Works have been completed satisfactorily.

  - The Employer shall pay the Contractor, upon the Engineer’s approval of the Contractor’s statement, within 30 days of the date of the certification of the statement.  If the Employer disagrees with any part of the Contractor’s statement, he shall specify his reasons for disagreement when making payment.  The Employer shall not be bound by any sum previously considered by him to be due to the Contractor.

Article 12:	DEFAULT

  - The Employer shall give notice to the Contractor where he considers the Contractor to be in default of the Contract.  If the Contractor fails to remedy the default within a stated period the Employer may give notice of the termination of the Contract.

  - After termination the Contractor shall be entitled to payment of any unpaid balance of the value of the Works executed.

Article 13:	RISK AND RESPONSIBILITY

  - The Contractor shall take responsibility for the care of the Works from the Commencement Date to the date of Take Over.

Article 14:	INSURANCES

  - The Contractor, prior to commencing the Works, shall effect Third Party and Workers Compensation Insurances and provide evidence to the Employer that the required policies are in force.

Article 15:	RESOLUTION OF DISPUTES

  - The Parties shall use their best efforts to settle amicably all disputes arising out of or in connection with the Contract.

  - Any dispute not resolved under 15.1 shall be referred to a single Arbitrator nominated by ……KeRRA…. in whose Area the Contract is located in accordance with, and subject to the provision of the Arbitration Act of Kenya or any Statutory Modification thereof.

Article 16	CONDITIONS OF CONTRACT

  - It is understood that these Conditions of Contract follow the FIDIC Short Form of Contract which shall be referred to for any necessary clarification

  - It is hereby understood that the Annexes to the Contract shall be read together with the Contract and shall form an integral part of it.

In consideration of the payment to be made by the Employer to the Contractor, the Contractor covenants with the Employer to execute and complete the Works in conformity with the provisions of the Contract.

The Employer covenants to pay the Contractor in consideration of the execution and completion of the Works, the Contract Price or such other sum as may become payable under the Contract at the times and in the manner prescribed by the Contract.

IN WITNESS WHEREOF the parties have caused this Agreement to be executed the day and year first before written at KEIYO MARAKWET REGION, Kenya

Signed and dated for and on behalf of the (Name of the Employer’s Representative)

Name: ……………………………………… Signature: ………………………………

Designation: Regional Director, KeRRA Keiyo Marakwet

Date: …………………………………

In the Presence of

Name ……………………………………… Signature………………………………

Designation: Regional Senior Supply Chain Management Officer

Date: ………………………………

Signed for and on behalf of the Contractor (Name of the Contractor’s Representative and Stamp of Contractor)

Name ……………………………………….……Signature…………………………

Designation ………………………………….  Date…………………………………

In the Presence of

Name …………………………………….……Signature…………………………

Company name and address …………………………………………..............................

Date…………………………………

FORM NO. 4 : PERFORMANCE SECURITY

[Option 1 - Unconditional Demand Bank Guarantee]

[Guarantor letterhead]

Beneﬁciary:	[insert name and Address of Procuring Entity] Date:

[Insert date of issue]

Guarantor: [Insert name and address of place of issue, unless indicated in the letterhead]

- We have been informed that			(hereinafter called "the Contractor") has entered into Contract No.		dated		with (name of Procuring Entity)	(the Procuring Entity as the Beneﬁciary), for the execution of

(hereinafter called "the Contract").

- Furthermore, we understand that, according to the conditions of the Contract, a performance guarantee is required.

- At the request of the Contractor, we as Guarantor, hereby irrevocably undertake to pay the Beneﬁciary any sum or sums not exceeding in total an amount of	(in words ),1 such sum being payable in the types and proportions of currencies in which the Contract Price is payable, upon receipt by us of the Beneﬁciary's complying demand supported by the Beneﬁciary's statement, whether in the demand itself or in a separate signed document accompanying or identifying the demand, stating that the Applicant is in breach of its obligation(s) under the Contract, without the Beneﬁciary needing to prove or to show grounds for your demand or the sum speciﬁed therein.

- This guarantee shall expire, no later than the …. Day of …………, 2.	2, and any demand for payment under it

must be received by us at the ofﬁce indicated above on or before that date.

- The Guarantor agrees to a one-time extension of this guarantee for a period not to exceed [six months] [oneyear], in response to the Beneﬁciary's written request for such extension, such request to be presented to the Guarantor before the expiry of the guarantee.”

[Name of Authorized Ofﬁcial, signature(s) and seals/stamps].

Note: All italicized text (including footnotes) is for use in preparing this form and shall be deleted from the ﬁnal product.

1The Guarantor shall insert an amount representing the percentage of the Accepted Contract Amount speciﬁed in the Letter of Acceptance, less provisional sums, if any, and denominated either in the currency of the Contract or a freely convertible currency acceptable to the Beneﬁciary. 2Insert the date twenty-eight days after the expected completion date as described in GC Clause 11.9. The Procuring Entity should note that in the event of an extension of this date for completion of the Contract, the Procuring Entity would need to request an extension of this guarantee from the Guarantor. Such request must be in writing and must be made prior to the expiration date established in the guarantee.

[Option 2– Performance Bond]

[Note: Procuring Entities are advised to use Performance Security – Unconditional Demand Bank Guarantee instead of Performance Bond due to difﬁculties involved in calling Bondholder to action]

[Guarantor letterhead or SWIFT identiﬁer code]

Beneﬁciary:	[insert name and Address of Procuring Entity] Date:

[Insert date of issue].

PERFORMANCE BOND No.:

Guarantor: [Insert name and address of place of issue, unless indicated in the letterhead]

- By this Bond	as Principal (hereinafter called “the Contractor”) and		] as Surety (hereinafter called		“the Surety”), are  held and  ﬁrmly bound unto			]			as Obligee (hereinafter called “the Procuring Entity”) in the amount of			for the payment of which sum well and truly to be made in the types and proportions of currencies in which the Contract Price is payable, the Contractor and the Surety bind themselves, their heirs, executors, administrators, successors and assigns, jointly and severally, ﬁrmly by these presents.

- WHEREAS the Contractor has entered into a written Agreement with the Procuring Entity dated the

day of	, 20 , for	in accordance with the documents, plans, speciﬁcations, and amendments thereto, which to the extent herein provided for, are by reference made part hereof and are hereinafter referred to as the Contract.

- NOW,THEREFORE, the Conditionofthis Obligation is suchthat, if the Contractorshallpromptly and faithfully perform the said Contract (including any amendments thereto), then this obligation shall be null and void; otherwise, it shall remain in full force and effect. Whenever the Contractor shall be, and declared by the Procuring Entity to be, in default under the Contract, the Procuring Entity having performed the Procuring Entity's obligations thereunder, the Surety may promptly remedy the default, or shall promptly:

  - complete the Contract in accordance with its terms and conditions; or

  - obtain a tender or tenders from qualiﬁed tenderers for submission to the Procuring Entity for completing the Contract in accordance with its terms and conditions, and upon determination by the Procuring Entity and the Surety of the lowest responsive Tenderers, arrange for a Contract between such Tenderer, and Procuring Entity and make available as work progresses (even though there should be a default or a succession of defaults under the Contract or Contracts of completion arranged under this paragraph) sufﬁcient funds to pay the cost of completion less the Balance of the Contract Price; but not exceeding, including other costs and damages for which the Surety maybeliablehereunder, theamountsetforth in theﬁrst paragraphhereof. The term “Balance of the Contract Price,” as used in this paragraph, shall mean the total amount payable by Procuring Entity to Contractor under the Contract, less the amount properly paid by Procuring Entity to Contractor; or

  - pay the Procuring Entity the amount required by Procuring Entity to complete the Contract in accordance with its terms and conditions up to a total not exceeding the amount of this Bond.

- The Suretyshall not be liable for a greater sum than the speciﬁed penaltyof this Bond.

- Any suit under this Bond must be instituted before the expiration of one year from the date of the issuing of the Taking- Over Certiﬁcate. No right of action shall accrue on this Bond to or for the use of any person or corporation other than the Procuring Entity named herein or the heirs, executors, administrators, successors, and assigns of the Procuring Entity.

- In testimony whereof, the Contractor has hereunto set his hand and afﬁxed his seal, and the Surety has caused these presents to be sealed with his corporate seal duly attested by the signature of his legal representative, this day	of

20	.

SIGNED ON	on behalfof By	in the capacity of

In the presence of

SIGNED ON…………..…………..………….. on behalf of By	in the capacity of

In the presence of

Beneﬁciary:	[Insert name and Address of Procuring Entity]

Date:	[Insert date of issue]

ADVANCE PAYMENT GUARANTEE No.:	[Insert guarantee reference number] Guarantor:

[Insert name and address of place of issue, unless indicated in the letterhead]

- We have been informed that		(hereinafter called “the Contractor”) has entered into Contract No.	dated	with the Beneﬁciary, for the execution of  	 (hereinafter called "the Contract").

- Furthermore, we understand that, according to the conditions of the Contract, an advance payment in the sum

(in words)is to be made against an advance payment guarantee.

- At the request of the Contractor, we as Guarantor, hereby irrevocably undertake to pay the Beneﬁciary any sum or sums not exceeding in total an amount of	(in words	)1 upon receipt by us of the Beneﬁciary's complying demand supported by the Beneﬁciary's statement, whether in the demand itself or in a separate signed document accompanying or identifying the demand, stating either that the Applicant:

  - has used the advance payment for purposes other than the costs of mobilization in respect of the Works; or

  - has failed to repay the advance payment in accordance with the Contract conditions, specifying the amount which the Applicant has failed to repay.

- A demand under this guarantee may be presented as from the presentation to the Guarantor of a certiﬁcate from the Beneﬁciary's bank stating that the advance payment referred to above has been credited to the Contractor on its account number	at	.

- The maximum amount of this guarantee shall be progressively reduced by the amount of the advance payment repaid by the Contractor as speciﬁed in copies of interim statements or payment certiﬁcates which shall be presented to us. This guarantee shall expire, at the latest, upon our receipt of a copy of the interim payment certiﬁcate indicating that ninety (90) percent of the Accepted Contract Amount, less provisional sums, has been certiﬁed for payment, or on the            day of 	, 2 ,2   whichever is earlier.  Consequently, naydemand for payment under this guarantee must be received by us at this ofﬁce on or before that date.

- The Guarantor agrees to a one-time extension of this guarantee for a period not to exceed [sixmonths][one year], in response to the Beneﬁciary's written request for such extension, such request to be presented to the Guarantor before the expiry of the guarantee.

[Name of Authorized Ofﬁcial, signature(s) and seals/stamps]

Note: All italicized text (including footnotes) is for use in preparing this form and shall be deleted from the ﬁnal product.

1The Guarantor shall insert an amount representing the amount of the advance payment and denominated either in the currency of the advance payment as speciﬁed in the Contract.

2Insert the expected expiration date of the Time for Completion. The Procuring Entity should note that in the event of an extension of the time for completion of the Contract, the Procuring Entity would need to request anextension of this guarantee fromthe Guarantor. Such request mustbeinwriting and must be madepriorto the expiration date established in the guarantee.

Beneﬁciary:	 	[Insert name and Address of Procuring Entity]

Date:	[Insert date of issue]

Advance payment guarantee no. [Insert guarantee reference number]

Guarantor: [Insert name and address of place of issue, unless indicated in the letterhead]

- We have beeninformed that	[insert name of Contractor, which in the case of a joint venture shall be the name of the joint venture] (hereinafter called "the Contractor") has entered into Contract No.

[insert reference number of the contract] dated	with the Beneﬁciary, for the execution of		[insert name of contract and brief description of Works] (hereinafter called "the Contract").

- Furthermore, we understand that, according to the conditions of the Contract, the Beneﬁciary retains moneys up to the limit set forth in the Contract (“the Retention Money”), and that when the Taking-Over Certiﬁcate has been issued under the Contract and the ﬁrst half of the Retention Money has been certiﬁed for payment, and payment of [insert the second half of the Retention Money] is to be made against a Retention Money guarantee.

- At the request of the Contractor, we, as Guarantor, hereby irrevocably undertake to pay the Beneﬁciary any sum or sums not exceeding in total an amount of [insert amount in ﬁgures]	([insert amount in words	])1 upon receipt by us of the Beneﬁciary's complying demand supported by the Beneﬁciary's statement, whether in the demand itself or in a separate signed document accompanying or identifying the demand, stating that the Contractor is in breach of its obligation(s) under the Contract, without your needing to prove or show grounds for your demand or the sum speciﬁed therein.

- A demand under this guarantee may be presented as from the presentation to the Guarantor of a certiﬁcate from the Beneﬁciary's bank stating that the second half of the Retention Money as referred to above has been credited to the Contractor on its account number          at	[insert name and address of Applicant's bank].

- This guarantee shall expire no later than the …........................... Day of	,

2. 	2, and any demand for payment under it must be received by us at the ofﬁce indicated

above on or before that date.

- The Guarantor agrees to a one-time extension of this guarantee for a period not to exceed [six months] [one year], in response to the Beneﬁciary's written request for such extension, such request to be presented to the Guarantor before the expiry of the guarantee.

[Name of Authorized Ofﬁcial, signature(s) and seals/stamps]

Note: All italicized text (including footnotes) is for use in preparing this form and shall be deleted from the ﬁnal product.

1The Guarantor shall insert an amount representing the amount of the second half of the Retention Money.

2Insert a date that is twenty-eight days after the expiry of retention period after the actual completion date of the contract. The Procuring Entity should note that in the event of an extension of this date for completion of the Contract, the Procuring Entity would need to request an extension of this guarantee from the Guarantor. Such requestmust be in writing andmustbemadepriortotheexpiration date established in theguarantee.

Section IX - Special Conditions of Contract

The following Special Conditions shall supplement the GCC. Whenever there is a conﬂict, the provisions here in shall prevail over those in the GCC.

Part A - Contract Data

|  |  | The Director General Kenya Rural Roads Authority P.O. Box 48151-00100 Nairobi |
| --- | --- | --- |
|  |  | KeRRA/008/KEMA/KS/039/GOK/2022/2023-106 UFANISI-ENDO KABOR ROAD |
|  |  | The Director (Road Asset Management) Kenya Rural Roads Authority P.O. Box 48151-00100 Nairobi |
| Employers representative name and address |  | The Regional Director, Keiyo Marakwet Region, Kenya Rural Roads Authority , P. O. Box 213-30700, ITEN. |
| of the project |  | 6 Months |
| The Budgeted Estimates for the project |  | Kshs.9,999,835.88 (Kenya Shillings Nine Million, Nine Hundred And Ninety-Nine Thousand, Eight Hundred and Thirty-Five Cents Eighty-Eight Only) |
|  |  | From 8:00am to 5:00pm during normal working days excluding weekends and Public holidays |
|  |  | 0.01% per day of the contract sum to a limit of 1% of the contract sum |
|  |  | 1 % of the contract sum |

| Provisional Sums | 13.5. (b)(ii) | [If there are Provisional Sums, insert a percentage for adjustment of Provisional Sums} --- % |  |
| --- | --- | --- | --- |
| Adjustments for Changes in Cost | 13.8 | Period "n" applicable to the adjustment multiplier "Po": [Insert the period if different from one (1) month;if period "n" is one (1) month, insert "not applicable''} |  |
| Total advance payment | 14.2.1 | % Percentage of the Accepted Contract Amount payable in the currencies and proportions in which the Accepted Contract Amount is payable [Insert number and timing of installments if applicable] |  |
| Repayment amortization rate of advance payment | 14.2.5 (b) | --- % |  |
| Percentage of Retention | 14.3.2 (c) | --- % |  |
| Limit of Retention Money | 14.3.2 (c) | % of the Accepted Contract Amount |  |
| Plant and Materials | 14.5(b)(i) | If Sub-Clause 14.5 applies: Plant and Materials for payment Free on Board [list]. |  |
|  | 14.5(C)(i) | Plant and Materials for payment when delivered to the Site [list]. |  |
| Minimum Amount of Interim Payment Certificates | 14.6 | 0% of the Accepted Contract Amount. |  |
| Publishing source of commercial interest rates for financial charges in case of delayed payment | 14.8 | Specify % rate per month of delayed payment. |  |
| Maximum total liability of the Contractor to the Procuring Entity | 17.6 Issue | [Select one of the two options below as appropriate} The product of [insert a multiplier dlbesysKorengyreaaRteur rtahlaRn oonade}s tAimuetshtohreitAyccepted Contract Amount, or ------[insert amount of the maximum total liability] |  |
| Periods for submission of insurance: - Evidence of insurance. - Relevant policies | 18.1 | [Insert period for submission of evidence of insurance and policy. Period may be from 14 days to 30 days.} days days |  |
| Maximum amount of deductibles for insurance of the Procuring Entity's risks | 18.2.4 (d) | [Insert maximum amount of deductibles} |  |
| Minimum amount of third-party insurance | 18.3 | [Insert amount of third-party insurance} |  |

149

FORM NO. 8 BENEFICIAL OWNERSHIP DISCLOSURE FORM

(Amended and issued pursuant to PPRA CIRCULAR No. 02/2022)

INSTRUCTIONS TO TENDERERS: DELETE THIS BOX ONCE YOU HAVE COMPLETED THE FORM

This Beneﬁcial Ownership Disclosure Form (“Form”) is to be completed by the successful tenderer pursuant to Regulation 13 (2A) and 13 (6) of the Companies (Beneficial Ownership Information) Regulations, 2020. In case of joint venture, the tenderer must submit a separate Form for each member. The beneﬁcial ownership information to be submitted in this Form shall be current as of the date of its submission.

For the purposes of this Form, a Beneﬁcial Owner of a Tenderer is any natural person who ultimately owns or controls the legal person (tenderer) or arrangements or a natural person on whose behalf a transaction is conducted, and includes those persons who exercise ultimate effective control over a legal person (Tenderer) or arrangement.

Tender Reference No.:		[insert identiﬁcation no] Name of the Tender Title/Description:	[insert name of the assignment] to:

[insert complete name of Procuring Entity]

In response to the requirement in your notiﬁcation of award dated [insert date of notiﬁcation of award] to furnish additional information on beneﬁcial ownership:	[select one option as applicable and delete the options that are not applicable]

- We here by provide the following beneﬁcial ownership information.

Details of beneﬁcial ownership

|  | Details of all Beneficial Owners |  | % of shares a person holds in the company Directly or indirectly | % of voting rights a person holds in the company | Whether a person directly or indirectly holds a right to appoint or remove a member of the board of directors of the company or an equivalent governing body of the Tenderer (Yes / No) | Whether a person directly or indirectly exercises significant influence or control over the Company (tenderer) (Yes / No) |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | Full Name |  | Directly------ ----- % sosfusehdarbesy Ken Indirectly---- ------ % of shares | Directly…… ……….% of yvaotRinugrraigl hRtsoads Indirectly------- ---% of voting rights | - Having the right to appoint a majority of Athe orboard of the duitrhectoirtsy or an equivalent governing body of the Tenderer: Yes -----No---- - Is this right held directly or indirectly?: Direct……………… … Indirect…………… …... | - Exercises significant influence or contr1o5l o0ver the Company body of the Company (tenderer) Yes -----No---- - Is this influence or control exercised directly or indirectly? Direct………….. Indirect………… |
|  | National identity card number or Passport number | I |  |  |  |  |
|  | Personal Identification Number (where applicable) |  |  |  |  |  |
|  | Nationality |  |  |  |  |  |
|  | Date of birth [dd/mm/yyyy] |  |  |  |  |  |
|  | Postal address |  |  |  |  |  |
|  | Residential address |  |  |  |  |  |
|  | Telephone number |  |  |  |  |  |
|  | Email address |  |  |  |  |  |
|  | Occupation or profession |  |  |  |  |  |

|  | Details of all Beneficial Owners |  | % of shares a person holds in the company Directly or indirectly | % of voting rights a person holds in the company | Whether a person directly or indirectly holds a right to appoint or remove a member of the board of directors of the company or an equivalent governing body of the Tenderer (Yes / No) | Whether a person directly or indirectly exercises significant influence or control over the Company (tenderer) (Yes / No) |
| --- | --- | --- | --- | --- | --- | --- |
| 2. | Full Name |  | Directly------ ----- % of shares Indirectly---- ------ % of shares | Directly…… ……….% of voting rights Indirectly------- ---% of voting rights | - Having the right to appoint a majority of the board of the directors or an equivalent governing body of the Tenderer: Yes -----No---- - Is this right held directly or indirectly?: Direct……………… … Indirect…………… …... | - Exercises significant influence or control over the Company body of the Company (tenderer) Yes -----No---- - Is this influence or control exercised directly or indirectly? Direct………….. Indirect………… |
|  | National identity card number or Passport number |  |  |  |  |  |
|  | Personal Identification Number (where applicable) |  |  |  |  |  |
|  | Nationality(ies) |  |  |  |  |  |
|  | Date of birth [dd/mm/yyyy] |  |  |  |  |  |
|  | Postal address |  |  |  |  |  |
|  | Residential address |  |  |  |  |  |
|  | Telephone number |  |  |  |  |  |
|  | Email address |  |  |  |  |  |
|  | Occupation or profession |  |  |  |  |  |
| 3. |  |  |  |  |  |  |
| e. |  |  |  |  |  |  |
| t. |  |  |  |  |  |  |
| c |  |  |  |  |  |  |

- Am fully aware that beneficial ownership information above shall be reported to the Public Procurement

Regulatory Authority together with otIhsseureddetbayilKs einnyrealaRtuiornaltRo ocaodnstrAacutthaowraitryds and shall be ma1i5n1tained in the Government Portal, published and made publicly available pursuant to Regulation 13(5) of the

Companies (Beneficial Ownership Information) Regulations, 2020.(Notwithstanding this paragraph Personally Identifiable Information in line with the Data Protection Act shall not be published or made public). Note that Personally Identifiable Information (PII) is defined as any information that can be used to distinguish one person from another and can be used to deanonymize previously anonymous data. This information includes National identity card number or Passport number, Personal Identification Number, Date of birth, Residential address, email address and Telephone number.

- In determining who meets the threshold of who a beneficial owner is, the Tenderer must consider a natural person who in relation to the company:

  - holds at least ten percent of the issued shares in the company either directly or indirectly;

  - exercises at least ten percent of the voting rights in the company either directly or indirectly;

  - holds a right, directly or indirectly, to appoint or remove a director of the company; or

  - exercises significant influence or control, directly or indirectly, over the company.

- What is stated to herein above is true to the best of my knowledge, information and belief.

Name of the Tenderer	*[insert complete name of the Tenderer]

Name of the person duly authorized to sign the Tender on behalf of the Tenderer: ** [insert complete name of person duly authorized to sign the Tender]

Designation of the person signing the Tender	[insert complete title of the person signing the Tender]

Signature of the person named above:	[insert signature of person whose name and capacity are shown

above]

Date this ....................... [insert date of signing] day of	[Insert month], [insert year]

Bidder Ofﬁcial Stamp

Issued by Kenya Rural Roads Authority